| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0089-v3 |
| --- | --- |

| Title: | EE1-related: more refinements on EE1-1.4 and EE1-1.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liqiang Wang Sheng Lin Xiaozhong Xu Shan Liu Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Email: | liqiangwang@tencent.com barneylin@tencent.com xiaozhongxu@tencent.com shanl@tencent.com xiezhihuang@oppo.com yue.yu@oppo.com v-yuhaoping@oppo.com wangdong7@oppo.com |
| Source: | Tencent, OPPO |  |  |

_____________________________

- Abstract

This contribution presents more refinements, including updating the models, revising the encoder configuration, as well as the improvement used in EE1-1.3 [], for EE1-1.4 [] and EE1-1.5 [] tests. In addition, the model with the int16 precision is also included in the tests. Based on the EE1 anchor, the related results are shown in order of RA and AI configurations as follows.

Filter 1 (flt32, two models, new models):

{-9.30%, -19.42%, -20.02%}, {-7.41%, -16.80%, -17.33%}

Filter 1 (flt32, two models, new models, EncDbOpt):

{-9.79%, -20.43%, -20.97%}, {-7.36%, -17.34%, -18.00%}

Filter 1 (int16, two models, new models, EncDbOpt):

{-9.83%, -21.30%, -21.16%}, {-7.46%, -17.21%, -18.29%}

Filter 2 (flt32, one model, new model):

{-9.06%, -19.15%, -19.38%}, {-6.50%, -15.34%, -16.54%}

Filter 2 (flt32, one model, new model, EncDbOpt):

{-9.61%, -20.12%, -20.41%}, {-6.54%, -15.96%, -17.15%}

Filter 2 (int16, one model, new model, EncDbOpt):

{-9.64%, -19.89%, -20.60%}, {-6.48%, -15.62%, -16.87%}

Introduction

In addition to the refinement used in EE1-1.3 [1], this proposal presents two following refinements for EE1-1.4 [2] and EE1-1.5 [3] tests. On the one hand, all the models are updated by the models in EE1-1.4 and EE1-1.5. On the other hand, an encoder optimization tool [] originally in EE1 anchor is enabled. Besides the flt32 precision, the models with the int16 precision are also included in the tests, where the parameters of the int16 models used in the tests can be quantized by the SADL quantization way into the int16 format without any loss.

Proposed method

The reconstructed image after LMCS is fed into the network. Then, the output of NN filter is processed by ALF and CCALF. In detail, as shown in Fig. 1, U and V channels are up-sampled and down-sampled in the pre-processing and post-processing, respectively.

Fig. 1 the pre-processing and post-processing units

Network architecture

The network structure of the proposed NN filter is shown in Fig. 2. Along with the reconstructed image (rec_yuv), the prediction image (pred_yuv) [] and the partition image (par_yuv) [] are also fed into the network. Based on the un-filtered reconstructed image, the partition image is generated by setting each sample value into the average value of the whole partition block.

Fig. 2: The network architecture of filter 1

Filter 1: There are two models used for the I slice and the B slice respectively. The network structure is shown in Fig. 2. Note that the partition image is only fed for the I slice, and the base QP is fed only for the network of the B slice.

Filter 2: There is one model used for both the I slice and the B slice commonly. The network structure is shown in Fig. 3.

Compared with the implementation in JVET-W0113[], the ResBlock is also modified as shown in Fig. 3. With the same spirit in JVET-W0113, the number of channels firstly goes up before the activation layer, and then goes down after the activation layer. In addition, and the convolution in the skip-layer connection is removed.

Fig. 3: The network architecture of filter 2

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla A100 40GB |
|  | Framework: | Pytorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~1000 |
|  | Batch size: | 64 |
|  | Loss function: | L1 |
|  | Training time: | ~150h |
|  | Training data information: | DIV2K, TVD, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations | 1200 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Training data update strategy: | Iterative training method |
|  | Other information: |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test items | Filter 1 (2 models) |  | Filter 2 (1 model) |  |
|  |  | flt32 | int16 | flt32 | int16 |
| Mandatory | HW environment: |  |  |  |  |
|  | GPU Type | CPU only |  |  |  |
|  | Framework: | Libtorch v1.9.0 |  |  |  |
|  | Number of GPUs per Task | 0 |  |  |  |
|  | Number of Parameters (Each Model) | 1.54M |  | 1.9M |  |
|  | Total Number of Parameters (All Models) | 3.08M |  | 1.9M |  |
|  | Parameter Precision (Bits) | 32 | 16 | 32 | 16 |
|  | Memory Parameter (MB) | 12.32MB | 6.16MB | 7.6MB | 3.8MB |
|  | Multiplay Accumulate (MAC)/pixel, worst-case | 401K (w/o block extension) 507K (w/ block extension) |  | 485K (w/o block extension) 615K (w/ block extension) |  |
|  | Calculation Method | block basis |  |  |  |
| Optional |  |  |  |  |  |
|  | Total Conv. Layers | 78 |  | 101 |  |
|  | Total FC Layers | 0 |  |  |  |
|  | Total Memory (MB) |  |  |  |  |
|  | Batch size: | 1 |  |  |  |
|  | Patch size | 144x144 |  |  |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |  |  |
|  | Peak Memory Usage (Total) |  |  |  |  |
|  | Peak Memory Usage (per Model) |  |  |  |  |
|  | Border handling |  |  |  |  |
|  | Other information: |  |  |  |  |

Implementation details

- For I slices and B slices, Deblock and SAO are both enabled. The optimal filtered result is decided between the two outputs from the proposed filter and the SAO.

- Multiple results are inferred by three models for the B slice, and these models are indexed by {BaseQP, BaseQP - 5, BaseQP + 5}, where the index is decided at the slice-level.

- An encoder optimization tool [4] originally in EE1 anchor, controlled by EncDbOpt in the software, is enabled. Instead of the unfiltered reconstructed pixels, the deblocking-filtered reconstructed pixels are used to calculate the distortion in the RDO process of the block partitioning decision.

- A scaling operation is carried out to refine the result of NN filter. The scaling factors are signaled in the slice header for each component. The specific process is the same as the scaling process used in []. Additionally, there are 3 fixed weights to blend the results of the proposed filter and the conventional filter. These fixed weights are 1, 0.75 and 0.5, which are similar as those in [].

- The proposed filter can be turned on/off at the CTU level and slice level.

Results

All the test results are provided as follows, which are based on the EE1 anchor [][]. The detailed results, such as LD results, can be found in the attached Excel files.

| Filters | Precision | Test items | Configurations | Results |
| --- | --- | --- | --- | --- |
| Filter 1 (two models) | flt32 | test1 | new models | Table 3 and Table 4 |
|  | flt32 | test2 | new models+ EncDbOpt | Table 5 and Table 6 |
|  | int16 | test3 | new models+ EncDbOpt | Table 7 and Table 8 |
| Filter 2 (one model) | flt32 | test4 | new model | Table 9 and Table 10 |
|  | flt32 | test5 | new model+ EncDbOpt | Table 11 and Table 12 |
|  | int16 | test6 | new model+ EncDbOpt | Table 13 and Table 14 |

Results of test1 (filter 1, two models, flt32, new models)

Table 3. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -11.16% | -9.46% | -13.97% | -18.57% | 132% | 32106% |
| Class A2 | -12.16% | -10.20% | -20.06% | -16.04% | 129% | 31063% |
| Class B | -11.86% | -8.90% | -20.15% | -21.31% | 133% | 33500% |
| Class C | -12.32% | -9.00% | -22.11% | -22.50% | 123% | 30806% |
| Class E |  |  |  |  |  |  |
| Overall | -11.90% | -9.30% | -19.42% | -20.02% | 129% | 31995% |
| Class D | -12.99% | -10.05% | -21.46% | -22.11% | 124% | 32136% |
| Class F | -5.92% | -3.92% | -12.26% | -11.55% | 149% | 14110% |

Table 4. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.94% | -6.55% | -14.67% | -17.54% | 150% | 21201% |
| Class A2 | -8.89% | -6.53% | -17.49% | -14.49% | 128% | 17661% |
| Class B | -9.15% | -6.56% | -16.89% | -17.00% | 123% | 17449% |
| Class C | -10.40% | -7.78% | -17.26% | -19.25% | 114% | 13694% |
| Class E | -11.97% | -10.06% | -17.48% | -17.92% | 126% | 20382% |
| Overall | -9.82% | -7.41% | -16.80% | -17.33% | 126% | 17563% |
| Class D | -10.21% | -7.77% | -15.97% | -19.05% | 113% | 14847% |
| Class F | -6.07% | -4.42% | -10.77% | -11.26% | 111% | 12475% |

Results of test2 (filter 1, two models, flt32, new models, EncDbOpt)

Table 5. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -11.94% | -10.18% | -15.02% | -19.43% | 137% | 32573% |
| Class A2 | -12.75% | -10.69% | -21.00% | -16.87% | 134% | 31202% |
| Class B | -12.47% | -9.37% | -21.26% | -22.27% | 138% | 33710% |
| Class C | -12.84% | -9.34% | -23.04% | -23.59% | 128% | 30815% |
| Class E |  |  |  |  |  |  |
| Overall | -12.52% | -9.79% | -20.43% | -20.97% | 134% | 32186% |
| Class D | -13.30% | -10.21% | -22.11% | -23.01% | 129% | 32090% |
| Class F | -6.33% | -4.23% | -12.94% | -12.31% | 153% | 13957% |

Table 6. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.09% | -6.62% | -15.13% | -17.88% | 161% | 21466% |
| Class A2 | -9.00% | -6.44% | -18.18% | -15.14% | 141% | 17986% |
| Class B | -9.25% | -6.47% | -17.49% | -17.65% | 138% | 17531% |
| Class C | -10.42% | -7.68% | -17.55% | -19.78% | 128% | 13610% |
| Class E | -12.22% | -10.07% | -18.21% | -19.17% | 139% | 20283% |
| Overall | -9.94% | -7.36% | -17.34% | -18.00% | 140% | 17638% |
| Class D | -10.21% | -7.58% | -16.67% | -19.53% | 127% | 14960% |
| Class F | -6.07% | -4.32% | -11.13% | -11.53% | 120% | 12492% |

Results of test3 (filter 1, two models, int16, new model, EncDbOpt)

Table 7. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -12.15% | -10.09% | -16.74% | -19.90% | 136% | 31970% |
| Class A2 | -12.72% | -10.62% | -21.38% | -16.67% | 133% | 30789% |
| Class B | -12.77% | -9.53% | -22.23% | -22.73% | 137% | 33135% |
| Class C | -12.93% | -9.41% | -23.50% | -23.52% | 127% | 30058% |
| Class E |  |  |  |  |  |  |
| Overall | -12.68% | -9.83% | -21.30% | -21.16% | 133% | 31587% |
| Class D | -13.41% | -10.21% | -22.41% | -23.61% | 128% | 31438% |
| Class F | -6.58% | -4.34% | -13.89% | -12.68% | 152% | 13961% |

Table 8. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.40% | -6.88% | -15.25% | -18.62% | 161% | 20908% |
| Class A2 | -9.02% | -6.59% | -17.83% | -14.84% | 141% | 17574% |
| Class B | -9.41% | -6.61% | -17.43% | -18.24% | 137% | 17528% |
| Class C | -10.46% | -7.63% | -17.60% | -20.27% | 128% | 13595% |
| Class E | -12.16% | -10.13% | -17.64% | -18.85% | 138% | 20129% |
| Overall | -10.03% | -7.46% | -17.21% | -18.29% | 140% | 17465% |
| Class D | -10.16% | -7.46% | -16.68% | -19.80% | 126% | 14786% |
| Class F | -6.56% | -4.46% | -13.14% | -12.61% | 118% | 13450% |

Results of test4 (filter 2, one model, flt32, new model)

Table 9. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -11.21% | -9.41% | -14.41% | -18.79% | 135% | 35994% |
| Class A2 | -11.65% | -9.73% | -20.01% | -14.80% | 132% | 34621% |
| Class B | -11.74% | -8.81% | -19.81% | -21.23% | 136% | 37597% |
| Class C | -11.74% | -8.62% | -21.23% | -20.96% | 125% | 34186% |
| Class E |  |  |  |  |  |  |
| Overall | -11.62% | -9.06% | -19.15% | -19.38% | 132% | 35743% |
| Class D | -12.47% | -9.72% | -20.43% | -21.03% | 127% | 36353% |
| Class F | -5.91% | -3.86% | -12.52% | -11.54% | 156% | 16205% |

Table 10. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.51% | -6.11% | -14.19% | -17.27% | 155% | 24167% |
| Class A2 | -7.92% | -5.74% | -15.93% | -12.97% | 130% | 19606% |
| Class B | -8.45% | -5.92% | -14.93% | -17.20% | 125% | 19020% |
| Class C | -9.06% | -6.37% | -16.30% | -17.94% | 115% | 14781% |
| Class E | -10.55% | -8.78% | -15.31% | -16.40% | 128% | 22152% |
| Overall | -8.86% | -6.50% | -15.34% | -16.54% | 129% | 19295% |
| Class D | -9.05% | -6.41% | -15.42% | -18.52% | 114% | 16055% |
| Class F | -5.98% | -4.04% | -11.92% | -11.72% | 113% | 16838% |

Results of test5 (filter 2, one model, flt32, new model, EncDbOpt)

Table 11. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -12.00% | -10.15% | -15.36% | -19.76% | 140% | 36354% |
| Class A2 | -12.32% | -10.32% | -20.88% | -15.77% | 137% | 34911% |
| Class B | -12.39% | -9.33% | -20.89% | -22.26% | 142% | 37929% |
| Class C | -12.29% | -9.01% | -22.18% | -22.08% | 131% | 34363% |
| Class E |  |  |  |  |  |  |
| Overall | -12.27% | -9.61% | -20.12% | -20.41% | 138% | 36029% |
| Class D | -12.94% | -9.98% | -21.44% | -22.19% | 132% | 36294% |
| Class F | -6.35% | -4.22% | -13.19% | -12.33% | 161% | 16128% |

Table 12. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.70% | -6.23% | -14.60% | -17.65% | 167% | 24105% |
| Class A2 | -8.09% | -5.77% | -16.56% | -13.48% | 144% | 19592% |
| Class B | -8.62% | -5.93% | -15.69% | -17.67% | 141% | 19079% |
| Class C | -9.17% | -6.34% | -16.76% | -18.54% | 130% | 14841% |
| Class E | -10.89% | -8.90% | -16.11% | -17.60% | 142% | 22140% |
| Overall | -9.04% | -6.54% | -15.96% | -17.15% | 143% | 19316% |
| Class D | -9.05% | -6.28% | -15.96% | -18.79% | 128% | 16256% |
| Class F | -5.92% | -3.94% | -12.05% | -11.69% | 122% | 16796% |

Results of test6 (filter 2, one model, int16, new model, EncDbOpt)

Table 13. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -12.24% | -10.07% | -17.16% | -20.39% | 140% | 36100% |
| Class A2 | -12.44% | -10.39% | -20.88% | -16.29% | 137% | 34426% |
| Class B | -12.21% | -9.27% | -20.11% | -21.92% | 141% | 37346% |
| Class C | -12.32% | -9.21% | -20.92% | -22.34% | 130% | 33967% |
| Class E |  |  |  |  |  |  |
| Overall | -12.29% | -9.64% | -19.89% | -20.60% | 137% | 35583% |
| Class D | -12.90% | -10.23% | -20.20% | -21.60% | 131% | 36111% |
| Class F | -6.46% | -4.40% | -12.39% | -12.89% | 159% | 15266% |

Table 14. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.36% | -6.04% | -14.39% | -16.27% | 165% | 23899% |
| Class A2 | -7.99% | -5.66% | -16.45% | -13.55% | 143% | 19718% |
| Class B | -8.54% | -5.83% | -15.65% | -17.64% | 139% | 18916% |
| Class C | -9.12% | -6.42% | -15.91% | -18.54% | 128% | 14628% |
| Class E | -10.78% | -8.89% | -15.61% | -17.27% | 141% | 22009% |
| Overall | -8.92% | -6.48% | -15.62% | -16.87% | 142% | 19183% |
| Class D | -8.70% | -6.35% | -14.04% | -17.46% | 126% | 15989% |
| Class F | -6.05% | -4.10% | -11.87% | -11.95% | 120% | 16290% |

Conclusions

In addition to the refinement used in EE1-1.3, more refinements are used to improve the performance for both EE1-1.4 and EE1-1.5. An additional coding performance can be achieved by applying these refinements.

References

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[] Z. Xie, Y. Yu, H. Yu, D. Wang. EE1-1.3: On BaseQP adjustment in CNNLF. JVET-AA0122, 27th Meeting, by teleconference, 13–22 July 2022.

[] L. Wang, S. Lin, X. Xu, S. Liu, F. Galpin. EE1-1.4: neural network based in-loop filter with 2 models. JVET-AA0087, 27th Meeting, by teleconference, 13–22 July 2022.

[] L. Wang, S. Lin, X. Xu, S. Liu, F. Galpin. EE1-1.5: neural network based in-loop filter with a single model. JVET-AA0088, 27th Meeting, by teleconference, 13–22 July 2022.

[] N. Hu, V. Seregin, W.-J. Chien, M. Karczewicz. Encoder optimization with deblocking filter. JVET-M0428, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

[]  H. Zhu, X. Xu, and S. Liu. Deep learning based in-loop filter with QP and residual distribution information. Doc. AVS-M5654, Aug. 2020.

[] W. Lin, X. He, X. Han, D. Liu, J. See, J. Zou, H. Xiong, and F. Wu, “Partition-Aware Adaptive Switching Neural Networks for Post-Processing in HEVC,” IEEE Transactions on Multimedia, vol. 22, no. 11, pp. 2749–2763, Nov. 2020.

[] L. Wang, W. Jiang, X. Xu, S. Liu. AHG11: neural network based in-loop filter. JVET-W0113, 23rd Meeting, by teleconference, 7–16 July 2021.

[] H. Wang, J. Chen, K. Reuze, A. M. Kotra, M. Karczewicz. EE1-1.4: Test on Neural Network-based In-Loop Filter with Large Activation Layer. JVET-W0130, 23rd Meeting, by teleconference, 7–16 July 2021.

[] K. Andersson, J. Ström, D. Liu, R. Sjöberg. EE1-related: Combination of VVC deblocking and NN loop filtering. JVET-Y0098, 25th Meeting, by teleconference, 12–21 January 2022.

[] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc.

[] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-Z2016, 26th Meeting, by teleconference, 20–29 April 2022.