| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0097-v2 |
| --- | --- |

| Title: | ECM fix for block-level out-of-bound checking |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, P. Andrivon, M. Radosavljević | Tel: Email: | fabricel@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

This contribution proposes to fix the ECM software implementation, with regards to the process that checks if a reference block of a bi-predicted inter block is out-of-bounds or not.

Indeed, it is stated the source code implementation of these checks does not exactly correspond to the description of the adopted methods described in JVET-Z0136 and JVET-Y0125. The ECM-5.0 implementation leads to a higher number of blocks detected as OOB than it should be during the bi-prediction process, leading to per-sample OOB checks for a larger number of blocks than is needed.

A fix is proposed in this contribution. It provides reduced runtime with no change in ECM CTC results.

# Introduction

In ECM-5.0 [1][2], bi-prediction is performed in a way that avoids relying on reference samples out of a reference picture bounds (OOB), if possible. To do so, in the case of a bi-predicted block with an OOB reference block in one of the two reference pictures, the OOB prediction samples are not used. The concerned part of the block is rather uni-predicted based on non-OOB prediction samples, if available in the other reference picture.

The criteria to determine a prediction sample position as OOB in JVET-Z0136 is the following one. Let Pos_xi,j and Pos_yi,j denote the position of one prediction sample in one current block, Mv_xi,jLx and Mv_yi,jLx (x = 0,1) denote the MV of the current block; PosLeftBdry, PosRightBdry, PosTopBdry and PosBottomBdry are the positions of four boundaries of the picture. A prediction sample is regarded as OOB when at least one of the following conditions is satisfied:

(Pos_xi,j+Mv_xi,jLx)>(PosRightBdry+half_pixel),

(Pos_xi,j+Mv_xi,jLx)<(PosLeftBdry- half_pixel),

(Pos_yi,j+Mv_yi,jLx)>(PosBottomBdry+ half_pixel),

(Pos_yi,j+Mv_yi,jLx)<(PosTopBdry- half_pixel)

However, in the ECM-5.0 implementation, a block-level OOB condition is computed first. Depending on that condition, a block being bi-predicted is considered OOB or not. If the block is not OOB, then the usual bi-prediction process occurs. Otherwise, an OOB/non-OOB check is performed for each sample position in the block. Then, in case of OOB prediction samples, the bi-prediction is adapted as proposed in JVET-Z0136.

The block-level OOB/non-OOB test, as implemented in ECM-5.0, is the following one.

Table 1: ECM-5.0 source code for block-level OOB criteria computation

int offsetX = (pos.x << MV_FRACTIONAL_BITS_INTERNAL) + rcMv.getHor(); int offsetY = (pos.y << MV_FRACTIONAL_BITS_INTERNAL) + rcMv.getVer(); bool isOOB = false; if ( (offsetX <= horMin) ¦¦ ((offsetX + (size.width << MV_FRACTIONAL_BITS_INTERNAL) - 1) >= horMax) ¦¦ (offsetY <= verMin) ¦¦ ((offsetY + (size.height << MV_FRACTIONAL_BITS_INTERNAL) - 1) >= verMax)) { isOOB = true; }

Where variable “pos” and “size” represent the block position and size, rcMv is the considered motion vector. Moreover horMin, horMax, vermin, verMax are the bounds used:

- horMin =PosLeftBdry- half_pixel

- verMin= PosTopBdry- half_pixel

- horMax=PosRightBdry+ hal_pixel

- verMax=PosBottomBdry+half_pixel

Finally, MV_FRACTIONAL_BITS_INTERNAL is the number of bits for fractional sample accuracy motion representation and half_pixel = 1 << ( MV_FRACTIONAL_BITS_INTERNAL - 1).

It is stated that the quantity (offsetX+(size.Width << MV_FRACTIONAL_BITS_INTERNAL)-1) is not the appropriate one to compare to the right-hand bound horMax. Indeed, with this approach, a block may be unnecessarily detected has OOB

As an example, if motion vector rcMv is zero and the block is aligned to the right picture boundary (pos.x+size.width =  PosRightBdry), then the block is detected as OOB, which is not correct observation. In the same way, for picture bottom boundary check, the quantity (offsetY + (size.height << MV_FRACTIONAL_BITS_INTERNAL) - 1) is not the right one to compare to verMax.

# Proposal

The modified source code proposed here is shown by the following

Table 2: proposed source code modification to correct the block-level OOB/non-OOB criteria computation

if ( (offsetX <= horMin) ¦¦ ((offsetX + ((size.width -1) << MV_FRACTIONAL_BITS_INTERNAL) ) >= horMax) ¦¦ (offsetY <= verMin) ¦¦ ((offsetY + ((size.height -1)<< MV_FRACTIONAL_BITS_INTERNAL) ) >= verMax)) { isOOB = true; }

# Simulation Results

The ECM software modified as proposed in this contribution has been run in JVET EE2 CTCs [3]. It can be seen the coding results of ECM-5.0 are unchanged, but runtime is decreased, due to less reference blocks detected as OOB. Encoding times are not accurately reliable.

Table 3: simulation results obtained with proposed software fix

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 109% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 105% | 91% |
| Class B | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 96% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 91% | 96% |
| Class TGM | 0.00% | 0.00% | 0.00% | 98% | 104% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 93% | 89% |
| Class C | 0.00% | 0.00% | 0.00% | 96% | 96% |
| Class E | 0.00% | 0.00% | 0.00% | 96% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 94% | 94% |
| Class D | 0.00% | 0.00% | 0.00% | 97% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 95% | 93% |
| Class TGM | 0.00% | 0.00% | 0.00% | 96% | 94% |

Finally, the following Table 4 shows, for some sequences of the EE2 CTCs, the number of blocks detected by the ECM decoder as OOB, respectively with the current software and the proposed modification. A significant reduction is noted with the proposal.

Table 4: exemplary numbers of blocks detected as OOB respectively with ECM5 and proposed correction

| Sequence | Qp | ECM | Proposal | Variation |
| --- | --- | --- | --- | --- |
| BasketballPass | 22 | 123316 | 57827 | -53.11% |
|  | 27 | 137917 | 75176 | -45.49% |
|  | 32 | 149027 | 91778 | -38.42% |
|  | 37 | 163376 | 109732 | -32.83% |
| BlowingBubbles | 22 | 171881 | 67622 | -60.66% |
|  | 27 | 172886 | 69225 | -59.96% |
|  | 32 | 177340 | 72643 | -59.04% |
|  | 37 | 181638 | 74946 | -58.74% |
| BQSquare | 22 | 279788 | 151255 | -45.94% |
|  | 27 | 294085 | 165037 | -43.88% |
|  | 32 | 294778 | 162340 | -44.93% |
|  | 37 | 294792 | 162072 | -45.02% |
| RaceHorses | 22 | 74219 | 57717 | -22.23% |
|  | 27 | 78280 | 62620 | -20.01% |
|  | 32 | 91688 | 73831 | -19.48% |
|  | 37 | 109348 | 89867 | -17.82% |
| BasketballDrill | 22 | 183899 | 43373 | -76.41% |
|  | 27 | 207245 | 59287 | -71.39% |
|  | 32 | 230879 | 86155 | -62.68% |
|  | 37 | 261733 | 126565 | -51.64% |
| BQMall | 22 | 390930 | 210074 | -46.26% |
|  | 27 | 410847 | 235495 | -42.68% |
|  | 32 | 428328 | 256628 | -40.09% |
|  | 37 | 464416 | 298733 | -35.68% |
| PartyScene | 22 | 399705 | 226006 | -43.46% |
|  | 27 | 436899 | 242448 | -44.51% |
|  | 32 | 448341 | 245072 | -45.34% |
|  | 37 | 458954 | 251853 | -45.12% |
| RaceHorses | 22 | 169409 | 145204 | -14.29% |
|  | 27 | 182024 | 160169 | -12.01% |
|  | 32 | 208490 | 186967 | -10.32% |
|  | 37 | 255556 | 230963 | -9.62% |
| MarketPlace | 22 | 2242122 | 1998389 | -10.87% |
|  | 27 | 2485616 | 2259354 | -9.10% |
|  | 32 | 2664803 | 2451493 | -8.00% |
|  | 37 | 2914474 | 2715091 | -6.84% |
| RitualDance | 22 | 1099951 | 985359 | -10.42% |
|  | 27 | 1294214 | 1182730 | -8.61% |
|  | 32 | 1458134 | 1345200 | -7.75% |
|  | 37 | 1651299 | 1535282 | -7.03% |
| Cactus | 22 | 667046 | 326392 | -51.07% |
|  | 27 | 768577 | 432604 | -43.71% |
|  | 32 | 816437 | 504204 | -38.24% |
|  | 37 | 800836 | 512060 | -36.06% |
| BasketballDrive | 22 | 699264 | 520961 | -25.50% |
|  | 27 | 884332 | 723018 | -18.24% |
|  | 32 | 1129608 | 980276 | -13.22% |
|  | 37 | 1404844 | 1267675 | -9.76% |
| BQTerrace | 22 | 738574 | 507087 | -31.34% |
|  | 27 | 814437 | 562430 | -30.94% |
|  | 32 | 893202 | 652967 | -26.90% |
|  | 37 | 1033216 | 805209 | -22.07% |
| console | 22 | 43084 | 1550 | -96.40% |
|  | 27 | 39718 | 2018 | -94.92% |
|  | 32 | 44518 | 2193 | -95.07% |
|  | 37 | 44553 | 3527 | -92.08% |
| desktop | 22 | 64954 | 55 | -99.92% |
|  | 27 | 62455 | 66 | -99.89% |
|  | 32 | 63249 | 227 | -99.64% |
|  | 37 | 53802 | 113 | -99.79% |
| flyingGraphics | 22 | 41137 | 33596 | -18.33% |
|  | 27 | 77563 | 65552 | -15.49% |
|  | 32 | 129042 | 106208 | -17.70% |
|  | 37 | 207989 | 167042 | -19.69% |
| ArenaOfValor | 22 | 624968 | 322694 | -48.37% |
|  | 27 | 704692 | 371091 | -47.34% |
|  | 32 | 736262 | 397168 | -46.06% |
|  | 37 | 736989 | 408185 | -44.61% |
| ChineseEditing | 22 | 119774 | 136 | -99.89% |
|  | 27 | 121620 | 288 | -99.76% |
|  | 32 | 119920 | 642 | -99.46% |
|  | 37 | 113079 | 1104 | -99.02% |

# Conclusion

A fix is proposed for computing the OOB/non-OOB criteria at block level. It removes “false positive” detection of OOB block that happen in the ECM-5.0 implementation. As a result, it leads to a reduced number of blocks for which the sample-by-sample OOB detection process occurs, during bi-prediction.

It is proposed to adopt the proposed software fix into ECM. Merge request !191 for ECM is created for this software fix.

# Cited references

[1] “EE2-Test2.2: Enhanced bi-directional motion compensation”, Y.-W. Chen, X. Xiu, H.-J. Jhu, N. Yan, X. Wang (Kwai), H. Huang, Y-J. Chang, C.-C. Chen, M. Karczewicz, V. Seregin, Y. Zhang, Z. Zhang, (Qualcomm), April 2022, JVET-Z0136

[2] “AHG12: Enhanced bi-directional motion compensation”, Y.-W. Chen, C.-W. Kuo, N. Yan, W. Chen, X. Xiu, X. Wang (Kwai Inc.), Jan 2022, JVET-Y0125

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021.

# Patent rights declaration(s)

Xiaomi does not have any current or pending patent rights relating to the technology described in this contribution.