| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0098-v1 |
| --- | --- |

| Title: | Ahg 12: encoder configuration proposal to reduce worst case encoding time |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fabrice Le Léannec, Pierre Andrivon, Miloš Radosavljević | Tel: Email: | fabricel@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

This contribution proposes to modify EE2 coding test conditions, to significantly reduce the worst-case encoding runtime for RA, with negligible impact on overall BD-rate results.

The CTC proposal consists in modifying the encoder configuration for class A sequences, QP 22, which is the worst case. The maximum MTT hierarchy depth is lowered to 2 from the 4th temporal layer (test 1) or from the 3rd temporal layer (test 2).

Test 1 reduces the worst-case encoding time by 7 days of overall computation (sequences DaylightRoad2 and ParkRunning3). Overall impact on BD-rate is +0.01% loss.

Test 2 reduces the worst-case encoding time by about 11 days of overall computation (sequences DaylightRoad2 and ParkRunning3). Overall impact on BD-rate is +0.02% loss.

# Introduction

ECM-5.0 encoding time is becoming problematic for investigating new coding tools. In particular, the worst-case encoding time makes it very long to wait for complete results.

When parallel chunk encoding is performed, about 8 days of computation are needed to get full results for the worst-case sequences, corresponding to ~40 days of computation in total.

Therefore, this contribution proposes to reduce the encoding worst case runtime by changing the encoder configuration for lowest QP in class A, random access. The following parameters are proposed in the two reported tests.

- Test 1: MaxMTTHierarchyDepthByTid=333222

- Test 2: MaxMTTHierarchyDepthByTid=332222

For other QPs and classes, EE2 CTC are kept unchanged.

# Impact on performances and runtime

The Table 1 and Table 2 below show how ECM-5.0 performances are changed by the test 1 and test 2, respectively. Negligible impact on average performances is observed.

Table 1: performance results of test 1 (MaxMTTHierarchyDepthByTid=333222)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.03% | 0.05% | 97% | 98% |
| Class A2 | 0.03% | 0.04% | 0.04% | 96% | 97% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.02% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 2: performance results of test 2 (MaxMTTHierarchyDepthByTid=332222)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.06% | 0.09% | 94% | 102% |
| Class A2 | 0.06% | 0.07% | 0.08% | 93% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | 0.03% | 97% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100% | 100% |

The Table 3 and Table 4 below show the reduction of worst-case encoding time obtained respectively with test 1 and test 2. In the worst cases, which corresponds to sequences DaylightRoad2 and ParkRunning3, the average reduction in encoding time per intra period is reduced by 1.4 and 2.1 days, respectively with test 1 and test 2.

This corresponds to the reduction in time to wait for full RA results, when parallel chunk coding is used.

In terms of total computation time, the two tests reduce it by about 7 and 11 days respectively. It is stated these modified encoder settings makes the ECM more practical to use.

Table 3: impact of first proposed encoder configuration (test 1) on ECM-5.0 random access

| ECM-5.0 Random Access, QP=22 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Enc Time |  | Chunks | Enc time/chunk |  | Enc time reduction |  | Enc time reduction/chunk |  |
| Sequence | [h] | [days] |  | [h] | [days] | [h] | [days] | [h] | [days] |
| Tango2 | 764.12 | 31.84 | 5 | 152.82 | 6.37 | -101.54 | -4.23 | -20.31 | -0.85 |
| FoodMarket4 | 545.44 | 22.73 | 5 | 109.09 | 4.55 | -38.24 | -1.59 | -7.65 | -0.32 |
| Campfire | 1112.95 | 46.37 | 10 | 111.29 | 4.64 | -138.21 | -5.76 | -13.82 | -0.58 |
| CatRobot1 | 619.82 | 25.83 | 5 | 123.96 | 5.17 | -67.12 | -2.80 | -13.42 | -0.56 |
| DaylightRoad2 | 891.20 | 37.13 | 5 | 178.24 | 7.43 | -169.63 | -7.07 | -33.93 | -1.41 |
| ParkRunning3 | 984.13 | 41.01 | 5 | 196.83 | 8.20 | -175.50 | -7.31 | -35.10 | -1.46 |

Table 4: impact of second proposed encoder configuration (test 2) on ECM-5.0 random access

| ECM-5.0 Random Access, QP=22 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Enc Time |  | Chunks | Enc time/chunk |  | Enc time reduction |  | Enc time reduction/chunk |  |
| Sequence | [h] | [days] |  | [h] | [days] | [h] | [days] | [h] | [days] |
| Tango2 | 764.12 | 31.84 | 5 | 152.82 | 6.37 | -177.71 | -7.40 | -35.54 | -1.48 |
| FoodMarket4 | 545.44 | 22.73 | 5 | 109.09 | 4.55 | -76.70 | -3.20 | -15.34 | -0.64 |
| Campfire | 1112.95 | 46.37 | 10 | 111.29 | 4.64 | -270.95 | -11.29 | -27.09 | -1.13 |
| CatRobot1 | 619.82 | 25.83 | 5 | 123.96 | 5.17 | -126.46 | -5.27 | -25.29 | -1.05 |
| DaylightRoad2 | 891.20 | 37.13 | 5 | 178.24 | 7.43 | -271.72 | -11.32 | -54.34 | -2.26 |
| ParkRunning3 | 984.13 | 41.01 | 5 | 196.83 | 8.20 | -251.37 | -10.47 | -50.27 | -2.09 |

# Conclusion

A modification of EE2 CTCs is proposed to significantly reduce the worst-case encoding time of ECM in random access.

It is shown the overall performances in EE2 CTCs would be unchanged, while the ECM test set is made more convenient to use in practice.

It is proposed to adopt one of the two proposed CTC modifications for next JVET meetings.

# Patent rights declaration(s)

Xiaomi does not have any current or pending patent rights relating to the technology described in this contribution.