| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0099-v1 |
| --- | --- |

| Title: | AHG9: On subpictures order |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seung Hwan Kim 10680 Treena St San Diego, CA, 92131, USA SangHeon Lee 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772, South Korea | Email: | dr.hendry@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution asserted that the order of subpictures can be not in raster-scan order unnecessarily without breaking the availability rules that are specified in the current specification. When subpictures’ order is not in raster scan order, it may cause additional complexity for implementation. It is further asserted that while such flexibility may be desired (i.e., allowing subpictures to be not raster scan order because of its structure/shape), most common use-case scenarios / applications can be supported with subpictures that are in raster scan order.

It is proposed that we simplify subpicture order such that if the structure of subpicture allows both raster scan order and non-raster scan order, the order shall be in raster scan order. This can be done by defining a constraint or use a new flag in general constraints information (GCI).

# Problem Statement

VVC specification does not specifically constraint the order of subpictures in a picture to be raster scan only as long as the order of subpictures does not violate the following constraint:

It is a requirement of bitstream conformance that the shapes of the subpictures shall be such that each subpicture, when decoded, shall have its entire left boundary and entire top boundary consisting of picture boundaries or consisting of boundaries of previously decoded subpictures.

Figure 1 below illustrates an example of a picture with 6 subpictures (i.e., SP 1, ..., SP 6). With such subpicture partitioning structure, the order of the subpictures can be in raster scan order (i.e., SP 1, SP 2, SP 3, SP 4, SP 5, and finally SP 6) or in non-raster scan order such as SP 1, SP 4, SP2, SP 5, SP 3, and SP 6. Whether the order of the subpictures follows raster scan order or not depends on how the subpicture structure is signalled in SPS.

Figure 1 – Example of subpictures

Traditionally, video decoders are implemented optimally with assumption that the order of blocks (i.e., CTU, slices, and also subpicture) is in raster scan order. When the order is not in raster scan order, additional steps or processes may be needed such as the handling of line buffers and column buffers. Figure 2 below illustrates a subpicture structure with some critical coding blocks, particularly when loop-filter is allowed to be across subpicture boundaries. Generally, we store every bottom right boundary samples in CTU to line and column buffers, respectively, and when CTU row is changed, we moves line buffer data to column buffer. Such strategy works fine when the order of subpictures is in raster scan order. However, when the order is not raster scan such simple strategy does not work and additional buffer is needed. Consider the case when the order is SP 1, SP 2, SP 4, SP 5, SP 3, and then SP 6. With such ordering, CTU row changes, for one particular CTU line / row happens more than once so that the strategy of moving line buffer data to column buffer cannot be used for the following problem: when decoding block E, we need samples from block A so we move / copy sample from line buffer to column buffer that are from block A, which is part of the routine we do when CTU row changes. However, such move then will overwrite the column buffer samples that were from block B and we will run into problem when we need to decode block C, unless we have additional buffer to back up the bottom right samples of block B.

Figure 2 – Example of subpictures with critical blocks for decoding

While subpictures with non-raster scan order can be implemented, it may require additional process or buffer. As far as authors’ knowledge, currently the potential use of subpictures in 3600 video use-case scenarios and teleconferencing use-case scenarios does not necessary require subpicture partitioning structure that makes the non-raster scan order of the subpictures required. Decoders that are implemented to target scenarios where non-raster scan order of subpicture is not needed may use the implementation that does not consider the issues above. It would be desired that subpictures are in raster scan order when their structure allow them so and have non-raster scan order only when subpicture structures is in irregular shape.

# Proposal

We propose to simplify subpicture order such that if the structure of subpicture allows both raster scan order and non-raster scan order, the order shall be in raster scan order. For that, we suggest two options a described in the following sub sections.

## Add a constraint

For this option, we suggest that in additional to the current availability rules for subpicture, we add a constraint to ensure that the order of subpictures is in raster scan order whenever possible.

The text implementation of it can be as follows:

...

It is a requirement of bitstream conformance that the shapes of the subpictures shall be such that each subpicture, when decoded, shall have its entire left boundary and entire top boundary consisting of picture boundaries or consisting of boundaries of previously decoded subpictures.

Let subpictures SubpicA and SubpicB be subpictures that belong to a picture, it is a requirement of bitstream conformance that SubpicA shall precede SubpicB when all of the following applies:

- The top offset of the first CTU of SubpicA is less than the top offset of the first CTU of SubpicB, or the top offset of the first CTUs of SubpicA and SubpicB are the same but the left offset of the first CTU of SubpicA is less than the left offset of first CTU of SubpicB.

- SubpicA can be decoded without having SubpicB be decoded first.

...

## Add a general constraint information

For this option, we suggest that have a flag in GCI. With such flag, it is easier to enforce that subpictures are in raster scan order for some certain class of applications without having to change the profile / level definition. The text implementation of it can be as follows:

...

| general_constraints_info( ) { | Descriptor |
| --- | --- |
| gci_present_flag | u(1) |
| if( gci_present_flag ) { |  |
| /* general */ |  |
| gci_intra_only_constraint_flag | u(1) |
| gci_all_layers_independent_constraint_flag | u(1) |
| gci_one_au_only_constraint_flag | u(1) |
| gci_subpictures_in_raster_scan_only_constraint_flag | u(1) |
| ... |  |
| } |  |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| } |  |

...

gci_subpictures_in_raster_scan_only_constraint_flag equal to 1 specifies that subpictures, when present, shall be in raster scan only. gci_subpictures_in raster_scan_only_constraint_flag equal to 0 does not impose such a constraint.

...

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).