| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0112-v1 |
| --- | --- |

| Title: | EE1-1.6-related: Deep In-Loop Filter with Additional Input Information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang, Li Zhang (Bytedance) | Tel: Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In EE1-1.6, a convolutional neural network-based in-loop filtering method is proposed and implemented with SADL using both floating point-based and fixed point-based calculations. In this contribution, two aspects are proposed to improve the CNN models presented in EE1-1.6 with additional input information:

Aspect #1: Forward/backward collocated blocks can be input to the in-loop filtering network.

Aspect #2: The input samples of CNN can be flipped and the output samples of CNN can be flipped back.

The proposed CNN model is implemented using SADL int16 precision. Compared with VTM-11.0 + NewMCTF, the proposed method reportedly show on average {10.67%, 22.05%, 21.67%} BD-rate reductions for {Y, Cb, Cr} under RA configuration, respectively.

# Introduction

In EE1-1.6 [1], the deep learning-based in-loop filtering method was proposed to improve the coding efficiency of VVC. The deep filter in EE1-1.6 is implemented with a standalone C++ library named SADL [2] supporting both floating point-based and fixed point-based calculations.

# Proposed method

This contribution improves the CNN models in EE1-1.6 in two aspects.

First, the collocated blocks from the forward and backward reference frames are additionally fed into the CNN model to facilitate a better exploitation of temporal correlation. Second, it is allowed to flip the input samples of CNN and flip back the output samples of CNN to better align with the geometric feature.

In the proposed method, two highest temporal layers are filtered by a multiple frames-based CNN model, which takes reconstruction, forward collocated block, the backward collocated block, and QP as input. For other temporal layers, the single frame-based CNN model from EE1-1.6 is used.

To better align with the geometric features, horizontal flipping can be applied to input samples of CNN. The reconstruction and all the auxiliary inputs are flipped horizontally and then fed into the CNN. The output of the CNN is then flipped again to get the final filtered samples. Other parts such as parameter selection, residual scaling, combination with deblocking, etc. remain the same as EE1-1.6.

Section 2.1 - 2.3 present the CNN structure, the parameter selection, and the residue scaling, respectively. In addition, section 2.4 presents the inference and section 2.5 describes the training process.

## Architecture

The backbone of the proposed inter CNN filter is shown in Fig. 1. Rec, Pred, Forw, BacW, and QP stand for the reconstruction, the prediction, the forward collocated reference block, the backward collocated reference block, and the sequence-level input quantization parameter respectively. The numbers of feature maps and residual blocks are set as 96 and 8.

Fig. 1. (a) Architecture of the proposed CNN filtering method. M denotes the number of feature maps. N stands for the number of samples in one dimension. (b) Construction of Residual Block in (a)

## Parameter Selection

In the proposed technique, each slice or block could determine whether to apply the CNN-based filter or not. When the CNN-based filter is determined to be applied to a slice/block, which conditional parameter from a candidate list including three candidates derived from QP could be further decided. Denote the sequence level QP as q, the candidate list includes conditional parameters {q, q - 5, q (hor)}. The selection process is based on the rate-distortion cost at the encoder side. Indication of on/off control as well as the conditional parameter index, if needed, are signalled in the bitstream. Other details regarding the parameter selection are provided as below.

- The granularity of the filter determination and the parameter selection is dependent on resolution and QP. In the proposed scheme, given a higher resolution and a larger QP, the determination and selection will be performed in a larger region.

- For all-intra configuration, parameter selection is disabled while on/off control is still preserved

- A shared conditional parameter is used for the two chroma components.

- For the third candidate, the input will be horizontally flipped while quantization parameter is still q.

## Residue Scaling

When a NN filter is being applied to reconstructed pictures, a scaling factor is signaled for each color component in the picture header. The difference between the input samples and the NN filtered samples (residues) are scaled by the scaling factors before being added to input samples. To enable a combination with deblocking, the input samples used in the residual scaling is the output of deblocking filtering. The residual scaling process is shown below, where RNN and RDB refer to the outputs of NN filtering and deblocking filtering respectively.

RRefine = (RNN-RDB)×w + RDB=w×RNN + (1-w)×RDB

## Inference

SADL is used for performing the inference of the proposed CNN filters in VTM. Both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 1 as suggested by [3].

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1.56M/model, 5 models in total |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | 3.1MB/model, 5 models in total |
|  | Multiply Accumulate (kMAC/pixel) | 539K/pixel (assuming frame-level input); 682K/pixel (assuming block-level input) |
| Optional |  |  |
|  | Total Conv. Layers | 25+16 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

## Training

PyTorch is used as the training platform. The DIV2K [4] and BVI-DVC [5] datasets are adopted to train the CNN filters of I slices and B slices, respectively. The network information in the training stage is provided in Table 2 as suggested by [3].

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 2 |
|  | Epoch: | 50 |
|  | Batch size: | 64 |
|  | Training time: | 60h/model |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Experimental results

The proposed CNN-based in-loop filtering method is tested on top of VTM-11.0 + new MCTF [6] according to the common test conditions defined in [3], where the new MCTF patch is from [7]. SAO is disabled while ALF (and CCALF) is placed after the proposed CNN-based filtering. Test results are shown in Table 3. It can be observed that the proposed CNN models could bring on average {10.67%, 22.05%, 21.67%} BD-rate reductions for {Y, Cb, Cr} under RA configuration. For an easier comparison, the performance of EE1-1.6 is shown in Table 4. It can be seen that the proposed method shows 0.47% additional gain on the Y component over EE1-1.6 without extra complexity.

Table 3. RA performance of the proposed method on top of VTM-11.0 + new MCTF

|  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -12.37% | -10.18% | -17.16% | -19.50% | 211% | 50840% |
| Class A2 | -13.30% | -11.34% | -21.44% | -18.34% | 202% | 48958% |
| Class B | -13.10% | -10.18% | -24.55% | -23.21% | 206% | 51317% |
| Class C | -13.66% | -11.15% | -23.04% | -23.87% | 177% | 46171% |
| Class E |  |  |  |  |  |  |
| Overall | -13.14% | -10.67% | -22.05% | -21.67% | 198% | 49332% |
| Class D | -15.38% | -12.97% | -24.89% | -25.08% | 168% | 41802% |
| Class F | -6.81% | -5.36% | -12.75% | -11.81% | 275% | 24135% |

Table 4. RA performance of the EE1-1.6

|  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -12.14% | -9.82% | -17.26% | -19.62% | 216% | 54472% |
| Class A2 | -13.03% | -10.98% | -21.45% | -18.38% | 206% | 51754% |
| Class B | -12.82% | -9.82% | -24.61% | -23.24% | 209% | 53872% |
| Class C | -13.06% | -10.38% | -23.14% | -23.86% | 178% | 48331% |
| Class E |  |  |  |  |  |  |
| Overall | -12.79% | -10.20% | -22.11% | -21.71% | 201% | 52032% |
| Class D | -14.49% | -11.88% | -24.79% | -24.96% | 172% | 43913% |
| Class F | -6.67% | -5.15% | -12.75% | -11.91% | 279% | 25148% |

# Conclusions

This contribution presents the test result of a CNN-based in-loop filtering method. The proposed CNN-based filtering method shows favorable trade-off in terms of the coding performance and complexity. It is suggested to study the proposed method in EE.

# References

- Y. Li, K. Zhang, J. Li, L. Zhang, H. Wang, M. Coban, A.M. Kotra, M. Karczewicz, F. Galpin, K. Andersson, J. Ström, D. Liu, R. Sjöberg, EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation, JVET-AA0111, by teleconference, July 2022

- F. Galpin, T. Dumas, P. Bordes, E. François, AHG11: SADL update, JVET-Z0161, by teleconference, April 2022.

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, by teleconference, April 2022.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-11.0.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master/Software%20Patches

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).