| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0120-v3 |
| --- | --- |

| Title: | Non-EE2: Template-based multiple reference line intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu Yue Yu Haoping Yu Dong Wang | Tel: Email: | xuluhang@oppo.com yue.yu@oppo.com v-yuhaoping@oppo.com wangdong7@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution proposes a template-based multiple reference line intra prediction (TMRL) mode. The proposed mode combines reference line and prediction mode together and uses a template matching method to construct a list of candidate combinations. An index to the candidate combination list is coded to indicate which reference line and prediction mode is used in coding the current block. In this proposal, the regular multiple reference line (MRL) for the non-TIMD part is replaced by the proposed mode.

Compared with ECM5.0 [1] anchor, the proposed method shows coding performance gains as follows.

Test-a:

AI -0.08%/-0.01%/-0.04%, Enc: 102%, Dec: 105%,

RA -0.03%/-0.08%/-0.13%, Enc: 101%, Dec: 101%,

Test-b:

AI -0.13%/-0.07%/-0.05%, Enc: 105%, Dec: 104%,

RA -0.05%/0.02%/-0.05%, Enc: 101%, Dec: 100%.

# Introduction

Multiple reference line (MRL) is used in VVC and an MRL candidate list consists of 2 additional reference lines, i.e., {1, 2}. In JVET-Y0116 [2], an extended MRL candidate list was proposed and later adopted into ECM [3] reference software. The extended MRL allows using more reference lines for intra prediction. In particular, the MRL candidate list is extended from {1, 2} to {1, 3, 5, 7, 12} for regular intra prediction and from {1, 2} to {1, 3} for TIMD intra prediction.

Figure 1. Extended MRL candidate list in the ECM

# Proposed Method

The proposed TMRL mode includes the following aspects:

- the extended reference line candidate list and the intra-prediction-mode candidate list.

The extended reference line candidate list is {1, 3, 5, 7, 12}. The restriction on CTU top row is unchanged. The size of the intra-prediction-mode candidate list is 10. The construction of the intra-prediction-mode candidate list is similar to MPM. The only difference is that PLANAR mode is excluded from the proposed intra-prediction-mode candidate list.

- construction of the TMRL combination list.

There are 5x10=50 combinations of the extended reference line and the allowed intra-prediction modes for a block. Since the extended reference line starts from reference line 1, the area covered by reference line 0 is used for template matching. The SAD costs over the template area (see Figure-2) are calculated between the predictions (generated by 50 combinations) and the reconstructions. Only the first 20 combinations with the least cost are pruned from the combinations in an ascending order to form the combination candidate list.

Figure-2. Illustration of template area

- signaling of TMRL

In ECM or VTM, MRL is always combined with the non-planar intra-prediction modes from primary MPM. When an extended reference line is selected, up to 6 context-coded bins and 3 equi-probable (EP) bins are needed to represent the MRL and MPM index. In the proposed TMRL mode, a truncated Golomb-Rice coding with a divisor 4 is employed to code selected combinations from the combination list. The binarization process and the codewords are shown in Table-2.

Table-2 the binarization process of TMRL index

| Index | Bin string (prefix) | Bin string (suffix) |
| --- | --- | --- |
| 0 | 0 | 00 |
| 1 | 0 | 01 |
| 2 | 0 | 10 |
| 3 | 0 | 11 |
| 4 | 10 | 00 |
| … | … |  |
| 18 | 1111 | 10 |
| 19 | 1111 | 11 |

- encoder-side modifications

Encoder-side modification is optionally tested to further improve the coding efficiency. For intra blocks larger than 8x8, more rounds of full RDO may be allowed to the selected intra prediction modes after comparing their SATD cost with a threshold.

# Simulation Results

Test-a: TMRL without encoder-side changes, the PLANAR mode is excluded.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.13% | 0.06% | 102% | 101% |
| Class A2 | -0.06% | -0.07% | -0.08% | 102% | 106% |
| Class B | -0.06% | -0.04% | -0.01% | 101% | 104% |
| Class C | -0.12% | -0.09% | -0.08% | 102% | 107% |
| Class E | -0.14% | 0.04% | -0.09% | 103% | 108% |
| Overall | -0.08% | -0.01% | -0.04% | 102% | 105% |
| Class D | -0.11% | 0.05% | -0.09% | 102% | 108% |
| Class F | -0.12% | 0.03% | -0.08% | 103% | 107% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | -0.10% | 101% | 101% |
| Class A2 | -0.05% | -0.01% | -0.13% | 101% | 101% |
| Class B | -0.02% | -0.21% | -0.13% | 100% | 100% |
| Class C | -0.04% | 0.03% | -0.17% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.08% | -0.13% | 101% | 101% |
| Class D | -0.03% | -0.01% | 0.09% | 100% | 98% |
| Class F | 0.00% | -0.03% | -0.07% | 102% | 102% |

Test-b: TMRL with encoder-side changes, the PLANAR mode is excluded.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.02% | -0.07% | 105% | 101% |
| Class A2 | -0.12% | -0.13% | -0.07% | 104% | 102% |
| Class B | -0.11% | -0.05% | -0.07% | 105% | 103% |
| Class C | -0.15% | -0.08% | -0.01% | 106% | 107% |
| Class E | -0.18% | -0.07% | -0.05% | 105% | 102% |
| Overall | -0.13% | -0.07% | -0.05% | 105% | 104% |
| Class D | -0.12% | 0.05% | -0.12% | 104% | 98% |
| Class F | -0.21% | -0.13% | -0.35% | 103% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.19% | -0.05% | 101% | 100% |
| Class A2 | -0.08% | 0.02% | 0.02% | 101% | 99% |
| Class B | -0.04% | -0.14% | -0.19% | 101% | 101% |
| Class C | -0.06% | 0.10% | 0.07% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | 0.02% | -0.05% | 101% | 100% |
| Class D | -0.04% | -0.07% | -0.03% | 101% | 100% |
| Class F | -0.05% | -0.13% | -0.26% | 101% | 102% |

# Conclusions

The proposed TMRL can further improve the coding efficiency of ECM with minor complexity increase. It is recommended to further study this method in EE2.

# Reference

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-5.0

- K. Cao, et. al., “EE2-2.1: Extended MRL candidate list,” JVET-Y0116, April 2022.

- M. Coban, et. al., “Algorithm description of Enhanced Compression Model 5 (ECM 5),” JVET-Z2025, July 2022

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).