| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0125-v1 |
| --- | --- |

| Title: | EE2-1.1b and 1.2: Filter-based linear model and gradient linear model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Che-Wei Kuo, Hong-Jheng Jhu, Xiaoyu Xiu, Ning Yan, Wei Chen, Xianglin Wang | Email: | {cheweikuo, jhuhong-jheng, xiaoyuxiu, yanning03, chenwei06, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports results of EE2 Test-1.1b and 1.2. Test-1.1b (filter-based linear model) extends the existing two-parameter linear model in the CCLM to seven-parameter linear model. Meanwhile, four filter shapes are supported and are switched at CU level. Test-1.2 (gradient linear model) uses alternative gradient filters to generate the input luma samples for the CCLM process, instead of the existing luma down-sampling filter.

Compared to ECM-5.0 anchors, the BD-rate impact is summarized as below.

| Class A to E |  | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1b | FLM | -1.27% | -2.01% | -1.75% | 103% | 99% |  |  |  |  |  |
| 1.2 | GLM | -0.82% | -0.51% | -0.61% | 114% | 99% | -0.33% | -0.17% | -0.46% | 107% | 99% |

| Class F |  | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1b | FLM | -0.68% | -2.76% | -2.71% | 102% | 99% |  |  |  |  |  |
| 1.2 | GLM | -0.44% | -2.61% | -2.27% | 108% | 100% | -1.23% | -2.42% | -2.22% | 105% | 100% |

| Class TGM |  | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1b | FLM | -2.37% | -4.65% | -4.55% | 102% | 98% |  |  |  |  |  |
| 1.2 | GLM | -3.48% | -9.76% | -9.87% | 109% | 98% | -2.36% | -4.56% | -4.83% | 107% | 100% |

# Introduction

At the JVET-Z meeting, in the same spirit of cross-component linear model (CCLM), filter-based linear model (FLM) and gradient linear model (GLM) were proposed in [1] to predict the chroma samples from reconstructed luma samples. Specifically, the FLM extends the two-parameter linear model in the CCLM to seven-parameter linear model. The GLM uses the gradients of the reconstructed luma samples to predict the chroma samples based on the same linear model (i.e., slope and offset) of the CCLM. The two methods were included in Exploration Experiment (EE) 2 as Test-1.1b and Test-1.2 for further evaluation. This contribution reports the corresponding results of two EE test.

# Proposal

## Filter-based Linear Model

The FLM extends the Simple Linear Regression (SLR) in the CCLM to Multiple Linear Regression (MLR), which can be formulated as

C=i=0N-1αi∙Li+β

where C is the to-be-predicted chroma sample; Li is the i-th reconstructed down-sampled luma sample surround the chroma sample, αi is the coefficient associated with Li, β is the offset, and N is the number of the used down-sampled luma samples. Similar to convolutional cross-component model (CCCM) [3] in EE2 Test-1.1a, up to 6 lines/columns of chroma samples above and left to the current CU are applied to the FLM filter coefficients. In the current design, the FLM is only applicable to CUs containing no less than 128 chroma samples and 4 FLM filter shapes (with N equal to 2 or 6) are supported which are adaptively switched at CU level. Additionally, the FLM can be combined with multi-model linear model (MMLM), but is always disabled with multi-directional linear model (MDLM). The combination of the FLM and the MMLM is only allowed for CUs with at least 256 chroma samples.

For signaling, one flag is signaled for two chroma components to indicate the usage of the FLM. If the FLM is enabled, one index is further signaled to select one from the 4 FLM filter shapes.

## Gradient Linear Model

Compared with the CCLM, instead of down-sampled luma values, the GLM utilizes luma sample gradients to derive the linear model. Specifically, when the GLM is applied, the input to the CCLM process, i.e., the down-sampled luma samples L, are replaced by luma sample gradients G. The other parts of the CCLM (e.g., parameter derivation, prediction sample linear transform) are kept unchanged.

C=α∙G+β

For signaling, when the CCLM mode is enabled to the current CU, two flags are signaled separately for Cb and Cr components to indicate whether GLM is enabled to each component; if the GLM is enabled for one component, one syntax element is further signaled to select one of 16 gradient filters (as illustrated in Appendix) for the gradient calculation. The GLM is allowed to be combined with the existing CCLM by signaling one extra flag in bitstream. When such combination is applied, the filter coefficients that are used to derive the input luma samples of the linear model is calculated as the combination of the selected gradient filter of the GLM and the down-sampling filter of the CCLM.

# Simulation results

The proposed techniques were implemented on ECM-5.0 reference software [4] and tested by following the common test condition (CTC) as specified in [5].

## Test 1.1b

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |  |
|  | Y | U | V | EncT |  | DecT |
| Class A1 | -0.79% | -1.72% | -2.41% | 103% | 98% |  |
| Class A2 | -5.71% | -3.23% | -6.12% | 104% | 98% |  |
| Class B | -0.44% | -2.03% | -0.41% | 103% | 99% |  |
| Class C | -0.07% | -0.28% | -0.23% | 104% | 99% |  |
| Class E | -0.28% | -3.35% | -0.97% | 103% | 98% |  |
| Overall | -1.27% | -2.01% | -1.75% | 103% | 99% |  |
| Class D | -0.04% | 0.10% | -0.09% | 102% | 100% |  |
| Class F | -0.68% | -2.76% | -2.71% | 102% | 99% |  |
| Class TGM | -2.37% | -4.65% | -4.55% | 102% | 98% |  |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |  |
|  | Y | U | V | EncT |  | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  | #VALUE! |
| Class B | -0.18% | -1.21% | -0.23% | 101% |  | 99% |
| Class C | -0.05% | -0.05% | -0.15% | 102% |  | 100% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  | #VALUE! |
| Class D | 0.02% | 0.19% | 0.30% | 102% |  | 100% |
| Class F | -1.00% | -2.39% | -2.42% | 101% |  | 99% |
| Class TGM | -1.42% | -2.00% | -1.95% | 101% |  | 99% |

## Test1.2

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.13% | 0.50% | 115% | 98% |
| Class A2 | -4.69% | -1.78% | -4.81% | 116% | 97% |
| Class B | -0.11% | -0.45% | 0.31% | 112% | 99% |
| Class C | 0.02% | 0.02% | 0.05% | 115% | 101% |
| Class E | -0.04% | -0.41% | 0.04% | 112% | 99% |
| Overall | -0.82% | -0.51% | -0.61% | 114% | 99% |
| Class D | 0.01% | 0.59% | 0.17% | 115% | 104% |
| Class F | -0.44% | -2.61% | -2.27% | 108% | 100% |
| Class TGM | -3.48% | -9.76% | -9.87% | 109% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.18% | 0.24% | 109% | 100% |
| Class A2 | -1.56% | -0.72% | -2.36% | 105% | 97% |
| Class B | -0.04% | -0.32% | -0.23% | 108% | 100% |
| Class C | 0.00% | 0.18% | 0.14% | 107% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.17% | -0.46% | 107% | 99% |
| Class D | -0.01% | 0.18% | 0.30% | 106% | 102% |
| Class F | -1.23% | -2.42% | -2.22% | 105% | 100% |
| Class TGM | -2.35% | -4.56% | -4.83% | 107% | 100% |

# References

- C.-W. Kuo, X. Xiu, N. Yan, H.-J. Jhu, W. Chen, H. Gao, X. Wang, “AHG12: Enhanced CCLM”, JVET-Z0140, Apr. 2022, Teleconference.

- P. Astola, J. Lainema, R. G. Youvalari, A. Aminlou, K. Panusopone, “EE2-1.1a: Convolutional cross-component intra prediction model”, JVET-AA0057, July 2022, Teleconference.

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Apr. 2022.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

Figure 3 16 gradient patterns for GLM.