| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0133-v1 |
| --- | --- |

| Title: | EE2 – 4.1: Inter-MTS optimization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | EE2 |  |  |
| Author(s) or Contact(s): | Bappaditya Ray Vadim Seregin Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121, USA | Email: | bray@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results for EE test 4.1 (4.1a and 4.1b) for Inter-MTS optimization.

Test 4.1.a (with KLT):

RA: -0.24% (Y)/ 0.31% (U)/ 0.31%(V), EncT: 101%, DecT: 99%

LDB: -0.24% (Y)/0.36%(U)/0.27% (V), EncT: 101%, DecT: 99%

Test 4.1.b (without KLT):

RA: -0.15% (Y)/ 0.27% (U)/ 0.28%(V), EncT: 101%, DecT: 99%

LDB: -0.07% (Y)/0.21%(U)/0.09% (V), EncT: 101%, DecT: 98%

# Introduction

Currently, in ECM-5.0, for the MTS of inter-coded CUs, four candidates: {(DST7, DST7), (DST7, DCT8), (DCT8, DST7), (DCT8, DCT8)} are used for every CU. In JVET-Z0147, the following modification of Inter-MTS is done.

- For the larger resolution sequences (width > 1080) maximum CU size for Inter-MTS usage is set to 32 (i.e., Inter-MTS is used for CU with width <=32 and height <=32), and for the remaining sequences (smaller resolution) it is set to 16.

- For 4-pt, 8-pt and 16-pt transforms, the current AMT transform cores, i.e., DST-7 and DCT-8, is replaced with separable KLTs, as proposed in JVET-J0021.

# Results

The proposed method was implemented on top ECM-5.0. The anchor is run with CTC condition [1] and for the test, Inter-MTS is enabled (MTS: 3). Two tests are carried out and the results of the proposed method are summarized below.

Test 4.1a: for blocks up to size 16, DST7/DCT8 is replaced with separable KLT.

Test 4.1b: results without KLT. DST7/DCT8 for all blocks.

Test 4.1a (with KLT):

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | 0.20% | 0.18% | 101% | 98% |
| Class A2 | -0.35% | 0.44% | 0.40% | 102% | 98% |
| Class B | -0.22% | 0.28% | 0.29% | 100% | 99% |
| Class C | -0.26% | 0.33% | 0.37% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.24% | 0.31% | 0.31% | 101% | 99% |
| Class D | -0.29% | 0.16% | 0.25% | 102% | 99% |
| Class F | -0.09% | 0.11% | 0.12% | 101% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.24% | 0.20% | 0.39% | 100% | 98% |
| Class C | -0.38% | 0.19% | 0.18% | 103% | 99% |
| Class E | -0.07% | 0.86% | 0.19% | 99% | 99% |
| Overall | -0.24% | 0.36% | 0.27% | 101% | 99% |
| Class D | -0.52% | 1.15% | 1.10% | 102% | 98% |
| Class F | -0.35% | -0.17% | 0.36% | 100% | 100% |

Test 4.1b (without KLT):

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.20% | 0.16% | 102% | 99% |
| Class A2 | -0.31% | 0.30% | 0.36% | 101% | 99% |
| Class B | -0.10% | 0.24% | 0.19% | 100% | 99% |
| Class C | -0.13% | 0.35% | 0.42% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.15% | 0.27% | 0.28% | 101% | 99% |
| Class D | -0.12% | 0.17% | 0.33% | 103% | 99% |
| Class F | -0.02% | 0.17% | 0.13% | 101% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.08% | 0.13% | 101% | 98% |
| Class C | -0.16% | 0.39% | 0.35% | 103% | 98% |
| Class E | 0.01% | 0.17% | -0.31% | 99% | 99% |
| Overall | -0.07% | 0.21% | 0.09% | 101% | 98% |
| Class D | -0.10% | 1.20% | 1.05% | 103% | 99% |
| Class F | -0.17% | -0.29% | 0.00% | 100% | 99% |

# References

[1] M. Karczewicz, Y. Yen, "Common Test Conditions and evaluation procedures for enhanced compression tool testing", JVET-X2017.

[2] B. Ray, V. Seregin, M. Karczewicz, “Non-EE2: inter MTS improvement in ECM”, JVET-Z0147.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).