| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0134-v1 |
| --- | --- |

| Title: | Non-EE2: POC based BCW weights derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang, Han Huang, Chun-Chi Chen, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to add BCW weights derived considering POC difference between reference and current pictures. The proposed change is used for AMVP and merge modes. It was implemented on top of ECM-5.0 and tested with and without BcwFast encoder speedup configuration. The test results are summarised as follows:

BcwFast is set to 1:

RA: -0.xx% (Y), -0.xx% (U), -0.xx% (V), 1xx% (EncT), 1xx% (DecT).

LDB: -0.xx% (Y), -0.xx% (U), -0.xx% (V), 1xx% (EncT), 1xx% (DecT).

BcwFast is set to 0:

RA: -0.xx% (Y), -0.xx% (U), -0.xx% (V), 1xx% (EncT), 1xx% (DecT).

LDB: -0.xx% (Y), -0.xx% (U), -0.xx% (V), 1xx% (EncT), 1xx% (DecT).

# Introduction

In ECM-5.0, BCW uses an index to select a weight value to be used in bi-prediction weighting to code the current block. Depending on whether the current picture is a low-delay picture (all reference pictures are from the past relatively to the current picture), BCW weight is selected from { -2, 3, 4, 5, 10 }, or from { 3, 4, 5 }, otherwise.

When the current picture is a non-low-delay picture, it is possible to have two reference pictures both from the past or both from the future relatively to the current picture. In this scenario, a block may prefer to use negative weight value on one reference picture, however, in the current BCW design, only positive weight values can be used for a non-low-delay picture.

# Proposal

This contribution proposes to add BCW weights derived based on POC difference as follows:

- If both reference pictures are from the past or from the future relatively to the current picture, the weight pair either (-2, 10) or (-3, 11) is added based on low-delay conditions.

- Otherwise, the (2, 6) weight pair is added.

The larger value from the weight pair is assigned to the closest POC reference picture or list 0 reference picture when the POC distance is the same.

The BCW index list size is increased by one when the proposed method is applied, and the proposed method is not used with MHP for AMVP mode. The proposed BCW weight is assigned to pair-wise and zero merge candidates in addition to BCW default weight.

# Simulation results

The proposed method was implemented on top of ECM-5.0. The simulation was conducted under the common test conditions as defined in [2].  Two tests are performed with and without BcwFast encoder setting controlled by config parameter.

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Test 1 |  |  |  |  | Test 2 (BcwFast = 0) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.11% | -0.22% | -0.13% | 103% | 100% | -0.19% | -0.30% | -0.32% | 117% | 100% |
| Class C | -0.08% | 0.07% | -0.08% | 102% | 100% | -0.12% | -0.11% | -0.02% | 114% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.06% | -0.28% | 0.04% | 101% | 100% | -0.11% | -0.21% | -0.07% | 112% | 101% |
| Class F | -0.16% | -0.22% | -0.21% | 102% | 100% | -0.28% | -0.31% | -0.27% | 113% | 100% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Test 1 |  |  |  |  | Test 2 (BcwFast = 0) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.02% | -0.17% | 0.19% | 102% | 100% | -0.11% | -0.26% | -0.24% | 128% | 101% |
| Class E | -0.10% | 0.10% | 0.40% | 108% | 101% | -0.21% | -0.08% | -0.29% | 142% | 101% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.08% | 1.11% | 0.24% | 102% | 99% | -0.16% | 0.91% | 0.86% | 123% | 100% |
| Class F |  |  |  |  |  |  |  |  |  |  |

# References

[1] ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).