| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0141_v2 |
| --- | --- |

| Title: | Non-EE2: Enhanced temporal motion information derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This proposal proposes two aspects to improve the temporal motion information derivation process for sub-block-based TMVP (sbTMVP) and AMVP modes. Firstly, two collocated frames are utilized to provide temporal motion information. Secondly, the motion shift to locate TMVP is adaptively determined from multiple locations according to template costs. On top of ECM-5.0, simulation results of the proposed method are reported as below:

RA: {-0.10%, -0.10%, -0.16%; 105%, 103%};

LDB: {-0.20%, -0.20%, -0.23%; 112%, 108%}.

# Introduction

In ECM, the temporal MVP for AMVP mode is derived by fetching the motion information from center or bottom-right location in the collocated frame. And a similar strategy is also applied to sbTMVP mode, where the motion information from the left neighbouring position is used as a motion shift, which is then employed to obtain a MVP at sub-CU level for the to-be-coded CU. It is asserted that such a design may not ensure the trajectory consistency between the pre-defined positions and current CU.

# Proposed methods

In this proposal, two aspects are proposed to further improve TMVP in sbTMVP and AMVP modes. Firstly, two collocated frames are utilized to provide temporal motion information. Secondly, the motion shift to locate TMVP is adaptively determined from multiple locations according to template costs.

When constructing sub-block-based merge candidate list and AMVP candidate list, two reference frames with the least POC distance relative to the to-be-coded frame are determined to be the collocated frames.

For the two collocated frames, two motion shift candidate lists are constructed respectively. When constructing the motion shift list Li for the i-th collocated frame Ci, the motion information of existing candidates in the motion candidate list are checked. If either MV of the motion candidate points to Ci, the corresponding MV is included into Li serving as a motion shift candidate. The motion shift with the minimum template matching cost is used to derive sbTMVP or TMVP candidate.

ARMC [1] for the sub-block-based merge candidate list is modified accordingly since one more sbTMVP candidate is included. In the proposed method, the sbTMVP candidate that are derived from the first collocated frame is placed in the first entry without reordering. While the other sbTMVP candidate is sorted together with AFFINE candidates.

# Experimental results

The proposed method has been implemented on top of the ECM-5.0 [2]. Results under RA/LDB configurations are presented in Table 1. The simulations were performed in accordance with the common test conditions [3]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method (tool-off test).

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.16% | -0.14% | 105% | 103% |  |  |  |  |  |
| Class A2 | -0.11% | -0.00% | -0.14% | 104% | 103% |  |  |  |  |  |
| Class B | -0.05% | -0.13% | -0.16% | 105% | 103% | -0.08% | -0.08% | -0.08% | 114% | 107% |
| Class C | -0.18% | -0.07% | -0.18% | 107% | 104% | -0.08% | -0.08% | -0.08% | 114% | 109% |
| Class E |  |  |  |  |  | -0.23% | 0.32% | -0.15% | 106% | 107% |
| Overall | -0.10% | -0.10% | -0.16% | 105% | 103% | -0.20% | -0.20% | -0.23% | 112% | 108% |
| Class D | -0.20% | -0.29% | 0.05% | 106% | 104% | -0.19% | 0.75% | -0.41% | 112% | 110% |
| Class F | -0.08% | -0.08% | -0.08% | 102% | 103% | -0.08% | -0.08% | -0.08% | 105% | 105% |

# Conclusions

Enhanced temporal motion information derivation methods are presented in this proposal. Simulation results indicate 0.10%/0.20% coding gain in RA/LDB configurations with 105%/103% and 112%/108% complexity increase. It is recommended to include the proposed methods in the next round of EE.

# References

[1] N. Zhang, K. Zhang, L. Zhang, H. Liu, Z. Deng, Y. Wang, “AHG12: Adaptive Reordering of Merge

Candidates with Template Matching,” JVET-V0099, Apr. 2021.

[2] ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-5.0.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).