| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0144x-v1 |
| --- | --- |

| Title: | Non-EE2: DMVR for affine merge coded blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen Ru-Ling Liao Xinwei Li Yan Ye | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group. |  |  |

_____________________________

- Abstract

This contribution proposes to apply DMVR on the blocks coded with affine merge mode. For simplicity, only the CU level base motion vector refinement is proposed in this contribution. It is reported that applying proposed DMVR on affine blocks achieves {-0.12%(Y), -0.02%(U), -0.09%(V)} coding gain in RA on average, and luma coding gain of -0.16% on class A1 and -0.27% on classA2.

Introduction

In ECM[1], a multi-pass decoder-side motion vector refinement is applied. In the first pass, bilateral matching (BM) is applied to the coding block. In the second pass, BM is applied to each 16x16 subblock within the coding block. In the third pass, MV in each 8x8 subblock is refined by applying bi-directional optical flow (BDOF). The refined MVs are stored for both spatial and temporal motion vector prediction. The DMVR is applied to refine the motion vector inherited from the previously coded blocks in merge mode as the motion information of the previously coded blocks may not be perfectly fit for the current block.

However, the DMVR in current ECM is only applied on non-affine motion compensation blocks. For affine blocks, they may also be coded with affine merge mode in which the control point motion vectors are inherited from or constructed by the motion information of the adjacent or non-adjacent previously coded blocks. Thus, DMVR can also be used in this case to refine the motion of affine blocks.

In this contribution, it is proposed to apply DMVR on affine merge blocks.

Proposed method

Generally, affine model can be described using the following equations

mvx=mv1x-mv0xWx+mv2x-mv0xHy+mv0xmvy=mv1y-mv0yWx+mv2y-mv0yHy+mv0y				(1)

wherein(mvx,mvy) is the motion vector at location (x,y), (mv0x, mv0y) is the base MV representing the translation motion of the affine model, and mv1x-mv0xW, mv2x-mv0xH, mv1y-mv0yW and mv2y-mv0yH are four non-translation parameters which defines rotation, scaling and other non-translation motion of the affine model. In ECM, besides 6-parameters affine model defined as (1), we also have 4-parameters affine mode described as (2) in which only two non-translation parameters are used.

mvx=mv1x-mv0xWx+mv0y-mv1yWy+mv0xmvy=mv1y-mv0yWx+mv1x-mv0xWy+mv0y				(2)

In this contribution, it is proposed to refine the base MV of the affine model of the coding block coded with the affine merge mode by only applying first step of multi-pass DMVR. That is, we add a translation MV offset to all the CPMVs of the candidate in the affine merge list if the candidate meets the DMVR condition. And the MV offset is derived by minimizing the cost of bilateral matching which is the same as conventional DMVR. And the DMVR condition is also not changed.

The MV offset searching process is the same as the first pass of multi-pass DMVR in ECM. 3x3 square search pattern is used to loop through the search range [-8, +8] in horizontal direction and [-8, +8] in vertical direction to find the best integer MV offset. And then half pel search is conducted around the best integer position and an error surface estimation is performed at last to find a MV offset with 1/16 precision.

The refine CPMV is stored for both spatial and temporal motion vector prediction as the multi-pass DMVR in ECM.

Simulation results

Coding performance

The tests are implemented on top of ECM5.0. The performance is evaluated based on ECM common test conditions in JVET-Y2017[2]. Following two tests are conducted.

The simulation results are shown in Table 1.

Table 1: Results of test 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.02% | -0.15% | 119% | 110% |
| Class A2 | -0.27% | -0.09% | -0.13% | 117% | 112% |
| Class B | -0.08% | -0.03% | -0.06% | 110% | 105% |
| Class C | -0.03% | 0.04% | -0.03% | 107% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.02% | -0.09% | 112% | 106% |
| Class D | -0.06% | -0.15% | 0.02% | 104% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Decoder statistics

The percentage of CUs coded with affine merge mode is collected in the decoder side both for ECM-5.0 anchor and this contribution. The data is given in Table 2. And compares the percentage number between ECM-5.0 and this contribution for each sequence. It is obvious that by applying DMVR on affine merge coded blocks, the percentage of the affine merge mode is increased from 5.83% to 7.90% on average, and especially high for sequences with affine motion. This shows the effectiveness of the proposed method.

Table 2: Percentage of affine merge coded CUs

| Sequence | ECM-5.0 | JVET-AA0144 |
| --- | --- | --- |
| Tango2 | 3.62% | 6.87% |
| FoodMarket4 | 11.08% | 15.74% |
| Campfire | 2.16% | 2.42% |
| CatRobot1 | 4.44% | 6.29% |
| DaylightRoad2 | 8.62% | 13.64% |
| ParkRunning3 | 14.99% | 20.66% |
| MarketPlace | 9.57% | 13.07% |
| RitualDance | 6.66% | 8.35% |
| Cactus | 4.82% | 5.79% |
| BasketballDrive | 4.68% | 5.71% |
| BQTerrace | 3.53% | 3.55% |
| BasketballDrill | 1.52% | 1.81% |
| BQMall | 1.62% | 2.37% |
| PartyScene | 6.92% | 7.89% |
| RaceHorses | 3.25% | 4.37% |
| Avg. | 5.83% | 7.90% |

Figure 1. Comparison of percentage of affine merge mode

Conclusion

This contribution proposed to refine the base MV of affine model for affine merge coded blocks. The refinement process is the same as the first step of multi-pass DMVR in the current ECM. Experimental results showed that by doing it, it achieves {-0.12%, -0.02%, -0.09%} coding gain with 112%/106% encoding/decoding time on average, with higher coding gain for 4K sequences. Thus, it is suggested to study this contribution in the next round of EE.

References

M. Coban, F. Le Léannec, K. Naser, J. Ström, “Algorithm description of Enhanced Compression Model 5 (ECM 5)”, JVET-Z2025, April. 2022

M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).