| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0147 |
| --- | --- |

| Title: | EE2-Related: Extended Offline-Filtering Taps for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Zhipin Deng1, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

In the current adaptive loop filter (ALF) design, online-trained filters contain 2 kinds of taps: spatial taps and offline-filtered taps. In ECM-5.0, an online-trained filter contains 2 offline-filtered taps. In this contribution, the number of offline-filtered taps is extended to 8.

On top of ECM-5.0, simulation results are reported as below:

AI: %, %, %, %, %.

RA: %, %, %, %, %.

LB: %, %, %, %, %.

When combining with EE2-5.1b, simulation results compared with ECM-5.0 are reported as below:

AI: %, %, %, %, %.

RA: %, %, %, %, %.

LB: %, %, %, %, %.

Introduction

In ALF of ECM-5.0, online-trained filters consist of 2 kinds of filter taps: spatial taps and offline-filtered taps, as shown in Figure 1-1. Spatial taps (i.e., tap #0 ~ tap #19) located in a diamond shape are followed by 2 additional offline-filtered taps (i.e., tap #20 ~ tap #21), resulting in 22 taps in total.

Offline-filtered taps are introduced to provide additional information for ALF luma filtering. But the extra information is limited since there are only 2 offline-filtered taps. In this contribution, a straightforward extension of offline-filtered taps is proposed to further enhance the performance of ALF.

Figure 1-1 The Currently Used Filter Shape in ALF.

Proposed method

The proposed filtering taps are shown in Figure 2-1, where spatial taps (i.e., tap #0 ~ tap #19) are kept the same with a diamond shape, and more offline-filtered taps (i.e., tap #20~tap #27) are introduced into luma online-trained filters. The coefficient signaling mechanism is the same as the current design in ECM and there is no filter shape switching.

We further combine the newly introduced extension with EE2-5.1b [1] to form a new and more efficient filter shape for ALF, which is shown in Figure 2-2.

Figure 2-1 The Newly Introduced Filter Shape for ALF.

Figure 2-2 Combination of the Proposed Method with EE2-5.1b

Simulation results

The proposed method was implemented on top of the ECM-5.0 [2]. Simulations were performed following the common test conditions [3]. More detailed results could be found in the attached excel sheets. The results compared to ECM-5.0 are shown in TABLE 3-1. The results of the combination with EE2-5.1b compared to ECM-5.0 are shown in TABLE 3-2.

TABLE 3-1 Performance of the proposed method

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.04% | 0.05% | 0.05% | 102% | 104% | -0.22% | -0.14% | 0.16% | 102% | 104% | -0.24% | 0.32% | -0.05% | 103% | 103% |
| Class F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

TABLE 3-2 Performance of the proposed method combined with EE2-5.1B

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.06% | 0.08% | 0.06% | 103% | 103% | -0.56% | -0.15% | 0.03% | 103% | 102% | -0.57% | 1.05% | 0.40% | 102% | 99% |
| Class F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Conclusions

In this contribution, an extension of offline-filtered taps for ALF is proposed. The proposed method can achieve a promising coding performance with negligible encoding time and decoding time increase. It is recommended to investigate the proposed method in the next round of EE2.

Reference

- W. Yin, K. Zhang and L. Zhang, “Non-EE2: Adaptive Filter Shape Switch for ALF”, JVET-Z0149, April 2022.

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-5.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).