| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0160-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AA0126 (EE2-1.3a: Combined tests of EE2-1.1a and 1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Zhipin Deng | Tel: Email: | zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-AA0126 EE2-1.3a. The simulation results reportedly match those provided by the proponents.

# Verification

JVET-AA0126 EE2-1.3a [1] reports the results of the combination of convolutional cross-component model (CCCM) and gradient linear model (GLM). In the combined test, the CCCM is the same as EE2-1.1a, and for GLM, only four out of sixteen gradient filters are enabled. Meanwhile, the combination of gradient filters with the existing CCLM down-sampling filter is disabled.

We verified that

- The source code implements match with what was described in the proposal.

- Simulation results match with those provided in the proposal.

The following test was arranged for the crosscheck.

- EE2-1.3a.: The combination of CCCM and GLM

The coding results of EE2-1.3a under the ECM CTC are summarized in Table 1.

Table 1: Performance of EE2-1.3a over ECM-5.0 (CTC)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.21% | -2.70% | -4.81% | 106% | 106% |
| Class A2 | -5.57% | -4.72% | -7.60% | 109% | 105% |
| Class B | -0.61% | -3.65% | -1.90% | 105% | 107% |
| Class C | -0.12% | -0.88% | -0.75% | 106% | 107% |
| Class E | -0.35% | -3.44% | -1.25% | 106% | 108% |
| Overall | -1.38% | -3.02% | -2.97% | 106% | 107% |
| Class D | -0.07% | -0.15% | -0.63% | 107% | 98% |
| Class F | -1.05% | -4.46% | -3.97% | 104% | 100% |
| Class TGM | -4.16% | -10.34% | -10.13% | 104% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.13% | -2.22% | -4.13% | 110% | 102% |
| Class A2 | -2.09% | -2.49% | -4.25% | 108% | 101% |
| Class B | -0.37% | -3.60% | -2.46% | 114% | 111% |
| Class C | -0.04% | -0.70% | -0.52% | 112% | 118% |
| Class E |  |  |  |  |  |
| Overall | -0.78% | -2.33% | -2.63% | 111% | 109% |
| Class D | -0.02% | -0.74% | -0.41% | 110% | 107% |
| Class F | -1.62% | -3.78% | -4.09% | 112% | 115% |
| Class TGM | -2.65% | -5.03% | -4.98% | 102% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -1.42% | -0.64% | 107% | 103% |
| Class C | -0.01% | -0.31% | 0.19% | 113% | 113% |
| Class E | -0.29% | -3.98% | -1.87% | 112% | 126% |
| Overall | -0.10% | -1.69% | -0.67% | 110% | 111% |
| Class D | -0.11% | 0.39% | -0.36% | 110% | 110% |
| Class F | -0.67% | -2.35% | -2.06% | 109% | 110% |
| Class TGM | -0.87% | -1.75% | -1.79% | 101% | 98% |

The BD rate differences match with the results from the proposal [1]. Note that timing results are not accurate in this report, since the crosscheck tests and anchor test were launched in different time slots with different simulation environments.

# References

P. Astola, J. Lainema, R. G. Youvalari, A. Aminlou, K. Panusopone (Nokia), C.-W. Kuo, H.-J. Jhu, X. Xiu, N. Yan, W. Chen, X. Wang (Kwai), “EE2-1.1c, 1.3a and 1.3b: Combined tests of EE2-1.1a, 1.1b and 1.2”, JVET-AA0126, July 2022.