| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0174-v1 |
| --- | --- |

| Title: | [EE1] Crosscheck of training stage for EE1-1.5 and EE1-1.6 tests |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Johannes Sauer Biao Wang Elena Alshina | Email: | johannes.sauer@huawei.com biao.wang@huawei.com elena.alshina@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This document reports difficulties encountered during the attempted crosschecks of EE1-1.5 and EE1-1.6 and suggests to align the training methods for both experiments. Training data preparation is the most time consuming part, it takes thousands of hours. Without unification at least naming convention, number of coded frames, QPs training cross-check efforts are double.

# Crosscheck of EE1-1.5

In general the author made a great job to introduce the training script. It consists three steps as shown below:

- generate raw data

- generate compression data

- training tasks

In each step a corresponding readme.md file is provided. In step 1, the provided script convert raw dataset (such as PNG) to yuv formats. In step 2, it is required to generate dataset for 1st and 2nd stage training. With macro in the provided software JVET-X0054, three different version of VVC are used to generate these training datasets. Figure 1 provides an overview of these three configurations using different yuv input based on the result generated in step 1.

| Frame Type | I frames | B frames |  |
| --- | --- | --- | --- |
| Dataset | DIV-2K (900) | BVI Class B (200 1080p) | TVD 10bit (74 4k) |
| 1st stage | Anchor | Anchor* |  |
| 2nd stage | Anchor | JVET-X0054 |  |

Figure 1: dataset generation using different configurations

Wherein Anchor, Anchor* and JVET-X0054 are configured as (1, 0), (2, 0) and (2, 1) for macro (DATA_GENERATIVE_TYPE, NN_FILTER) correspondingly.

We found this process is time consuming. For a powerful desktop server (14 cores of Intel i9-10940 CPU with two NVIDIA GTX Titan GPUs), it is estimated 2500+ hours are required based on Figure 2.

Figure 2: estimate time needed for step 2

For both TVD and BVI ClassB, they are required to generate training data for both stage 1 and 2. We run 4 and 2 sequences with all five QPs and get the encoding and decoding time. Based on this partial time, we estimated the overall time for all sequences in TVD (74 4K sequences) and BVI ClassB (200 1080p). Furthermore, this generating process is error prone when we do the encoding in parallel, probably because the memory consumption is too high when encoding several 4K sequences in parallel. Because of this, we are not able to finish step 2, but complete several sequences only for stage 1 and 2 in step 2.

Based on the crosscheck activity, we suggested to reduce the overhead for training dataset preparation stage in step 2. Since this is the most time (and memory) consuming part, it is desirable to unify this step for training all relevant NNVC technologies.

# Crosscheck of EE1-1.6

## Data pre-processing for intra

For Intra the DIV2K dataset is used, which is in png format. To convert to YUV a matlab function is used along with the down-sampling used by CCLM, but for chroma. We could not reproduce this, since we do not have a matlab licence in our lab. Further, it is unclear how to apply the down-sampling, some code should be provided for this as well.

We suggest to use ffmpeg for the conversion to YUV420 instead. We found a command as follows can be used to do the conversion:

### BT601
ffmpeg -i DIV2K/DIV2K_train_HR/0001.png  -vf scale=out_color_matrix=bt601:flags=full_chroma_int+accurate_rnd,format=yuv420p10le yuv420p_601.yuv

### BT709
ffmpeg -i DIV2K/DIV2K_train_HR/0001.png  -vf scale=out_color_matrix=bt709:flags=full_chroma_int+accurate_rnd,format=yuv420p10le yuv420p_709.yuv

Further, the following settings may obtain a slightly better PSNR, but we did not do an extensive test (only one image):

ffmpeg -i DIV2K/DIV2K_train_HR/0001.png  -vf scale=out_color_matrix=bt601:flags=area+full_chroma_int+accurate_rnd,format=yuv420p10le yuv420p_601_plusAreaFlag.yuv

Although we could not reproduce the pre-processing, we could still continue with the crosscheck, since the proponent provided the already pre-processed files as download.

We encoded the provided YUV files with the provided VTM based software and decoded the bitstreams in order to generate the data required for intra model training. Some script had to be written for this.

## Intra model training

When trying to run intra model training, we found that code for data loading was missing. The proponent indicated in their description that their training scripts are highly adapted to their cluster. However, without a basic version of the training script than can be run on regular system (even if this may take a long time) we were not able to implement a training script. In particular the class KVReader is used in the provided training script, but the implementation for it was not provided. We were not able to replace it.

# Suggestions for next experiment

Table 1 provides a comparison of various aspects of the training methods of EE1-1.5 and EE1-1.6. The methods differ in various aspects. Since the training strategy can have a significant impact of the performance of NN-based technologies we suggest to align the methods for the next experiment. This will enable a fairer comparison, i.e. evaluate benefits of a given model, not of the training strategy. Further it may be possible to reduce required storage (YUV for BVI ~400Gb). Table 2 and Table 3 show suggestions of how some of the aspects can be aligned, not even including training hyper parameters. Alignment of the latter is also desirable, but will need discussion.

Table 1: Comparison of training methodology for EE1-1.5 and EE1-1.6

| Aspect | Tencent Intra | ByteDance Intra | Tencent Inter | ByteDance Inter |
| --- | --- | --- | --- | --- |
| Data set | DIV-2K | DIV-2K first 800 training (plus 800 via ds by factor 2) last 100 validation | BVI (class B), TVD 10 bits | BVI (class B), TVD 10 bits last frame duplicated to get 65 frames only 620 of 800 sequences used (15 of the 620 for validation, rest training) 20 sequences of 2160p and all sequences of other resolutions are used. |
| RGB YUV | ffmpeg default BT 601 | rgb2ycbcr ds of chroma: like in CCLM, meaning what? second 800 images: cv2.resize(img, dsize=(w/2, h/2), interpolation=cv2.INTER_AREA) | ffmpeg default BT 601 |  |
| VTM Version | NNVC anchor | NNVC anchor | NNVC anchor and NNVC anchor +JVET- X0054 | NNVC anchor + intra models (and inter for 3 phase) |
| QPs | 22 27 32 37 42 | 17, 22, 27, 32, 37, 42 | 22 27 32 37 42 | 17, 22, 27, 32, 37, 42 |
| #frames | 1 | 1 | TVD 65 frames BVI-DVC 64 frames | 65 |
| cfg | encoder_intra_vtm.cfg | encoder_intra_vtm.cfg | encoder_randomaccess_vtm.cfg | encoder_randomaccess_vtm.cfg |
| Naming Convention | Bin_CodingOption_TrainingStageCodecCfg_SeqKey_Rate+startframe_framesTobeCoded_TemporalSubsampleRatio e.g. Bin_--LoopFilterDisable=1--SAO=0_T1AI_A_S0658_R2_qp27_s0_f1_t1.bin | example: 0001.cfg ?! | Bin_ TrainingStageCodecCfg _SeqKey_Rate+startframe_framesTobeCoded_TemporalSubsampleRatio e.g. Bin_T2RA_A_S01_R22_qp22_s0_f65_t1.bin | example: ACityScapesS2IRIS_3840x2176_24fps_10bit_420.yuv |
| Training details | Adam solver Batch size is 64 Training patch size is 144×144. Maximum 2000 epochs. The learning rate is 0.0001 L1 loss is used. | Adam solver Mini-batch size is 64. Training patch size is 128×128. 50 epochs. The learning rate starts from 0.0001 and decays by a factor of 0.1 at 20th epoch, 30th epoch, and 40th epoch. L1 loss is used during the first 40 epochs and then replaced by MSE loss. | Adam solver Batch size is 64 Training patch size is 144×144. Maximum 2000 epochs. The learning rate is 0.0001 L1 loss is used. | Adam solver Mini-batch size is 64. Training patch size is 128×128. 50 epochs. The learning rate starts from 0.0001 and decays by a factor of 0.1 at 20th epoch, 30th epoch, and 40th epoch. L1 loss is used during the first 40 epochs and then replaced by MSE loss. |

Table 2: Harmonized training methodology for EE1-1.5 and EE1-1.6 (only use common aspects)

| Aspect | Tencent Intra | ByteDance Intra | Tencent Inter | ByteDance Inter |
| --- | --- | --- | --- | --- |
| Data set | DIV-2K | DIV-2K | BVI (class B) | BVI (class B) |
| RGB YUV | ffmpeg default BT 601 | ffmpeg default BT 601 | ffmpeg default BT 601 | ffmpeg default BT 601 |
| VTM Version | NNVC anchor | NNVC anchor | NNVC anchor and NNVC anchor +JVET- X0054 | NNVC anchor + intra models (and inter for 3 phase) |
| QPs | 22 27 32 37 42 | 22 27 32 37 42 | 22 27 32 37 42 | 22 27 32 37 42 |
| #frames | 1 | 1 | 65 (repeat last frame) or 64 | 65 (repeat last frame) or 64 |
| cfg | encoder_intra_vtm.cfg | encoder_intra_vtm.cfg | encoder_randomaccess_vtm.cfg | encoder_randomaccess_vtm.cfg |
| Naming Convention | to be defined | to be defined | to be defined | to be defined |
| Training details | tbd | tbd | tbd | tbd |

Table 3: Harmonized training methodology for EE1-1.5 and EE1-1.6  (merged aspects)

| Aspect | Tencent Intra | ByteDance Intra | Tencent Inter | ByteDance Inter |
| --- | --- | --- | --- | --- |
| Data set | DIV-2K first 800 training (plus 800 via ds by factor 2) last 100 validation | DIV-2K first 800 training (plus 800 via ds by factor 2) last 100 validation | BVI (class B), TVD 10 bits | BVI (class B), TVD 10 bits |
| RGB YUV | ffmpeg default BT 601 | ffmpeg default BT 601 | ffmpeg default BT 601 | ffmpeg default BT 601 |
| VTM Version | NNVC anchor | NNVC anchor | NNVC anchor and NNVC anchor +JVET- X0054 | NNVC anchor + intra models (and inter for 3 phase) |
| QPs | 17, 22, 27, 32, 37, 42 | 17, 22, 27, 32, 37, 42 | 17, 22, 27, 32, 37, 42 | 17, 22, 27, 32, 37, 42 |
| #frames | 1 | 1 | TVD 65 frames BVI-DVC 64 frames | TVD 65 frames BVI-DVC 64 frames |
| cfg | encoder_intra_vtm.cfg | encoder_intra_vtm.cfg | encoder_randomaccess_vtm.cfg | encoder_randomaccess_vtm.cfg |
| Naming Convention | to be defined | to be defined | to be defined | to be defined |
| Training details | tbd | tbd | tbd | tbd |

# Suggestions for faster crosschecks

It is expected that a successful crosscheck will produce a model which is close in performance to the one of the proponent. If we can assume that this implies the performance of the generated model for a given epoch during the training process should also be similar for different training runs we can speed-up the crosscheck procedure as follows:

- The proponent runs his training. Proponent should keep checkpoints for each epoch and the log files for training and validation error

- The crosschecker requests checkpoints for certain epochs, decided (possibly randomly) by the crosschecker
Question/Note: First few epochs might have to be excluded. After how many epochs can similar model performance be expected?

- Proponent provides the requested checkpoints

- The crosschecker continues/resumes training from the provided checkpoints for some small number of epochs. Note that if resources are available, training resuming from those checkpoints can be done in parallel. It is expected that training and validation loss should be close to the one the proponent observed.

- Performance of final model should still be checked in VTM/ECM.