| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0194-v1 |
| --- | --- |

| Title: | On VTM results for HDR content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This contribution reports results for VTM 17.0 vs VTM 16.0 for both the HDR CTC (JVET-Z2011) and the high bit depth CTC (JVET-U2018). It is reported that these results show differences between BD-rate changes for the PSNR and wPSNR metrics. Further, it is reported that the results show that there are also different results in terms of these metrics on a per sequence basis, with some sequences performing much better than others. It is suggested that if HDR is considered an important future use case of VVC that it should be strongly encouraged that any proposal is backed with results for both HDR and SDR CTCs.

# Introduction

Since VVC v1 and more recently v2 were standardized, a number of non-normative encoder optimizations have been adopted. These optimizations have typically consisted of configuration updates and have typically reported results for the 10 bit SDR CTC [1] or its predecessors and not for either the 10 bit HDR CTC [2] or its predecessors or the high bit depth CTC [3]. Unlike the SDR CTC which only reports coding efficiency changes in terms of PSNR, HDR content also reports wPSNR, DE100 and PSNR-L100 for PQ content. One purpose of this contribution is to identify occasions when a coding efficiency improvement in PSNR is not matched with one for the other metrics.

This contribution reports results for VTM 17 against an anchor of VTM 16 for the HDR CTC and standard QP range of the high bit depth CTC.

# VTM 17.0 vs VTM 16.0

The two proposals that were adopted at the JVET Z Meeting which affect the configuration of VTM were as follows:

JVET-Z0072 “AHG10/AHG12: Enhanced reference picture structures for ECM and VTM”

JVET-Z0099 “AHG10: Deblocking in RDO and beta offset minus 2 for VTM”

The impact of these configuration changes is explored in the next sections.

## HDR CTC

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM16.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 101% |

Figure 1. VTM 17.0 vs VTM 16.0 for JVET-Z2011 (AI)

No contributions had any effect on the All Intra configuration and therefore as expected the results are unchanged.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM16.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 2.47% | -0.53% | -0.41% | 3.70% | 0.48% | -0.40% | -3.37% | -6.23% | 110% | 102% |
| Class H2 |  |  |  |  |  | -0.96% | -1.89% | -2.19% | 109% | 102% |
| Overall | 2.47% | -0.53% | -0.41% | 3.70% | 0.48% | -0.60% | -2.83% | -4.76% | 109% | 102% |

Figure 2. VTM 17.0 vs VTM 16.0 for JVET-Z2011 (RA)

The results for VTM 17.0 show gains for PSNR across all summary figures, but these are not matched when using the wPSNR metric for the chroma channels. On average the difference between PSNR and wPSNR figures is almost 7%.

Looking at more detail into the results for PQ content the following results were obtained.

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Market | 4.60% | -0.57% | -0.54% | 10.74% | 2.98% | -0.55% | -2.60% | -6.66% |
| SunRise | 0.30% | -0.25% | 0.06% | 1.18% | 0.24% | 0.22% | -6.99% | -9.22% |
| ShowGirl2 | -0.85% | -0.66% | -0.46% | 1.69% | -6.22% | -0.40% | -5.95% | -14.75% |
| BalloonFestival | 0.52% | -0.57% | -0.29% | 0.94% | 0.72% | -0.26% | -5.45% | -5.29% |
| Hurdles | 9.85% | -0.52% | -0.63% | 16.33% | 14.35% | -0.73% | 10.55% | 9.93% |
| Starting | 7.08% | -0.70% | -0.64% | 7.29% | 7.14% | -0.67% | 0.17% | -0.18% |
| Cosmos1 | -4.21% | -0.43% | -0.40% | -12.25% | -15.84% | -0.38% | -13.32% | -17.40% |

Figure 2.Per sequence  VTM 17.0 vs VTM 16.0 for JVET-Z2011 (RA)

These results indicate a combination of both gains and losses for both PSNR and wPSNR, but in all cases the PSNR result is better than the wPSNR result. The DE100 and PSNR-L100 results match the relative importance of chroma within each metric.

Equally noticeable is the fact the relative performance of VTM17.0 and VTM 16.0 for HDR content is unstable, with significant gains and losses reported for both metrics.

## High bit depth CTC

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM16.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 99% |

Figure 4. VTM 17.0 vs VTM 16.0 for JVET-U2018 (Standard QP AI)

Again, as expected, no changes are present for AI.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM16.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 4.51% | -0.60% | -0.52% | 8.24% | 5.63% | -0.57% | 0.20% | -1.15% | 108% | 101% |
| Class H2 |  |  |  |  |  | -0.78% | -1.12% | -1.96% | 108% | 101% |
| Overall | 4.51% | -0.60% | -0.52% | 8.24% | 5.63% | -0.67% | -0.46% | -1.56% | 108% | 101% |

Figure 5. VTM 17.0 vs VTM 16.0 for JVET-U2018 (Standard QP RA)

For Random Access using the standard QP range (22-37) of the high bit depth CTC the results for 12 bit PQ shows a similar pattern to that observed in the 10 bit HDR CTC.

Looking in more depth at each sequence, the following results are reported.

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BallonFestival | 0.13% | -0.71% | -0.35% | 0.58% | 0.20% | -0.34% | -5.69% | -5.82% |
| EBU_04_Hurdles | 10.21% | -0.40% | -0.55% | 16.26% | 13.92% | -0.66% | 10.46% | 9.48% |
| Market3Clip4000 | 3.47% | -0.60% | -0.58% | 9.48% | 2.01% | -0.59% | -3.54% | -7.37% |
| EBU_06_Starting | 4.23% | -0.70% | -0.61% | 6.63% | 6.37% | -0.68% | -0.42% | -0.90% |

Figure 6. Per sequence VTM 17.0 vs VTM 16.0 for JVET-U2018 (Standard QP RA)

The instability of the PSNR results can again be identified in the PSNR results with the gains for Market and BalloonFestival contrasting with the loss for Hurdles. For wPSNR, chroma losses are reported for all sequences.

# Conclusions

Results from recent CTC-related adoptions have been presented and show that there is variability between HDR and SDR CTCs. It is proposed that further encouragement should be given to verify HDR CTCs prior to adopting changes that have been applied solely to SDR CTCs.

# References

[1] F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring, “VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video”, JVET-Y2010

[2] A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “VTM and HM common test conditions and evaluation procedures for HDR/WCG video”, JVET-Z2011

[3] A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018

# Patent rights declaration(s)

Sony Corporation or Sony Group companies do not have any current or pending patent rights relating to the technology described in this contribution.