| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0201-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AA0061 (EE2-3.1: IBC Merge Mode with Block Vector Differences) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Damian Ruiz Coll, Vikas Warudkar | Tel: Email: | duizcoll@ofinno.com, vwarudkar@ofinno.com, |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for JVET-AA0061 as they are described in EE2-3.1, which proposes the use of the IBC merge mode with block vector differences (IBC-MBVD) as an extension of MMVD. Specifically, it proposes to refine the selected IBC merge block vector candidate using block vector differences (BVD). An IBC-MBVD flag and an index to refined candidates are signaled to specify whether IBC-MBVD mode is used for the IBC merge mode and the distance of the BVD.

# Introduction

In the JVET-AA0061 proposal [1], the base candidates are selected from the first five candidates in the reordered IBC merge list, and the BVD directions are evaluated just in two horizontal and two vertical directions, using a set of 20 distances as follows:

{1-pel, 2-pel, 4-pel, 8-pel, 12-pel, 16-pel, 24-pel, 32-pel, 40-pel, 48-pel, 56-pel, 64-pel, 72-pel, 80-pel, 88-pel, 96-pel, 104-pel, 112-pel, 120-pel, 128-pel}

Based on the SAD cost between the template (one row above and one column left to the current block) and its reference for each refinement position, all the possible MBVD refinement positions (20×4) for each base candidate are reordered. Finally, the top 8 refinement positions with the lowest template SAD costs are kept as available positions, and an index to the eight refined positions is signaled.

Cross-check of the source code and simulation results has been conducted. Inspected source code follows the description of the EE2-3.1 test. The simulation is performed on top of ECM-5.0. The cross-check results are provided in the tables below and match the results provided by the proponent. Note that coding time may not be accurate.

Table I. Coding performance of the EE2-3.1 test compared to ECM-5.0 (PLT on).

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.39% | -0.37% | -0.39% | 96% | 104% |
| Class TGM | -1.03% | -0.93% | -0.98% | 94% | 101% |
| Overall | -0.71% | -0.65% | -0.68% | 95% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.35% | -0.08% | -0.22% | 104% | 100% |
| Class TGM | -0.99% | -1.05% | -1.14% | 103% | 100% |
| Overall | -0.67% | -0.56% | -0.68% | 104% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.35% | -1.75% | -1.32% | 100% | 99% |
| Class TGM | -0.93% | -0.94% | -1.02% | 89% | 90% |
| Overall | -0.64% | -1.34% | -1.17% | 94% | 94% |

# Conclusion

Results reported in the EE2-3.1 test have been confirmed.

# Reference

[1] Na Zhang, Jizheng Xu, Kai Zhang, Mehdi Salehifar, Li Zhang, “EE2-3.1: IBC Merge Mode with Block Vector Differences”, JVET-AA0061, July 2022