| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0045-v2 |
| --- | --- |

| Title: | AHG10: study of VVC spatial scalability performance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | P. de Lagrange G. Marquant C. Salmon-Legagneur F. Urban | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the results VVC tests in spatial scalability mode, following work reported previously in JVET-X0202, JVET-Y0047 and JVET-Y0048.

Multiple profiles have been specified in VVC, including multilayer profiles for 4:2:0 and 4:4:4 content that  give access to spatial scalability using a single decoder instance. The VVC 4:2:0 multilayer profile was tested in a spatially scalable configuration with two layers and a spatial resolution ratio of 2x and 3x both vertically and horizontally. The tests were performed using various bitrate allocations between the base layer and the enhancement layer, and the best tradeoff leading to optimal enhancement layer compression performance was investigated. Both objective and subjective evaluations have been carried out and show that with VVC, spatial scalability using dual-layer coding can be on par or outperform single-layer coding in specific conditions. It is noted that visual comparison is more often in favor of dual-layer coding than PSNR.

# Introduction

The impact of bitrate allocation between layer in spatial scalability has been studied previously in JVET-Y0048. In this contribution, an updated VTM version has been used, and some other sequences were tested. Contrary to JVET-Y0048, a fixed base layer QP was used, and enhancement layer QP was varied to evaluate the impact of layer bitrate allocation. Then, a formal visual test was performed using the optimal QP pairs. Finally, preliminary tests with 3x scaling ratio (1280x720 + UHD) were performed.

# Test settings

The configuration used is based on the CTCs, with two layers of different resolutions, and testing different QP offsets between layers.

## Layer settings

Spatial scalability was tested with the following horizontal/vertical spatial ratios:

- 2x, with HD resolution as base layer (BL) and UHD resolution as enhancement layer (EL).

- 3x, with 1280x720 resolution as base layer (BL) and UHD resolution as enhancement layer (EL).

Downscales were generated from UHD source using the “TAppDownConvertStatic” software provided in the SHM [3]. The downscale filters are the same as those used in the VVC RPR coding tool, and it is possible to control the chroma location (different for BT.709 or BT.2020 color space).

For example, the following command line was used to downscale at BT.709-color content (chroma location 0) from UHD to HD:

TAppDownConvertStatic 3840 2160 <input.yuv> 1920 1080 <output.yuv> 10 10

## Anchor

The performance of UHD layer from dual-layer coding was measured against UHD single layer; the performance of base layer upscale to UHD was also monitored to check the actual contribution of the enhancement layer.

## Test content

Six SDR UHD sequences were tested. All are 10-bit, 4:2:0. They are listed in section 4.

## Encoding settings

VVC encodings were performed by the VTM in version 16.0 (17.0 for 3x tests), in random access CTC configuration.

The EL/BL bitrate allocation was varied to study its impact on performance, by keeping BL QP constant, and varying EL QP.

# Visual tests settings

Visual tests were performed in InterDigital labs using the degradation category rating (DCR) test method [1], where the reference and the test videos are shown successively, with an 11-grade impairment scale [2], shown in Figure 1 below:

Figure 1 - 11-grade impairment scale used for visual tests

The test involved a panel of 17 expert viewers, in a controlled environment. The setup consisted in an LG OLED55C1 UHD screen in a dark room with controlled lighting. The viewers were placed at 1.5 time the height of the screen. At most two viewers simultaneously performed the test.

# Sequences and rate points

Table 1 – Test sequences and QP settings

| Test sequence & format | HD Base layer QPs | UHD Enhancement layer QPs | Thumbnail |
| --- | --- | --- | --- |
| ElFuente-12000 (Procession) 3840×2160, 60fps, 600 frames | 26 29 32 36 40 | 31-45 34-48 36-50 42-48 44-52 |  |
| Marathon2 3840×2160, 30fps, 300 frames | 26 29 32 36 40 | 31-45 34-48 36-50 42-48 44-54 |  |
| FoodMarket4 3840×2160, 60fps, 600 frames | 23 26 28 30 34 38 42 | 27-35 30-38 32-38 34-40 38-44 42-50 46-54 |  |
| Fortnite 3840×2160, 60fps, 600 frames | 27 30 32 34 38 42 | 31-45 34-48 36-50 38-50 42-52 46-54 |  |
| CorsaireJoy 3840×2160, 25fps, 250 frames | 26 29 32 36 | 29-45 33-48 36-50 36-48 |  |
| Campfire 3840×2160, 30fps, 300 frames | 19 21 24 28 34 38 | 23-27 25-29 28-32 32-40 36-42 42-46 |  |

# Impact of bitrate balance between layers on performance

Figure 2 illustrates dual-layer coding performance depending on the bitrate balance between layers by varying quantization parameters (QP): for multiple QPBL we vary QPEL. Quality is measured using PSNR of Y component. The graphs depict PSNRY curves: single layer (displayed as a dashed blue curve), and dual layer configurations for a set of (QPBL, QPEL) points. Additionally, upscaled base layer is displayed as a dashed green curve.

….. Upscaled HD base layer ….. single layer x dual layer: BR_EL/BRtot > 15% x dual layer: 2%<BR_EL/BRtot<15% x dual layer: BR_EL/BRtot < 2%

Figure 2 : rate-distortion (RD) curves for video sequence ‘CorsaireJoy’ for spatial scalability HD > UHD. On top: curve with 4 fixed QPBL and varying QPEL. Bottom: RD curve indicating bitrate allocation between BL and EL out of total bitrate

Exploring various quantization parameters on each layer is equivalent to study the bitrate allocation between base layer (BL) and enhancement layer (EL). For each QPBL, sweep is expected to start on the upscaled base layer curve (with high enough QPEL), then quality increases with the EL bitrate and can outperform the single layer quality. Finally, the R/D slope diminishes when QPEL decreases. Figure 2-bottom displays the same RD results but adding information about the ratio of the enhancement layer bitrate to the global bitrate. It can be noticed that when the QPEL is very low, i.e. EL bitrate is large, then the enhancement layer does not improve any more the rate-distortion performance and the curve drops below the single layer (pink crosses in Figure 2-right) because of base layer overhead.

When allocating between 2% and 15% of the global bitrate to the enhancement layer and the remaining to the base layer, it can be noticed that, for low to medium bitrates, dual-layer coding can outperform both single-layer and the upscaled base layer. At high bitrates, quality of upscaled base layer saturates, while UHD details coded by EL or single layer start to make a significant difference. Actually, inter-layer prediction is less and less used when EL bitrate increases, ultimately making the BL a useless overhead. According to Figure 2, best results for dual layer are obtained using 5-15% of the global bitrate for the enhancement layer (generally when QPEL = QPBL + 6 to 10). Figure 3 compares two dictinct dual layer configurations: one with the enhancement layer taking about 5-15% of the total bitrate and the other one with the enhancement layer taking about 50% of the total bitrate. Distortion is measured using PSNR and MS-SSIM.

Figure 3: RD curves with 2 quality metrics (left: PSNR-Y, right: MS-SSIM-Y)

# Test results

## Rate points / QP selection

QP pairs giving the best coding performance were used to perform a visual assessment. The QP values that were found to span the range of MOS values were then used to compute BD-rate differences using various objective metrics.

## Objective metrics

Table 2 reports the BD-rate variations for VTM dual layer using VTM single layer as reference for various metrics

Table 2: BD-rate for VTM dual layer (test) vs VTM single layer (reference)

| Sequence | QPBL/EL | QPSL | BD-rate (PSNRY) | BD-rate (MS-SSIMY) | BD-rate (VMAF) |
| --- | --- | --- | --- | --- | --- |
| Campfire | 24/32, 28/36, 34/42, 38/46 | 29, 35, 40, 44 | -2.7 % | -8.7 % | -5.8 % |
| CorsaireJoy | 26/33, 29/36, 32/40, 36/44 | 31, 35, 38, 42 | -2.9 % | -8.7 % | -6.5 % |
| Procession | 29/36, 32/40, 36/44, 40/47 | 35, 38, 42, 46 | -3.9 % | -6.6 % | -6.2 % |
| FoodMarket4 | 30/38, 34/44, 38/48, 42/50 | 36, 40, 44, 48 | -4.9 % | -9.5 % | -7.7 % |
| Fortnite | 30/38, 34/42, 38/46, 42/51 | 36, 40, 44, 48 | +4.2 % | -7.2 % | -2.0 % |
| Marathon2 | 26/35, 32/44, 36/46, 40/50 | 32, 38, 43, 46 | +0.0% | -5.1% | -3.6% |

## Visual tests (2x spatial scalability)

The measured MOS values of the reconstructed video on the 11-grade scale are plotted over the bit rate of the corresponding bitstream. The ±95% confidence intervals for the MOS values are indicated with a color shade enclosing each curve.

Figure 4: Rate-MOS plot for Campfire 	Figure 5: Rate-MOS plot for CorsaireJoy

Figure 6: Rate-MOS plot for Procession	Figure 7: Rate-MOS plot for FoodMarket4

Figure 8: Rate-MOS plot for Fortnite	Figure 9: Rate-MOS plot for Marathon2

Even though confidence intervals overlap between anchor and test most of the time, the general trend is that dual-layer is preferred to single-layer, and mostly in the full range of bitrates (including higher bitrates where the PSNR performance is lower). We found that MS-SSIM-Y metric better approximates visual observation than PSNR-Y or even VMAF.

## Preliminary tests for 3x spatial scalability

Preliminary tests with 3x scaling ratio (1280x720 + UHD) indicate that coding performance is lower than with a 2x scaling ratio (HD + UHD), as shown on Figure 10. However, on the sequence tested, enhancement layer still provides a good contribution to quality even with relatively low additional rate. It is also noted that the pure 3x	 upscale outperforms single layer at much lower bitrate than the 2x upscale, which is not surprising.

Figure 10: rate-distortion curves for video sequence ‘CorsaireJoy’
for spatial scalability {720p + UHD} (black) and {HD + UHD} (red)

# References

- Recommendation ITU-T P.910 (2008), Subjective video quality assessment methods for multimedia applications.

- Recommendation ITU-R BT.500-14 (2019), Methodologies for the subjective assessment of the quality of television images.

- JCTVC SHM reference software, available at https://vcgit.hhi.fraunhofer.de/jvet/SHM