| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0051-v2 |
| --- | --- |

| Title: | AHG9: On the SEI processing order SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang (Bytedance) 8910 University Center Lane, San Diego, CA 92122, USA Hendry (LGE) 10680 Treena St San Diego, CA, 92131, USA | Email: | yekui.wang@bytedance.com dr.hendry@lge.com |
| Source: | Bytedance Inc., LG Electronics Inc. |  |  |

_____________________________

# Abstract

The following changes for the SEI processing order SEI message are proposed:

- It is required that an SEI processing order SEI message shall contain at least two pairs of po_sei_payload_type[ i ] and po_sei_payload_type[ i ].

- The value 0 for po_sei_processing_order[ i ] is treated the same as other values of the syntax element.

- For any two different integer values of m and n that are greater than or equal to 0, po_sei_processing_order[ m ] greater than 0 and equal to po_sei_processing_order[ n ] indicates that there is no preferred order of processing between the SEI messages with payloadTypes equal to po_sei_payload_type[ m ] and po_sei_payload_type[ n ].

# Introduction

The following two minor issues have been observed with the current text for the SEI processing order SEI message:

- The syntax allows for signalling of only one entry of po_sei_payload_type[ i ] and po_sei_payload_type[ i ] in the loop. However, it would only make sense when two or more entries are signalled, because only then a relative processing order among multiple types of SEI messages can be signalled.

- In the semantics of the SEI processing order SEI message, the value 0 for po_sei_processing_order[ i ] is specified with a special treatment. po_sei_processing_order[ i ] equal to 0 specifies that the preferred order of processing SEI messages with payloadType equal to po_sei_payload_type[ i ] is unknown or unspecified or determined by external means. However, it seems that this special treatment is neither necessary nor useful but would rather introduce unclarity and even operation problems. For example, if the preferred processing order is unknown or unspecified, then it would be cleaner to just not list that payload type value in the SEI message. If the preferred processing order is determined by external means, then what would be the interaction of that externally-determined order with those signalled in this SEI message itself?

- In the semantics of the SEI processing order SEI message, it is specified that po_sei_processing_order[ m ] greater than 0 and equal to po_sei_processing_order[ n ] indicates that the preferred order of processing of SEI messages with payloadTypes equal to po_sei_payload_type[ m ] and po_sei_payload_type[ n ] is unknown or unspecified or determined by external means not specified in this Specification. However, per our understanding, the intent was that in case the indicated order values are the same for two SEI messages, it indicates that there is no preferred order of processing between the two SEI messages.

# Proposal

The following are proposed for resolving the issues:

- It is required that an SEI processing order SEI message shall contain at least two pairs of po_sei_payload_type[ i ] and po_sei_payload_type[ i ].

- The value 0 for po_sei_processing_order[ i ] is treated the same as other values of the syntax element.

The detailed proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts):

SEI processing order SEI message semantics

The SEI processing order SEI message carries information indicating the preferred processing order, as determined by the encoder (i.e., the content producer), for different types of SEI messages that may be present in the bitstream. When an SEI processing order SEI message is present in any access unit of a CVS, an SEI processing order SEI message shall be present in the first access unit of the CVS. The SEI processing order SEI message persists in decoding order from the current access unit until the end of the CVS. When there are multiple SEI processing order SEI messages present in a CVS, they shall have the same content.

It is a requirement of bitstream conformance that, within an SEI processing order SEI message, there shall be at least two pairs of the syntax elements po_sei_payload_type[ i ] and po_sei_payload_type[ i ], i.e., the syntax elements po_sei_payload_type[ 0 ], po_sei_payload_type[ 0 ], po_sei_payload_type[ 1 ], and po_sei_payload_type[ 1 ] shall be present.

po_sei_payload_type[ i ] specifies the value of payloadType for the i-th SEI message for which information is provided in the SEI processing order SEI message. The values of po_sei_payload_type[ m ] and po_sei_payload_type[ n ] shall not be identical when m is not equal to n.

po_sei_processing_order[ i ] indicates the preferred order of processing any SEI message with payloadType equal to po_sei_payload_type[ i ]. For any two different integer values of m and n that are greater than or equal to 0, po_sei_processing_order[ m ] greater than 0 and less than po_sei_processing_order[ n ] indicates any SEI message with payloadType equal to po_sei_payload_type[ m ], when present, should be processed before any SEI message with payloadType equal to po_sei_payload_type[ n ], when present. , and po_sei_processing_order[ m ] greater than 0 and equal to po_sei_processing_order[ n ] indicates that the there is no preferred order of processing between of the SEI messages with payloadTypes equal to po_sei_payload_type[ m ] and po_sei_payload_type[ n ] is unknown or unspecified or determined by external means not specified in this Specification. po_sei_processing_order[ i ] equal to 0 specifies that the preferred order of processing SEI messages with payloadType equal to po_sei_payload_type[ i ] is unknown or unspecified or determined by external means not specified in this Specification.

# Patent rights declaration(s)

Bytedance Inc. and LG Electronics Inc.may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).