| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0054-v1 |
| --- | --- |

| Title: | EE1-1.3: CNN Based In-Loop Filter with WCDANN |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hao Zhang Cheolkon Jung Dan Zou Ming Li | Email: | 13227706628@163.com zhengzk@xidian.edu.cn zoudan@oppo.com myron.li@oppo.com |
| Source: | [Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd.] |  |  |

_____________________________

- Abstract

In JVET-AA0074, two NN-based in-loop filters were presented based on the QP value, namely Filter 1 and Filter 2. In this contribution, three groups of loss function weights are chosen to verify the performance of Filter 1. The three groups of weights are {6,1,1}, {8,1,1}, and {10,1,1}. Compared with VTM-11.0-NNVC-2.0, the performance of each group of weights in AI configuration is as follows:

{6, 1, 1}: {4.36%, 12.59%, 11.95%} BD-rate reductions for {Y, Cb, Cr} channel,

{8, 1, 1}: {4.78%, 10.95%, 10.24%} BD-rate reductions for {Y, Cb, Cr} channel,

{10, 1, 1}: {4.38%, 11.11%, 9.20%} BD-rate reductions for {Y, Cb, Cr} channel.

Introduction

In JVET-AA0074 [1], some improvements were made to Y0052 filter. The input part of the network was adjusted to be suitable for processing luminance and chrominance components together. At the same time, some additional auxiliary information was introduced in the input to further improve the network performance. In addition, AA0074 use down sampling and channel splitting operations to lighten the network. In this contribution, an ablation study on the weight of loss function was conducted to verify the impact of different weights on performance.

Experimental description

Test-1.3.1

In JVET-AA0074, two filters were proposed: Filter 1 can be used to process videos with multiple QPs by using QP map as input, and Filter 2 contains five models to process videos under different QPs respectively. Test 1.3.1 aims to verify the performance of these two filters.

Test-1.3.2

In JVET-AA0074, a weighted L1 and L2 loss was used to train the proposed filter. Test 1.3.2 aims to verify the impact of different loss weights on network performance. Specifically, three groups of weights were used, namely {6, 1, 1}, {8,1,1} and {10,1,1}.

Training

Table 1. Network information of the proposed in-loop filters in the training stage.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: GeForce RTX 3090 |
|  | Framework: | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 200 |
|  | Batch size: | 16 |
|  | Training time: | ~100h/model |
|  | Training data information: | BVI-DVC, DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0, QP {22, 27, 32, 37, 42} |
|  | Loss function: | Weighted L1 and L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | random flipped |
|  | Other information: |  |

Inference

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Libtorch v1.9.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.3M |
|  | Total Number of Parameters (All Models) | Filter 1: 1.3M total Filter 2: 6.5M total |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | Filter 1: 5.4M total Filter 2: 27.0M total |
|  | Multiply Accumulate (MAC)/pixel | 434K |
| Optional |  |  |
|  | Total Conv. Layers | 48 Depthwise separable convolution 24 Common convolution |
|  | Total FC Layers | 40 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Experimental results

Each sequence was test based on VTM-11.0_NNVC-2.0 according to the common test conditions (CTCs) defined in [4]. And the evaluation was preformed in the All Intra (AI) configuration.

Results of Test-1.3.1

Experimental results of Filter 1 and Filter 2 are shown in Tables 3 and 4. In the AI configuration, the proposed Filter 1 achieves {4.36%, 12.59%, 11.95%} BD-rate reduction on average for {Y, Cb, Cr} channel, and the proposed Filter 2 achieves {4.58%, 12.67%, 13.21%} BD-rate reduction on average for {Y, Cb, Cr} channel.

Table 3 Performance of the proposed Filter 1 in AI configuration

Table 4 Performance of the proposed Filter 2 in AI configuration

Results of Test-1.3.2

Experimental results of Filter 1 with three groups of weights are shown in Tables 5, 6 and 7. Filter 1 with {6,1,1}, {8,1,1} and {10,1,1} achieves {4.36%, 12.59%, 11.95%}, {4.78%, 10.95%, 10.24%} and {4.38%, 11.11%, 9.20%} BD-rate reduction on average for {Y, Cb, Cr} channel, respectively.

Table 5 Performance of the proposed Filter 1 with {6, 1, 1} in AI configuration

Table 6 Performance of the proposed Filter 1 with {8, 1, 1} in AI configuration

Table 7 Performance of the proposed Filter 1 with {10, 1, 1} in AI configuration

Conclusions

This contribution mainly includes two experiments. Experiment 1 aims to verify the performance of AA0074 filter, and Experiment 2 conducts ablation study for loss function based on Filter 1. Experimental results show that the proposed Filter 1 can achieve better performance trade-off between luma and chroma when the weight of loss function is {8,1,1}.

References

[1]	H. Zhang, C. Jung, D. Zou and M. Li, “[EE1-related] Lighter WCDANN: CNN Based In-Loop Filters,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 Document JVET-AA0074, July 2022.

[2]	Di Ma, Fan Zhang, and David Bull, “BVI-DVC: A Training Database for Deep Video Compression”, arXiv:2003.13552, 2020.

[3]	https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

[4]	S. Liu, A. Segall, E. Alshina, and R.-L. Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology,” JVET-W2016, April 2021.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).