| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0068 -v1 |
| --- | --- |

| Title: | EE1-1.6: RDO Considering Deep In-Loop Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junru Li, Yue Li, Kai Zhang, Li Zhang | Tel: Email: | lijunru@bytedance.com yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

- Abstract

This proposal reports the experimental results of EE1-1.6. The proposed method is asserted to improve the compression efficiency at encoder by introducing CNN-based filtering into the rate-distortion optimization (RDO) process. In the RDO process, CNN-based filtering is implemented with SADL using fixed point-based calculation. Compared with VTM11.0_NNVC2, the proposed method reportedly shows on average {7.56%, 20.30%, 20.70%}, { 9.78%, 20.66%, 20.33%}, and {x%, x%, x%} BD-rate reductions for {Y, Cb, Cr} components, under AI, RA, and LDB configurations, respectively.

Introduction

A deep learning-based in-loop filtering method proposed in JVET-AA0111 [1] is adopted in NNVC common SW base (NCS). However, the filtering procedure is not considered in the RDO process. In JVET-Z0155 [2], CNN-based filtering is integrated into the RDO process for a better estimation of RD cost when deep learning-based in-loop filtering is used. In JVET-AA0113 [3], a standalone C++ library named SADL [4] is utilized to implement the CNN-based filtering in the RDO process. This contribution presents the experimental results of EE1-1.6.

Proposed method

It is proposed that CNN-based filtering is involved during the partitioning mode selection. In particular, the samples obtained after CNN-based filtering are compared with original samples to calculate the distortion. The optimal partitioning mode is then selected based on the refined rate-distortion (RD) cost.

To reduce the complexity of applying CNN-based filtering in RDO, several fast algorithms are proposed. First, a simplified version of CNN model as shown in Fig. 1 is additionally trained and used in the RDO stage where the simplified model is implemented with SADL using fixed point-based calculation. Second, only one filter is included in the RDO process without considering filter selection. Finally, the proposed technique is only applied to the coding units with height and width no larger than 64.

Fig. 1. (a) Architecture of the CNN filtering method. M denotes the number of feature maps. N stands for the number of samples in one dimension. (b) Construction of Attention Residual Block in (a).

The inference and training processes of models are same as those in JVET-AA0111 [1].

Inference

SADL is used for to perform the inference of the proposed CNN filters in RDO process. The network information in the inference stage is provided in Table 1 as suggested by [5].

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1.56 M/model (4 models) |
|  | Parameter Precision (Bits) | 16 for fixed point version |
|  | Memory Parameter (MB) | 3.1 MB/model (4 models) |
|  | MAC (Giga) | 539 K/pixel |
| Optional |  |  |
|  | Total Conv. Layers | 25+16 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

PyTorch is used as the training platform. The DIV2K [6] and BVI-DVC [7] datasets are adopted to train the CNN filters of I slices and B slices, respectively. The network information in the training stage is provided in Table 2 as suggested by [5].

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 2 |
|  | Epoch: | 50 |
|  | Batch size: | 64 |
|  | Training time: | 60h/model |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

The proposed method is tested according to the common test conditions defined in [5]. The test results on top of VTM-11.0_NNVC2 [8] are shown in Table 3 ~ Table 5. Under AI configurations, the proposed method can bring 7.56%, 20.30%, and 20.70% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under RA configurations, the proposed method can bring 9.78%, 20.66%, and 20.33% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under LDB configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 3. Performance of the proposed method on top of VTM-11.0_NNVC2 (RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.09% | -16.07% | -18.57% | 218% | 52412% |
| Class A2 | -10.53% | -20.14% | -17.25% | 214% | 50257% |
| Class B | -9.38% | -22.94% | -21.73% | 219% | 51262% |
| Class C | -10.25% | -21.64% | -22.21% | 198% | 46987% |
| Class E |  |  |  |  |  |
| Overall | -9.78% | -20.66% | -20.33% | 212% | 50109% |
| Class D | -11.91% | -23.84% | -23.94% | 196% | 47204% |
| Class F | -4.89% | -11.89% | -10.81% | 296% | 24610% |

Table 4. Performance of the proposed method on top of VTM-11.0_NNVC2 (LDB)

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -9.43% | -15.14% | -14.51% | 186% | 45173% |
| Class E | -8.81% | -17.09% | -16.04% | 362% | 43287% |
| Overall |  |  |  |  |  |
| Class D | -11.05% | -19.34% | -18.41% | 181% | 39007% |
| Class F |  |  |  |  |  |

Table 5. Performance of the proposed method on top of VTM-11.0_NNVC2 (AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.63% | -17.99% | -19.80% | 248% | 41223% |
| Class A2 | -6.79% | -20.58% | -17.96% | 204% | 33585% |
| Class B | -6.80% | -22.12% | -21.81% | 201% | 31355% |
| Class C | -7.80% | -19.01% | -21.91% | 184% | 21295% |
| Class E | -10.22% | -21.04% | -20.86% | 206% | 35557% |
| Overall | -7.56% | -20.30% | -20.70% | 205% | 31106% |
| Class D | -7.71% | -19.55% | -21.59% | 184% | 20236% |
| Class F | -4.27% | -11.83% | -11.12% | 147% | 21387% |

Compared with the JVET-AA0111 [1], experimental results are shown in Table 6 ~ Table 8. Under AI configurations, the proposed method can bring 0.32%, 0.20%, and 0.16% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under RA configurations, the proposed method can bring 0.38%, -0.09%, and -0.04% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under LDB configurations, the proposed method can bring x%, x%, and x% BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 6. Performance of the proposed method on top of AA0111 with fixed point-based model (RA)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.17% | -0.06% | 106% | 100% |
| Class A2 | -0.42% | 0.13% | 0.04% | 109% | 100% |
| Class B | -0.40% | 0.17% | 0.05% | 109% | 100% |
| Class C | -0.47% | 0.17% | 0.11% | 109% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.38% | 0.09% | 0.04% | 108% | 100% |
| Class D | -0.53% | -0.48% | -0.07% | 111% | 100% |
| Class F | -0.22% | -0.19% | -0.17% | 105% | 99% |

Table 7. Performance of the proposed method on top of AA0111 with fixed point-based model (LDB)

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.32% | 0.07% | 0.41% | 112% | 100% |
| Class E | -0.20% | 0.25% | 1.39% | 107% | 100% |
| Overall |  |  |  |  |  |
| Class D | -0.30% | -0.76% | -0.48% | 112% | 100% |
| Class F |  |  |  |  |  |

Table 8. Performance of the proposed method on top of AA0111 with fixed point-based model (AI)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | 0.00% | 0.12% | 123% | 99% |
| Class A2 | -0.31% | -0.18% | -0.16% | 129% | 100% |
| Class B | -0.29% | -0.33% | -0.29% | 133% | 100% |
| Class C | -0.44% | -0.19% | -0.16% | 131% | 98% |
| Class E | -0.39% | -0.23% | -0.21% | 127% | 99% |
| Overall | -0.32% | -0.20% | -0.16% | 129% | 99% |
| Class D | -0.41% | -0.07% | -0.20% | 135% | 99% |
| Class F | -0.22% | -0.07% | -0.21% | 114% | 100% |

Conclusions

This contribution presents an encoder optimization method based on the CNN-based filtering method. The proposed optimization method shows promising coding gains. It is recommended to include the proposed encoder optimization into the common code base of EE1.

References

- Y. Li, K. Zhang, J. Li, L. Zhang, H. Wang, K. Reuze, A.M. Kotra, M. Karczewicz, Franck Galpin, K. Andersson, J. Ström, D. Liu, R. Sjöberg, EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation, JVET-AA0111, July 2022.

- J. Li, Y. Li, K. Zhang, L. Zhang, EE1-1.7-related: Improved RDO Considering Deep In-Loop Filter and Deblocking, JVET-Z0155, April 2022.

- J. Li, Y. Li, K. Zhang, L. Zhang, EE1-1.6-related: RDO Considering Deep In-Loop Filter with SADL, JVET-AA0113, July 2022.

- F. Galpin, T. Dumas, P. Bordes, E. François, AHG11: SADL update, JVET-Z0161, by teleconference, April 2022.

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AA2016, July 2022.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- Di Ma, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

- https://vcgit.hhi.fraunhofer.de/jvet-aa-ee1/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc-2.0

- J. Li, Y. Li, K. Zhang, L. Zhang. "Joint Rate Distortion Optimization with CNN-Based In-loop Filter for Hybrid Video Coding." IEEE Data Compression Conference (DCC), 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).