| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0072-v1 |
| --- | --- |

| Title: | VTM Encoder Implementation for Green-MPEG SEI Messaging |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christian Herglotz Matthias Kränzler André Kaup | Email: | christian.herglotz@fau.de matthias.kraenzler@fau.de andre.kaup@fau.de |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander University Erlangen-Nürnberg (FAU) |  |  |

_____________________________

# Abstract

This document proposes a reference encoder implementation for generation and signaling of Green MPEG complexity metrics for decoder power reduction and quality metrics for quality recovery after low power encoding, which are both defined for VVC SEI messages. The implementation complies with the issued FDIS ISO/IEC 23001-11 3rd edition Energy-Efficient Media Consumption (Green Metadata). The implementation is available for the VTM-17.2 encoder implementation. We propose to include the code for a future version of VTM.

# Introduction

In the July 2022 meeting of ISO/IEC JTC 1/SC 29/WG3, the FDIS of ISO/IEC 23001-11 3rd edition Energy-Efficient Media Consumption (Green Metadata) [2] was issued. In this document, SEI metadata for VVC for energy-efficient media consumption is specified. In particular, Chapters 6.2 and 9 of ISO/IEC 23001-11 3rd edition define syntax used in VVC SEI messages targeting complexity metrics for decoder-power reduction and metrics for quality recovery after low-power encoding, respectively. In the April 2022 meeting of JVET, a decoder implementation for parsing Green-Metadata SEI messages was adopted into the VTM software [3]. In this document, we present a complementary software patch for the VTM encoder that implements the generation of Green-Metadata SEI messages, i.e. the analysis, the derivation, and the signaling mechanisms.

# Green-Metadata Encoder Implementation

For the generation of Green-Metadata in VVC SEI messages, functionality for generating complexity metrics, quality metrics, and for writing the corresponding syntax in the bit stream is implemented on top of VTM-17.2 and available at https://gitlab.lms.tf.fau.de/LMS/vtm-analyzer.

Concerning complexity metrics (CMs) for decoder power reduction, the determination of the CMs is implemented using the bit stream feature analyzer (BSFA) presented in [4]. The BSFA counts the occurrence of certain bit stream features during encoder runtime. Examples for features are non-zero transform coefficients, transform blocks of a certain size, or the number of block borders filtered by the de-blocking filter. To determine CMs, the feature numbers corresponding to the CMs defined in [2] are exploited to determine the values of the CMs. The code for writing the syntax to the bit stream and the BSFA implementation for the encoder are wrapped by the preprocessor JVET_Z0046. The current implementation is restricted to write picture-wise CM signaling (period_type = 0x0; granularity_type = 0x0).

For verification of CM signaling, the following mechanisms are included in the encoder implementation: 
- It is tested whether each CM is located in the allowed range (0,…,1). 
- It is tested whether the sum of all prediction blocks covers all pixels of the frame.

Concerning quality metrics (QMs) for quality recovery after low-power encoding, the code for determining picture-wise PSNR, WPSNR, and SSIM values is already included in the VTM software. We exploit the precomputed PSNR value and implemented the code for signaling the PSNR as SEI metadata using the preprocessor JVET_Z0046. This implementation allows to write picture-wise PSNRs, which means that a segment is considered to be a single frame.

A full example of an encoder configuration is provided in the file cfg/sei_vui/greenMetadataSEI.cfg. Note that functionality in this implementation is only available for the cases described above.

# Conclusion

This document presented a reference encoder implementation of the Green MPEG complexity metrics for decoder power reduction and the quality recovery for low power encoding defined for VVC SEI messages. The implementation is fully compliant with the FDIS of Green MPEG [2] and allows writing SEI messages on picture granularity. We propose to adopt the implementation for a future version of the VTM software.

# References

[1] 	C. Herglotz, M. Kränzler, A. Kaup (FAU), E. Francois, M. Radosavljevic, E. Reinhard (InterDigital), X. Ducloux (Harmonic), D. Menard (INSA), Y. He, M. Coban, D. Rusanovskyy, M. Karczewicz (Qualcomm), “AHG9: Green Metadata SEI message for VVC”, JVET-W0071, July 2021.

[2] WG 03 MPEG Systems, “Text of ISO/IEC FDIS 23001-11 3rd edition Energy-Efficient Media Consumption (Green Metadata)”, output document MDS21785_WG03_N00656 , July 2022.

[3] M. Kränzler, C. Herglotz, A. Kaup: "Bit Stream Feature Analyzer (BSFA) for Coding Tool Statistics based on VTM-10.0", document JVET-T0067, Teleconference, October 2020.

[4] C. Herglotz, M. Kränzler, A. Kaup, „VTM Software Implementation for GREEN-MPEG SEI Messaging“, document JVET-Z0046, Teleconference, April 2022.

# Patent rights declaration(s)

The Friedrich-Alexander University Erlangen-Nürnberg does not have any current or pending patent rights relating to the technology described in this contribution.