| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0077-v1 |
| --- | --- |

| Title: | EE1-2.2: CNN Filter for Super-Resolution with RPR functionality in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shimin Huang Cheolkon Jung Yang Liu Ming Li | Email: | shimin_huang2022@163.com zhengzk@xidian.edu.cn serena@oppo.com myron.li@oppo.com |
| Source: | [Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd.] |  |  |

_____________________________

- Abstract

The EE1-2.2 test reports BD-rate gains of JVET-AA0065 (AA0065 filter) over VTM-11.0_NNVC-2.0 anchor. Compared with VTM-11.0_NNVC-2.0, AA0065 filter achieves average {-3.89% (Y), 5.52% (U), -9.07% (V)} and {-8.5% (Y), 17.03% (U), -8.28% (V)} BD-rate gains over the anchor in AI and RA configurations, respectively. Moreover, this test also reports BD-rate gains of AA0065 filter over JVET-Z0065 anchor. JVET-Z0065 provided a method that the encoder adaptively selects a scale factor from ×1.0, ×1.5 and ×2.0 at the GOP level. Compared with JVET-Z0065 anchor, AA0065 filter achieves average {-4.36% (Y), -1.73% (U), -21.25% (V)} BD-rate gains under AI configuration.

Introduction

JVET-AA0065 [1] presented the convolutional neural network (CNN)-based super-resolution (SR) filter with existing RPR functionality in VVC while achieving coding gains. JVET-Z0065 [2] proved that the RPR function selection at the GOP level provides a better coding performance. The EE1-2.2 test reports BD-rate gains over VTM-11.0_NNVC-2.0 and JVET-Z0065.

Proposed method

MMSDANet for Luma Channel

MMSDANet consists of 8 MMSDABs, three convolutional layers, one concatenate layer, one shuffle layer and five shortcut connections. Except for the 1x1 and last convolutional layers, all convolutional layers and MMSDABs have 64 input and output channels. 1x1 convolution is used to reduce the number of channels and reduce the complexity of the operation. The output channel of the last convolutional layer is 4 so that the pixel shuffle layer can do 2x upsampling. Since the input and output resolutions are different, the input needs to be upsampled when applying global connections.

Fig. 1 Illustration of the proposed MMSDANet for luma channel.

MMSDANet for Chroma Channels

For the chroma channels, the backbone and pixel shuffle layer of MMSDANet are consistent with the luma channel, and their main difference lies in the input and head of the network. Figs. 2 and 3 illustrate the network architecture of MMSDANet for U and V channels, respectively. To utilize the cross-component correlation, the reconstructed luma component is taken as one input. For the network applied to the chroma channels, its input contains three channels of Y, U, and V. The size of the guided component Y is twice of the U/V channel, thus the Y channel needs to be first downsampled. We choose to use a 3x3 convolution with stride 2 for downsampling.

Fig. 2 Illustration of the proposed MMSDANet for U channel.

Fig. 3 Illustration of the proposed MMSDANet for V channel.

MMSDAB

Fig. 4 shows the network structure of MMSDAB. MMSDAB consists of seven convolutional layers, one concatenate layer, one channel shuffle layer and one SE attention layer. The convolutional layer is used to extract the scale information and depth information of the image features, while the SE attention [4] layer is used to enhance important information.

Fig. 4 Illustration of the proposed MMSDAB.

Train and inference

The network information in the inference stage is provided in Table 1.

Table 1 Network information for the proposed model in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel Core i9 12900k @3.9GHz |
|  | Framework: | LibTorch v1.9 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | luma up-sampling model: 2.0M/model chroma up-sampling model: 2.1M/model |
|  | Total Parameter Number | 31M |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 15 models in total: 118 MB |
|  | Multiply Accumulate (MAC) | 2044kMAC/pixel |
| Optional | Total Conv. Layers | 75 for up-sampling the luma, 77 for up-sampling the chroma |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 |
|  | Patch size | Whole frame |

DIV2K [3] and BVI_DVC [4] datasets are used for training. The official NNVC-2.0 anchor (VTM-11.0 [5] + new MCTF [6]) is utilized to compress these images with QPs {22, 27, 32, 37, 42}. In the context of RPR scheme, the training data is down-sampled by a factor of 2 before encoding. Then, we use the decoded low-resolution images and the corresponding original images for training. We train a separate model for each QP for the three channels Y, U, V, thus there are 15 models in total. The models are fixed. Compared to VTM 11.0, the bitstream is reduced a lot because the video frame is downsampled to half of the original size.

Table 2 Network information for the proposed model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA 3090 24GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | luma:50, chroma:30 |
|  | Batch size: | 64 |
|  | Training time: | 50h/model |
|  | Training data information: | DIV2K, BVI_DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

Experiments are performed at five QPs (22,27,32,37,42) according to the common test condition (CTC) [7]. Two anchors of VTM-11.0_NNVC-2.0 and JVET-Z00065 [2] are used for comparison.

Tables 1 and 2 show BD-rates of AA0065 filter over VTM-11.0_NNVC-2.0 anchor in AI and RA configurations, respectively.

- Compared with VTM-11.0_NNVC-2.0 anchor, AA0065 filter achieves average BD-rate gains of {-3.89% (Y), 5.52% (U), -9.07% (V)} and {-8.5% (Y), 17.03% (U), -8.28% (V)} in RA and AI configurations, respectively.

Table 3 shows BD-rates of AA0065 filter over JVET-Z0065 anchor in AI configuration.

- Compared with JVET-Z0065 anchor, AA0065 filter achieves average BD-rate gains of {-4.36% (Y), -1.73% (U), -21.25% (V)} in AI configuration.

Table 1 BD-rates of AA0065 filter over VTM-11.0_NNVC-2.0 anchor in AI configuration.

|  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27, 32, 37, 42) |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | bit DIFF |
| Cass A1 4K | Tango2 | -9.45% | -17.31% | -15.16% | #DIV/0! | #DIV/0! | 47% |
|  | FoodMarket4 | -4.48% | -6.65% | -6.73% | #DIV/0! | #DIV/0! | 45% |
|  | Campfire | -12.88% | 149.67% | -23.80% | #DIV/0! | #DIV/0! | 68% |
| Class A2 4K | CatRobot1 | -7.72% | -11.63% | -9.82% | #DIV/0! | #DIV/0! | 52% |
|  | DaylightRoad2 | -2.67% | -22.70% | -19.24% | #DIV/0! | #DIV/0! | 55% |
|  | ParkRunning3 | -13.80% | 21.27% | -0.90% | #DIV/0! | #DIV/0! | 63% |
| Average on A1 |  | -8.94% | 41.90% | -15.23% | #DIV/0! | #DIV/0! | 53% |
| Average on A2 |  | -8.06% | -4.35% | -9.99% | #DIV/0! | #DIV/0! | 57% |
| Overall |  | -8.50% | 18.78% | -12.61% | #DIV/0! | #DIV/0! | 55% |

Table 2 BD-rates of AA0065 filter over VTM-11.0_NNVC-2.0 anchor in RA configuration.

|  |  | Random Access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27, 32, 37, 42) |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | bit DIFF |
| Cass A1 4K | Tango2 | -7.04% | -25.37% | -15.49% | #DIV/0! | #DIV/0! | 55% |
|  | FoodMarket4 | -6.17% | -9.78% | -11.50% | #DIV/0! | #DIV/0! | 54% |
|  | Campfire | -17.36% | 38.87% | -23.00% | #DIV/0! | #DIV/0! | 65% |
| Class A2 4K | CatRobot1 | 3.93% | -10.72% | -2.94% | #DIV/0! | #DIV/0! | 55% |
|  | DaylightRoad2 | 13.44% | -18.60% | -15.16% | #DIV/0! | #DIV/0! | 58% |
|  | ParkRunning3 | -10.11% | 58.69% | 13.66% | #DIV/0! | #DIV/0! | 68% |
| Average on A1 |  | -10.19% | 1.24% | -16.66% | #DIV/0! | #DIV/0! | 58% |
| Average on A2 |  | 2.42% | 9.79% | -1.48% | #DIV/0! | #DIV/0! | 60% |
| Overall |  | -3.89% | 5.52% | -9.07% | #DIV/0! | #DIV/0! | 59% |

Table 3 BD-rates of AA0065 filter over JVET-Z0065 anchor in AI configuration.

|  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27, 32, 37, 42) |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | bit DIFF |
| Cass A1 4K | Tango2 | -3.03% | -19.28% | -20.04% | #DIV/0! | #DIV/0! | 52% |
|  | FoodMarket4 | -2.40% | -12.64% | -12.45% | #DIV/0! | #DIV/0! | 48% |
|  | Campfire | -10.97% | 98.22% | -23.86% | #DIV/0! | #DIV/0! | 66% |
| Class A2 4K | CatRobot1 | -4.10% | -20.37% | -18.56% | #DIV/0! | #DIV/0! | 53% |
|  | DaylightRoad2 | -1.10% | -23.42% | -19.81% | #DIV/0! | #DIV/0! | 56% |
|  | ParkRunning3 | -4.51% | -32.91% | -32.77% | #DIV/0! | #DIV/0! | 60% |
| Average on A1 |  | -5.47% | 22.10% | -18.79% | #DIV/0! | #DIV/0! | 55% |
| Average on A2 |  | -3.24% | -25.56% | -23.71% | #DIV/0! | #DIV/0! | 56% |
| Overall |  | -4.36% | -1.73% | -21.25% | #DIV/0! | #DIV/0! | 55.5% |

Conclusions

The EE1-2.2 test provides BD-rate gains of AA0065 filter over two anchors of VTM-11.0_NNVC-2.0 and JVET-Z0065. Compared with VTM-11.0_NNVC-2.0 anchor, AA0065 filter achieves significant BD-rate reductions in Y and V channels, but it causes BD-rate increase in U channel. Compared with JVET-Z0065 anchor, AA0065 filter achieves BD-rate reductions in all channels. In case of Y channel in AI configuration, BD-rate reduction of AA0065 filter over VTM-11.0_NNVC-2.0 anchor is nearly two times of that over JVET-Z0065 anchor.

Reference

- S. Huang, C. Jung, Y. Liu, and M. Li, AHG11: CNN Filter for Super-Resolution with RPR functionality in VVC, JVET-AA0065.

- J. Nam, S. Yoo, J. Lim, S. Kim, EE1-2.1: RPR encoder with multiple scale factors, JVET-Z0065.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/, R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- D. Ma, F. Zhang, and D. R. Bull, BVI-DVC: A Training Database for Deep Video Compression (https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn).

- VTM-11.0 https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-11.0

- P. Wennersten, C. Hollmann, and J. Ström, GOP-based temporal filter improvements, JVET-V0056.

- S. Liu, A. Segall, E. Alshina, and R. -L. Liao, JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-X2016.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).