| Joint Video Exerts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0080-v1 |
| --- | --- |

| Title: | AHG10: GOP-based RPR encoder control |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Ström, R. Yu, P. Wennersten, W. Ahmad | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

The VVC reference software VTM currently lacks support for selecting when to use Reference Picture Resampling (RPR). This contribution proposes a GOP-based selection method that decides when it is likely better to encode pictures in reduced resolution with RPR and when it is likely better to encode them in the source resolution. The selection is based on QP and picture self-similarity after re-scaling. The self-similarity test is only conducted on the first source picture in display order within each GOP. If the re-scaled picture is determined to have sufficient similarity with the source picture, all pictures in the GOP are encoded at reduced resolution, otherwise they are encoded in the source resolution.

The BD-rate performance (Y/Cb/Cr) compared to CTC with VTM-18.0 as anchor is as follows:

RA (luma/Cb/Cr):   -0,44%/1,30%/1,26%

LDB (luma/Cb/Cr): -0,03%/0,23%/0,17%

AI (luma/Cb/Cr): -0,30%/0,62%	/0,46%

The method is especially beneficial for higher resolutions and higher QPs.

It is asserted that the method, when it is selected, often can improve subjective quality compared to encoding in source resolution.

It is suggested to include the method in VTM.

# Introduction

When encoding at restricted bitrates, it can sometimes be useful to encode at a lower resolution rather than at the source resolution. Since VVC provides a technique for enabling flexible switching of resolution without requiring encoding of an intra picture at the switching point, it is possible to do this more efficiently than in previous standards. However, the VVC reference encoder lacks a method to decide when to enable encoding at reduced resolution. This contribution proposes to add such functionality to VTM. The contribution is based on previous contributions on the topic, JVET-W0082 [1] and JVET-Y0068 [2]. Compared to those, the selection to encode at a lower resolution is less aggressive and also includes an additional reduced resolution to better close the gap between full and 2/3 resolution.

# Proposal

The first picture in display order in each GOP is downscaled according to the selected scaling ratio. The downscaled picture is then upscaled to the source resolution and compared with the source picture by means of PSNR, and this PSNR score is then compared with a configurable threshold parameter. If the re-scaled picture is sufficiently similar to the source picture, all pictures in the GOP are encoded at reduced resolution. Otherwise, all pictures are encoded at the source resolution. The configurable threshold parameter is individual for respective reduced resolution (1/2, 2/3 and 4/5) such that it requires greater similarity with the source to invoke a larger reduction in resolution. The QP is reduced by -6 for 1/2, -4 for 2/3, and -2 for 4/5 to achieve similar bitrates as the anchor.

# Results

The method is implemented on top of VTM-18.0 [3], and objective results are provided for VTM CTC [4] by evaluation in the source resolution based on the VTM upscaling approach (--UpscaledOutput=2). Encoding and decoding time are not reliable. As can be seen, the approach can provide a significant benefit in BDR, especially for 4K. The benefit is also consistent with MSSSIM. Some loss in chroma BDR is observed but visual inspection has not shown any chroma issues. Viewing has shown improved subjective quality, especially for higher QPs compared to encoding in source resolution.

## CTC

|  | Random access Main10 (QP22,27,32,37) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-18.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR |  |  | U-PSNR |  |  | V-PSNR |  |  | Y-MSIM |  |  | U-MSIM |  |  | V-MSIM |  |  | EncT |  |  | DecT |
| Class A1 | -1,86% |  |  | -0,57% |  |  | 0,51% |  |  | -3,62% |  |  | -2,22% |  |  | -1,36% |  |  | 82% |  |  | 88% |
| Class A2 | -0,56% |  |  | 5,83% |  |  | 5,53% |  |  | -2,69% |  |  | 3,63% |  |  | 2,48% |  |  | 87% |  |  | 97% |
| Class B | 0,13% |  |  | 0,75% |  |  | 0,16% |  |  | -0,26% |  |  | 0,31% |  |  | -0,22% |  |  | 93% |  |  | 92% |
| Class C | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 97% |  |  | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0,44% |  |  | 1,30% |  |  | 1,26% |  |  | -1,35% |  |  | 0,38% |  |  | 0,15% |  |  | 90% |  |  | 93% |
| Class D | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 95% |  |  | 94% |
|  | Low delay B Main10 (QP22,27,32,37) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-rate Over VTM-18.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  |  | V-PSNR |  |  | Y-MSIM |  |  | U-MSIM |  |  | V-MSIM |  |  | EncT |  |  | DecT |  |  |
| Class A1 | #VALUE! | #VALUE! |  |  | #VALUE! |  |  | #VALUE! |  |  | #VALUE! |  |  | #VALUE! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| Class A2 | #VALUE! | #VALUE! |  |  | #VALUE! |  |  | #VALUE! |  |  | #VALUE! |  |  | #VALUE! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| Class B | -0,07% | 0,56% |  |  | 0,41% |  |  | -0,28% |  |  | 0,78% |  |  | 0,58% |  |  | 97% |  |  | 97% |  |  |
| Class C | 0,00% | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 90% |  |  | 94% |  |  |
| Class E | 0,00% | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 94% |  |  | 100% |  |  |
| Overall | -0,03% | 0,23% |  |  | 0,17% |  |  | -0,12% |  |  | 0,33% |  |  | 0,24% |  |  | 94% |  |  | 97% |  |  |
| Class D | 0,00% | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 89% |  |  | 96% |  |  |
|  | All Intra Main10 (QP22,27,32,37) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-rate Over VTM-18.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR |  | U-PSNR |  |  | V-PSNR |  |  | Y-MSIM |  |  | U-MSIM |  |  | V-MSIM |  |  | EncT |  |  | DecT |  |
| Class A1 | -0,58% |  | 0,14% |  |  | 0,04% |  |  | -0,74% |  |  | 0,51% |  |  | 0,56% |  |  | 94% |  |  | 92% |  |
| Class A2 | -1,23% |  | 3,14% |  |  | 2,21% |  |  | -2,17% |  |  | 1,87% |  |  | 0,89% |  |  | 92% |  |  | 95% |  |
| Class B | 0,01% |  | 0,26% |  |  | 0,30% |  |  | -0,18% |  |  | 0,35% |  |  | 0,53% |  |  | 92% |  |  | 97% |  |
| Class C | 0,00% |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 89% |  |  | 91% |  |
| Class E | 0,00% |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 97% |  |  | 100% |  |
| Overall | -0,30% |  | 0,62% |  |  | 0,46% |  |  | -0,53% |  |  | 0,50% |  |  | 0,39% |  |  | 93% |  |  | 95% |  |
| Class D | 0,00% |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 0,00% |  |  | 99% |  |  | 96% |  |

## QP (22, 27, 32, 37, 42)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-18.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -4,13% | 2,00% | 1,94% | -7,05% | 0,48% | -0,02% | 81% | 84% |
| Class A2 | -1,51% | 9,42% | 8,21% | -4,78% | 8,04% | 6,44% | 84% | 94% |
| Class B | -0,41% | 0,94% | 0,24% | -3,44% | -1,26% | -2,16% | 93% | 92% |
| Class C | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 97% | 96% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1,26% | 2,60% | 2,11% | -3,51% | 1,28% | 0,56% | 89% | 92% |
| Class D | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 95% | 93% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-18.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0,36% | 1,73% | 1,39% | -0,93% | 2,56% | 2,32% | 98% | 97% |
| Class C | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 93% | 96% |
| Class E | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 95% | 101% |
| Overall | -0,15% | 0,72% | 0,58% | -0,39% | 1,07% | 0,97% | #DIV/0! | #DIV/0! |
| Class D | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 92% | 96% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-18.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -1,46% | 1,94% | 1,98% | -2,25% | 3,02% | 3,57% | 100% | 91% |
| Class A2 | -2,19% | 5,22% | 3,77% | -3,70% | 4,95% | 3,87% | 95% | 96% |
| Class B | -0,23% | 1,11% | 1,35% | -0,65% | 1,89% | 2,53% | 95% | 98% |
| Class C | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 90% | 92% |
| Class E | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 96% | 98% |
| Overall | -0,67% | 1,50% | 1,33% | -1,17% | 1,85% | 1,94% | 95% | 95% |
| Class D | 0,00% | 0,01% | 0,01% | 0,00% | 0,03% | 0,02% | 98% | 97% |

# Conclusions

It has been shown that the GOP-based RPR encoder control can provide some additional benefit for VTM and especially for 4K resolution. It is therefore suggested to include the method to the VTM reference software.

# References

[1] K. Andersson, J. Ström, R. Yu, “AHG10: GOP-based RPR encoder control”, JVET-W0082, Meeting by Teleconference, July 2021.

[2] J. Nam, S. Yoo, J. Lim, “EE1-2.1-related: RPR encoder with multiple scale factors”, JVET-Y0068, Meeting by Teleconference, Jan 2022.

[3] VTM-18.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-18.0

[4] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, January 2021

# Patent rights declaration(s)

Ericsson AB may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).