| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0083-v3 |
| --- | --- |

| Title: | EE1-1.8: More refinements on NN based in-loop filter with a single model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liqiang Wang Xiaozhong Xu Shan Liu Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Email: | liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com xiezhihuang@oppo.com yue.yu@oppo.com v-yuhaoping@oppo.com wangdong7@oppo.com |
| Source: | Tencent, OPPO |  |  |

_____________________________

- Abstract

This contribution reports the EE results of JVET-AA0089. The proposed method in JVET-AA0089 is implemented on NCS-1.0 (filter set #0), and then the model is fine-tuned for a better performance. Based on the NNVC-2.0, the test results are shown in order of RA and AI configurations as follows.

Test 1.8.1 (Implemented on NCS-1.0 with filter set #0):

RA: -8.92%		-18.48%	-19.20%	EncT: 150%	DecT: 64963%

AI : -6.52%		-15.52%	-16.61%	EncT: 146%	DecT: 35076%

Test 1.8.2 (Model fine-tunning based on Test 1.8.1):

RA: -9.18%		-19.65%	-19.17%	EncT: 150%	DecT: 65582%

AI : -6.63%		-15.33%	-16.40%	EncT: 146%	DecT: 35084%

Compared with NCS-1.0 (filter set #0) under RA configuration, an additional 0.47% (8.71% to 9.18%) BD-rate luma gain is observed without any increase in both MAC result and memory size.

Introduction

In this contribution, the original solution in JVET-AA0089[] is implemented on NCS-1.0 (filter set #0) []. In order to further improve the performance, the model is fine-tuned by updating the training dataset. In brief, there are two EE tests as follows:

Test 1.8.1: Test the same solution as in JVET-AA0089 on NCS-1.0 (filter set #0).

Test 1.8.2: Optimize the trade-off of JVET-AA0089 by fine-tuning the model based on Test 1.8.1.

Proposed method

The reconstructed image after LMCS is fed into the network. Then, the output of NN filter is processed by ALF and CCALF. Only one model is used to improve the image quality of luma and chroma components. U and V channels are up-sampled and down-sampled in the pre-processing and post-processing, respectively. The network architecture is firstly presented in Section 2.1, then the network information in both training stage and inference stage is provided in Section 2.2 and Section 2.3, respectively. Section 2.4 presents the related implementation details.

Network architecture

The network structure of the proposed NN filter is shown in Fig. 1. Along with the reconstructed image (rec_yuv), additional side information is also fed into the network, such as the prediction image (pred_yuv) [], slice QP, base QP and slice type. As for the structure of Resblock, the number of channels firstly goes up before the activation layer, and then goes down after the activation layer. Specifically, K and M are set to 64 and 160 respectively, and the number of Resblock is set to 32.

Fig. 1: Network architecture

Network information in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla V100 32GB |
|  | Framework: | Pytorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~100 |
|  | Batch size: | 64 |
|  | Loss function: | L1 |
|  | Training time (for 1 model): | ~60h |
|  | Training data information: | DIV2K, TVD, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations | 1200 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Learning rate update strategy |  |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Training data update strategy: |  |
|  | Other information: |  |

Network information in inference stage

In the inference stage, SADL [] is adopted as the inference library. In addition, all the models in this contribution are quantized into int16 precision.

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.9M |
|  | Total Number of Parameters (All Models) | 1.9M |
|  | Parameter Precision (Bits) | 16 |
|  | Memory Parameter (MB) | 3.8MB/model, 1 model in all |
|  | Multiply Accumulate (kMAC/pixel) | 485K (w/o block extension) 615K (w/ block extension) |
|  | Calculation Method | block basis |
| Optional |  |  |
|  | Total Conv. Layers | 101 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Implementation details

- For I slices and B slices, Deblock and SAO are both enabled. The optimal filtered result is decided between the two outputs from the proposed filter and the SAO.

- Multiple results are inferred by the common one model but with the different inputs {BaseQP, BaseQP - 5, BaseQP + 5} into the BaseQP interface of the network, where the index is decided at the slice-level.

- A scaling operation is carried out to refine the result of NN filter. The scaling factors are signaled in the slice header for each component. The specific process is the same as the scaling process used in []. Additionally, there are 3 fixed weights to blend the results of the proposed filter and the conventional filter. These fixed weights are 1, 0.75 and 0.5, which are similar as those in [].

- The proposed filter can be turned on/off at the CTU level and slice level.

Results

All the experiments are performed based on the common test conditions in []. The test results of EE1-1.8.1 and EE1-1.8.2 are provided in Section 3.1 and Section 3.2, respectively. For the convenient comparison, the test results of NCS-1.0 (filter set #0) is provided in Section 3.3. The average results can also be summarized as follows.

EE1-1.8.1:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -8.92% | -18.48% | -19.20% | 150% | 64963% |
| AI | -6.52% | -15.52% | -16.61% | 146% | 35076% |

EE1-1.8.2:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -9.18% | -19.65% | -19.17% | 150% | 65582% |
| AI | -6.63% | -15.33% | -16.40% | 146% | 35084% |

NCS-1.0 (filter set #0):

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -8.71% | -18.24% | -18.87% | 139% | 63434% |
| AI | -6.52% | -15.52% | -16.61% | 145% | 35028% |

Results of EE1-1.8.1

Table 1. Performance of EE1-1.8.1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.26% | -13.15% | -18.90% | 159% | 71034% |
| Class A2 | -9.50% | -19.53% | -14.73% | 154% | 67003% |
| Class B | -8.60% | -19.08% | -20.82% | 155% | 68626% |
| Class C | -8.61% | -20.95% | -20.75% | 135% | 55428% |
| Class E |  |  |  |  |  |
| Overall | -8.92% | -18.48% | -19.20% | 150% | 64963% |
| Class D | -9.74% | -20.01% | -21.14% | 133% | 49017% |
| Class F | -3.79% | -12.19% | -11.09% | 187% | 28129% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.12% | -14.26% | -17.26% | 192% | 47251% |
| Class A2 | -5.73% | -15.81% | -12.98% | 151% | 38415% |
| Class B | -5.93% | -15.54% | -17.46% | 140% | 35071% |
| Class C | -6.40% | -16.42% | -18.16% | 120% | 23613% |
| Class E | -8.83% | -15.25% | -16.12% | 148% | 40304% |
| Overall | -6.52% | -15.52% | -16.61% | 146% | 35076% |
| Class D | -6.41% | -15.30% | -18.56% | 115% | 21658% |
| Class F | -4.02% | -11.81% | -11.71% | 117% | 29491% |

Results of EE1-1.8.2

Table 2. Performance of EE1-1.8.2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.48% | -16.04% | -18.26% | 159% | 71370% |
| Class A2 | -9.84% | -19.26% | -14.99% | 154% | 67099% |
| Class B | -8.72% | -19.88% | -19.33% | 155% | 68824% |
| Class C | -9.04% | -22.36% | -22.80% | 135% | 56964% |
| Class E |  |  |  |  |  |
| Overall | -9.18% | -19.65% | -19.17% | 150% | 65582% |
| Class D | -10.06% | -21.18% | -22.57% | 133% | 50114% |
| Class F | -4.10% | -13.15% | -12.05% | 187% | 28434% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.20% | -14.25% | -16.91% | 193% | 47275% |
| Class A2 | -5.83% | -15.58% | -12.94% | 151% | 38308% |
| Class B | -5.99% | -15.20% | -16.60% | 140% | 34985% |
| Class C | -6.60% | -16.33% | -18.55% | 121% | 23724% |
| Class E | -8.94% | -15.05% | -16.12% | 148% | 40366% |
| Overall | -6.63% | -15.33% | -16.40% | 146% | 35084% |
| Class D | -6.52% | -15.21% | -18.12% | 114% | 21619% |
| Class F | -4.23% | -12.11% | -12.15% | 117% | 29366% |

Results of NCS-1.0 (filter set #0)

Table 3. Performance of NCS-1.0 (filter set #0)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.15% | -13.89% | -18.71% | 147% | 70134% |
| Class A2 | -9.37% | -19.11% | -14.46% | 143% | 66945% |
| Class B | -8.33% | -18.69% | -20.42% | 142% | 66591% |
| Class C | -8.37% | -20.30% | -20.35% | 126% | 53176% |
| Class E |  |  |  |  |  |
| Overall | -8.71% | -18.24% | -18.87% | 139% | 63434% |
| Class D | -9.58% | -19.42% | -20.42% | 123% | 47565% |
| Class F | -3.72% | -11.90% | -10.92% | 169% | 27501% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.12% | -14.26% | -17.26% | 193% | 47421% |
| Class A2 | -5.73% | -15.81% | -12.98% | 150% | 38269% |
| Class B | -5.93% | -15.54% | -17.46% | 139% | 34851% |
| Class C | -6.40% | -16.42% | -18.16% | 121% | 23730% |
| Class E | -8.83% | -15.25% | -16.12% | 147% | 40139% |
| Overall | -6.52% | -15.52% | -16.61% | 145% | 35028% |
| Class D | -6.41% | -15.30% | -18.56% | 114% | 21660% |
| Class F | -4.02% | -11.81% | -11.71% | 116% | 29506% |

Conclusions

In this contribution, the performance of NCS-1.0 (filter set #0) is further improved by the slice-level adaptive QP adjusting method and model fine-tuning. Compared with NCS-1.0 (filter set #0), an additional 0.47% (8.71% to 9.18%) BD-rate luma gain is observed under RA configuration without any increase in both MAC result and memory size.

References

Patent rights declaration(s)

Tencent, Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[] L. Wang, S. Lin, X. Xu, S. Liu, Z. Xie, Y. Yu, H. Yu, D. Wang. EE1-related: more refinements on EE1-1.4 and EE1-1.5. JVET-AA0089, 27th Meeting, by teleconference, 13–22 July 2022.

[] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tags/NCS-1.0.

[] H. Zhu, X. Xu, S. Liu. Deep learning based in-loop filter with QP and residual distribution information. AVS-M5654, Aug. 2020.

[] F. Galpin, T. Dumas, P. Bordes, E. François. AHG11: Small Ad-hoc Deep-Learning Library (SADL) update. JVET-AA0086, 27th Meeting, by teleconference, 13–22 July 2022.

[] H. Wang, J. Chen, K. Reuze, A. M. Kotra, M. Karczewicz. EE1-1.4: Test on Neural Network-based In-Loop Filter with Large Activation Layer. JVET-W0130, 23rd Meeting, by teleconference, 7–16 July 2021.

[] K. Andersson, J. Ström, D. Liu, R. Sjöberg. EE1-related: Combination of VVC deblocking and NN loop filtering. JVET-Y0098, 25th Meeting, by teleconference, 12–21 January 2022.

[] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AA2016, 27th Meeting, by teleconference, 13–22 July 2022.