| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0087-v1 |
| --- | --- |

| Title: | Multilayer coding use cases for broadcasting and streaming applications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | S. Nemoto S. Iwamura A. Ichigaya | Tel: Email: | nemoto.s-fy@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution provides information of a number of use cases of the broadcasting applications using multilayer coding techniques. In broadcasting applications, video distribution with different format (e.g. resolution and/or quality) for various type of devices are demanded since displaying capability and processing power may differ among devices. For such demand, it is suitable to distribute multiple-resolution videos using spatial scalability. Meanwhile, there has been increasing another demand for personalized video experiences in recent years. Picture-in-picture effect for additional information (e.g. sign language, multi-view videos) and open caption are often used in Japanese TV programs, however, it is not possible for the users to turn on/off those “sub-contents” in conventional broadcasting system. Using multilayer coding, main content is encoded as base layer and sub-contents are coded as enhancement layers. By doing so, it is possible to select sub-contents at the TV set side based on the user’s demand.

We believe that multilayer coding technology has a wide range of applicability, therefore developing use cases of multilayer coding would be beneficial not only for broadcasting applications but also streaming applications.

Introduction

In the conventional broadcasting, the same TV program is broadcasted simultaneously for both home TV sets (high-resolution video) and mobile terminals (low-resolution video), with different resolutions. Because the screen sizes and processing capacities of large-screen home TV sets and mobile terminals are different, it is necessary to transmit the videos with a resolution suitable for each device. Accordingly, the requirement of the next-generation broadcasting service also includes different resolution video distribution. In such case, it is well-known that spatial scalability makes more efficient than transmitting them separately. In addition to this case, this contribution provides new use cases using multilayer coding techniques.

Figure 1. Two-layer coding example (4K layer for home use/HD layer for mobile terminal)

Use case-1: Service extensibility using broadband

Due to the limitation of the bandwidth, it is not practical that high-quality contents (e.g. 8K resolution) are transmitted using terrestrial radio wave. One possible solution is to transmit HD/4K layers by terrestrial radio wave and 8K layer is transmitted by broadband. Using this method, it can be expanded without any impact on the existing services (4K/HD layers) when the 8K service deployment is required in the future.

Figure 2. Three-layer coding example (8K layer bitstream is sent using broadband)

Use case-2: On-demand content scalable service

Conventional broadcasting services could not provide different contents for each user. In recent years, there has been an increasing demand for improved accessibility to broadcasting services such as subtitles or sign languages. Furthermore, in sports programs etc., in addition to a main content, there is a need for multi-view videos from different viewpoints that users want to see. Thus, it is desirable to deliver on-demand content service.

Multilayer coding is also useful to realize this use case. Firstly, the main content is encoded in a base layer (main content layer). Secondly, the base layer bitstream is locally decoded and sub-content is superimposed on top of decoded picture of the base layer to make the input image of enhancement layer (sub-contents layer). By doing so, user can turn on/off the sub-contents at the receiver side depending on which layer the user chooses to decode. The concept of this flexible content selection is called “content scalability”. The advantage of this approach is that an additional bitrate for the enhancement layer is very small because the inter-layer prediction is completely matched in the non-overlapped regions. Additionally, in the case of the area of main content is changed, such as L-shaped content, using scaling window is suitable, shown in Fig.4. This use case can be used in combination with the other use case, specifically, it is possible to provide content scalable services together with multi-resolution services.

Figure 3. Content scalable service using multilayer coding

Figure 4. L-shaped sub-content

Conclusion

This contribution provides information of a number of use cases of the broadcasting applications. There is a possibility that new broadcasting services can be realized by using multilayer coding. We believe that multilayer coding has a wide range of applicability, therefore developing use cases of multilayer coding would be beneficial not only for broadcasting applications but also streaming applications.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).