| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0092-v1 |
| --- | --- |

| Title: | EE2-1.8: Gradient linear model with luma value |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li Yan Ye Ru-Ling Liao Jie Chen | Tel: Email: | {sid.lxw, yan.ye, ruling.lrl, jiechen.cj}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the simulation results of EE2 Test 1.8. In Test 1.8, a new GLM mode is proposed where the chroma sample is predicted based on both the gradient and the luma value with different parameters. The model parameters are derived by the adjacent samples based on the LDL decomposition method used in CCCM. In Test 1.8a, the original GLM mode is replaced by the proposed new GLM mode, while in Test 1.8b, the proposed new GLM mode is signaled as an additional mode. It is reported that on top of ECM-6.0, the overall coding performance impact for {Y, U, V, EncT, DecT} are:

- Test 1.8a (Gradient linear model with luma value):

Class A to E:{ 0.00%, -0.35%, -0.35%, 100%, 99%} AI, {-0.02%, -0.12%, -0.19%, 100%, 99%} RA;

Class F:    {-0.27%, -1.52%, -1.55%, 99%, 98%} AI, {-0.47%, -1.74%, -1.51%, 99%, 100%} RA;

Class TGM: {-1.32%, -3.99%, -3.57%, 99	%, 101%} AI, {-0.67%, -2.69%, -2.34%, 100%, 98%} RA;

- Test 1.8b (Gradient linear model with luma value as an additional mode):

Class A to E:{ 0.00%, -0.34%, -0.36%, 100%, 99%} AI, {-0.01%, -0.10%, -0.26%, 100%, 99%} RA;

Class F:    {-0.28%, -1.94%, -1.96%, 100%, 99%} AI, {-0.47%, -1.87%, -1.94%, 100%, 100%} RA;

Class TGM: {-1.34%, -4.86%, -4.13%, 100%, 101%} AI, {-0.69%, -2.96%, -2.49%, 100%, 96%} RA.

Introduction

In ECM-6.0 [1], GLM utilizes the gradient of luma samples to predict a chroma sample as:

predCi,j=α∙Gi,j+β#( LISTNUM  \s 1 )

where predCi,j represents the predicted value of a chroma sample, Gi,j represents the gradient of the corresponding reconstructed luma samples, and the linear model parameters α and β are derived by adjacent reconstructed samples based on the linear minimum mean square error (LMMSE) method as CCLM.

For signaling, when the CCLM_LT, CCLM_L or CCLM_T mode is enabled for the current CU, two flags are signaled separately for Cb and Cr components to indicate whether GLM is enabled to each component. If the GLM is enabled for one component, one syntax element is further signaled to select one of 4 gradient patterns for the gradient calculation as shown in Figure 1.

Figure 1. 4 gradient patterns for GLM

In order to improve the coding efficiency of GLM, a new GLM mode is proposed in the last JVET meeting [2], and studied in EE2 Test 1.8 [3], where both the gradient and the luma value are used to predict the chroma sample. This contribution reports the simulation results of EE2 Test 1.8.

Proposed methods

In Test 1.8, a new GLM mode is proposed that a chroma sample is predicted based on both the gradient Gi,j of luma samples and the reconstructed value recL'i,j of the down-sampled luma sample with different parameters:

predCi,j=α0∙Gi,j+α1∙recL'i,j+α2∙midValue#2

where the model parameters α0, α1 and α2 are derived from six rows and columns adjacent samples based on the LDL decomposition method as the CCCM mode in ECM-6.0.

For signaling, one flag is signaled to indicate whether GLM is enabled to both Cb and Cr components, and the syntax element that indicate the gradient pattern is coded by truncated unary code.

In Test 1.8a, the original GLM mode is replaced by the proposed new GLM mode. In Test 1.8b, the original GLM mode is reserved and the proposed new GLM mode is signaled as an additional mode by signaling one extra flag in the bitstream.

Simulation results

The proposed methods are implemented on top of ECM-6.0 and are tested under CTC for enhanced compression tool testing [4] with all intra and random access configurations. The simulation results of Test 1.8a and Test 1.8b are shown in Table 1 and Table 2, respectively.

Table 1. Test 1.8a: Gradient linear model with luma value

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.51% | -0.29% | 101% | 99% |
| Class A2 | 0.01% | -0.69% | -0.87% | 100% | 100% |
| Class B | 0.00% | -0.29% | -0.26% | 99% | 99% |
| Class C | 0.01% | -0.22% | -0.22% | 100% | 99% |
| Class E | 0.00% | -0.14% | -0.19% | 100% | 101% |
| Overall | 0.00% | -0.35% | -0.35% | 100% | 99% |
| Class D | 0.01% | -0.43% | -0.32% | 99% | 99% |
| Class F | -0.27% | -1.52% | -1.55% | 99% | 98% |
| Class TGM | -1.32% | -3.99% | -3.57% | 99% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.14% | -0.08% | 101% | 99% |
| Class A2 | -0.03% | -0.26% | -0.49% | 100% | 100% |
| Class B | -0.01% | -0.07% | -0.15% | 100% | 99% |
| Class C | 0.02% | -0.07% | -0.10% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.12% | -0.19% | 100% | 99% |
| Class D | 0.03% | -0.31% | -0.30% | 100% | 99% |
| Class F | -0.47% | -1.74% | -1.51% | 100% | 100% |
| Class TGM | -0.67% | -2.69% | -2.34% | 100% | 98% |

Table 2. Test 1.8b: Gradient linear model with luma value as an additional mode

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.51% | -0.30% | 102% | 99% |
| Class A2 | 0.01% | -0.69% | -1.02% | 101% | 99% |
| Class B | -0.01% | -0.30% | -0.27% | 100% | 99% |
| Class C | 0.01% | -0.10% | -0.16% | 101% | 99% |
| Class E | -0.02% | -0.18% | -0.18% | 101% | 100% |
| Overall | 0.00% | -0.34% | -0.36% | 101% | 99% |
| Class D | 0.00% | -0.14% | -0.20% | 100% | 100% |
| Class F | -0.28% | -1.94% | -1.96% | 100% | 99% |
| Class TGM | -1.34% | -4.86% | -4.13% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.12% | -0.21% | 101% | 99% |
| Class A2 | -0.02% | -0.21% | -0.54% | 100% | 100% |
| Class B | 0.00% | 0.02% | -0.22% | 100% | 99% |
| Class C | 0.00% | -0.14% | -0.13% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.10% | -0.26% | 100% | 99% |
| Class D | 0.04% | -0.20% | -0.07% | 100% | 99% |
| Class F | -0.47% | -1.87% | -1.94% | 100% | 100% |
| Class TGM | -0.69% | -2.96% | -2.49% | 100% | 99% |

Conclusion

This contribution reports the simulation results of EE2 Test 1.8. Experimental results show that the proposed new GLM mode can bring bitrate savings with negligible runtime change, especially for screen content sequences. It is recommended to adopt the proposed method to ECM.

References

- M. Coban and et. al., “Algorithm description of Enhanced Compression Model 6 (ECM 6),” JVET-AA2025, Oct. 2022.

- X. Li and et. al., “EE2-related: On Gradient Linear Model (GLM),” JVET-AA0138, July 2022.

- V. Seregin and et al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AA2024, July 2022.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Feb. 2022.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).