| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0094-v1 |
| --- | --- |

| Title: | Non-EE2: Direct block vector (DBV) mode for chroma prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Xue Hao, Yanzhuo Ma, Fuzheng Yang, Jing Ren, Ming Li | Email: | jyhuo@mail.xidian.edu.cn xhao@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn renjing1@oppo.com myron.li@oppo.com |
| Source: | Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

In this contribution, a new chroma intra prediction mode, direct block vector (DBV) mode, is proposed for screen content. This mode is used for chroma components when dual tree is activated in intra slice. After sampling format scaling, the BV at the co-located luma position is used to apply block prediction for chroma. It is reported that the coding performance of ECM 6.0 is as below:

For AI configuration:

F:-0.27%, -0.38%, and -0.31%, with xxx % EncT, 99% DecT.

TGM:-1.81%, -1.93%, and -2.48%, with xxx % EncT, 99% DecT.

For RA configuration:

F: -0.24%, -0.10%, and -0.26 %, with xxx % EncT, xxx% DecT.

TGM: -0.58%, -0.57%, and -0.74%, with xxx% EncT, xxx% DecT.

Introduction

In ECM-6.0, the intra prediction modes for chroma components include 6 cross component linear model (LM) modes, convolution cross component model (CCCM) mode, gradient linear model (GLM) mode, DIMD mode, direct mode (DM) , and four default intra prediction modes.

In the signaling of chroma intra mode, an intra_chroma_pred_mode is signaled to indicate the specific coding mode, as shown in Table 1.

Table 1 The binarization process for intra_chroma_pred_mode in ECM6.0

| intra_chroma_pred_mode | bin string | chroma intra mode |
| --- | --- | --- |
| 0 | 1100 | list[0] |
| 1 | 1101 | list[1] |
| 2 | 1110 | list[2] |
| 3 | 1111 | list[3] |
| 4 | 10 | DIMD chroma |
| 5 | 0 | DM |

ECM6.0 includes a method of chroma prediction using block vector in MODE_IBC. In single tree partition, the prediction process of IBC is applied for both luma and chroma components, and the chroma block vector is derived from the corresponding luma block vector depending on the chroma format sampling structure. In dual tree partition, the prediction process of IBC is only applied for luma component.

Proposed Methods

This contribution proposes a method to improve the coding efficiency of chroma intra prediction for screen content, namely direct block vector (DBV) mode.

For chroma components, when chroma dual tree is activated in intra slice, if one of the luma blocks (the following five locations in Figure 1) is coded with MODE_IBC, its block vector bvL is used and scaled to derive chroma block vector bvC. The scaling factor depends on the chroma format sampling structure.

Figure 1 the five locations in reconstructed luma samples

Then, by using the position of the current chroma block (xCb, yCb) and its bvC, the corresponding offset position (xCb+ bvC[0] , yCb + bvC[1]) is determined, and a block copying prediction is performed.

Figure 2 the prediction process of DBV mode

A CU level flag is signaled to indicate whether the proposed DBV mode is applied as shown in Table 2.

Table 2 The binarization process for intra_chroma_pred_mode in the proposed method

| intra_chroma_pred_mode | bin string | chroma intra mode |
| --- | --- | --- |
| 0 | 11100 | list[0] |
| 1 | 11101 | list[1] |
| 2 | 11110 | list[2] |
| 3 | 11111 | list[3] |
| 4 | 110 | DIMD chroma |
| 5 | 10 | DM |
| 6 | 0 | DBV |

Experimental results

The proposed method was implemented based on ECM-6.0 under common test conditions [2]. Table 3 summarizes the results with all intra and random access configurations.

Table 3. Simulation results of the proposed method

|  |  |  | All Intra Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.27% | -0.38% | -0.31% | #NUM! | 99% |
| Class TGM | -1.81% | -1.93% | -2.48% | #NUM! | 99% |

|  |  |  | Random Access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.24% | -0.10% | -0.26% | #NUM! | #NUM! |
| Class TGM | -0.58% | -0.57 % | -0.74 % | #NUM! | #NUM! |

Conclusions

A new DBV mode is proposed to improve the coding efficiency of chroma intra prediction for screen content. Compared with ECM-6.0, the proposed method shows average BD-rates of -0.27% and -1.81% for class F and TGM under AI case and average BD-rates of -0.24% and -0.58% for class F and TGM under RA case. It is suggested to investigate in EE.

- 5 References

- ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).