| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0095-v1 |
| --- | --- |

| Title: | Non-EE2: Block Vector Difference Sign Prediction (BVDSP) for IBC blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Xue Hao, Yanzhuo Ma, Fuzheng Yang, Jing Ren, Ming Li | Email: | jyhuo@mail.xidian.edu.cn xhao@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn renjing1@oppo.com myron.li@oppo.com |
| Source: | Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

In ECM6.0, signs of block vector difference (BVD) in IBC blocks are transmitted with equal probability (EP) and signs of motion vector difference(MVD) in INTER blocks are coded by MVD sign prediction. In this contribution, the block vector difference sign prediction (BVDSP) is proposed to improve coding efficiency. It is reported that the coding performance to ECM 6.0 is as below:

For AI configuration:

F:-0.21%, -0.25%, and -0.04%, with xxx % EncT, xxx % DecT.

TGM:-0.43%, -0.40%, and -0.43%, with xxx % EncT, xxx % DecT.

For RA configuration:

F: -0.06 %, -0.04%, and -0.11 %, with xxx % EncT, xxx% DecT.

TGM: -0.27 %, -0.36 %, and -0.28 %, with xxx% EncT, xxx% DecT.

Introduction

In ECM6.0[1], the BVD signs are directly coded in EP mode. The MVD sign prediction indices representing the correct MVD signs are encoded with the context model. At decoder side, the MVD signs are derived as follows [2]:

- Parse the absolute values of MVD components

- Parse context-coded MVD sign prediction index

- Build MV candidates by creating combinations between possible signs and the absolute MVD value and add them to the MV predictor

- Derive MVD sign prediction cost for each derived MV based on template matching cost and sort

- Use MVD sign prediction index to pick the true MVD sign

- Add the true MVD to the MV predictor for final MV

The transition from EP coding to context-based coding provides gains.

Proposed Methods

Block vector difference sign prediction (BVDSP) can be applied for IBC blocks when the block vector difference contains non-zero component. Possible BVD sign combinations are sorted according to template matching cost and index corresponding to the true BVD sign is derived and coded with context model. At decoder side, the BVD signs are derived as following major steps:

- Step1: 4 candidate BVs are derived by BVP and combinations between possible signs and absolute BVD

( BVP[0] + absBVD[0], BVP[1] + absBVD[1] )

( BVP[0] + absBVD[0], BVP[1] - absBVD[1] )

( BVP[0] - absBVD[0], BVP[1] + absBVD[1] )

( BVP[0] - absBVD[0], BVP[1] - absBVD[1] )

- Step2: Derive prediction costs of 4 candidate BVs based on template matching and sort them         according to the costs

- Step3: Use BVDSP index to pick the true BVD sign

- Step4: Add the true BVD to the BVP to get the final BV

Bilinear filter is used in reference template generation. The template matching cost is measured by the SAD between the neighbouring samples of the current CU and their corresponding reference samples, as illustrated in Figure 1.

Figure 1. Template matching cost derivation in BVDSP

In general case 4 BVD candidates are evaluated (+/– bvd_abs.x, +/– bvd_abs.y). If one of BVD components (x or y) is zero, only 2 BVD candidates are possible. Case when all BVD component are zero is trivial and no sign derivation is needed.

Experimental results

The proposed method was implemented based on ECM-6.0 under common test conditions [3]. Table 1 summarizes the results with all intra and random access configurations.

Table 1. Simulation results of the proposed method

|  |  |  | All Intra Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.21% | -0.25% | -0.04% | #NUM! | #NUM! |
| Class TGM | -0.43% | -0.40% | -0.43% | #NUM! | #NUM! |

|  |  |  | Random Access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.06% | -0.04% | -0.11% | #NUM! | #NUM! |
| Class TGM | -0.27% | -0.36% | -0.28% | #NUM! | #NUM! |

Conclusions

In this contribution, instead of direct signaling of BVD signs in EP mode, BVDSP mode is proposed where possible BVD signs combinations are sorted according to template matching cost and index of true BVD signs is coded with context model. Compared to ECM-6.0, it is reported that the proposed method shows average BD-rates of -0.21% and -0.43% for class F and TGM under AI case and average BD-rates of -0.06% and -0.27% for class F and TGM under RA case. It is suggested to investigate in EE.

- 5 References

- ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- Yan Zhang, Bappaditya Ray, “Non-EE2: On MVD sign prediction”, JVET-X0132, October 2021.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).