| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0124-v1 |
| --- | --- |

| Title: | EE2-2.3: POC based BCW weights derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang, Han Huang, Chun-Chi Chen, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of test EE2-2.3. The test proposed to add BCW weights derived considering POC difference between reference and current pictures. The proposed change is used for AMVP and merge modes. It was implemented on top of ECM-6.0 and the test results are as follows:

RA: -0.10% (Y), -0.10% (U), -0.16% (V), 105% (EncT), 99% (DecT).

LDB: -0.10% (Y), -0.03% (U), 0.22% (V), 101% (EncT), 99% (DecT)

# Introduction

In ECM-6.0, BCW uses an index to select a weight value to be used in bi-prediction weighting to code the current block. Depending on whether the current picture is a low-delay picture (all reference pictures are from the past relatively to the current picture), BCW weight is selected from the set of { -2, 3, 4, 5, 10 }, or from { 3, 4, 5 }, otherwise.

When the current picture is a non-low-delay picture, it is possible to have two reference pictures both from the past or both from the future relatively to the current picture. In this scenario, a block may prefer to use negative weight value on one reference picture, however, in the current BCW design, only positive weight values can be used for a non-low-delay picture.

# Proposal

This contribution proposes to add BCW weights derived based on POC difference as follows:

- If both reference pictures are from the past or from the future relatively to the current picture, and the current picture is a non-low-delay picture, the weight pair (-2, 10) is added.

- Otherwise, the weight pair (2, 6) is added.

The larger value from the weight pair is assigned to the closest POC reference picture or list 0 reference picture when the POC distance is the same. The proposed method is not used with MHP for AMVP mode. One bypass bin is added to indicate whether to use default BCW weight or POC based BCW weight. The rest of BCW index signaling is remain as in ECM-6.0 and the first bin is context coded based on the low delay condition.

# Simulation results

The proposed method was implemented on top of ECM-6.0. The simulation was conducted under the common test conditions as defined in [2].

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.21% | -0.01% | 107% | 100% |  |  |  |  |  |
| Class A2 | -0.10% | -0.11% | -0.11% | 108% | 99% |  |  |  |  |  |
| Class B | -0.12% | -0.06% | -0.28% | 106% | 102% | -0.08% | -0.13% | -0.08% | 102% | 99% |
| Class C | -0.07% | -0.06% | -0.15% | 102% | 96% | -0.12% | -0.06% | 0.24% | 103% | 98% |
| Class E |  |  |  |  |  | -0.11% | 0.19% | 0.71% | 98% | 102% |
| Overall | -0.10% | -0.10% | -0.16% | 105% | 99% | -0.10% | -0.03% | 0.22% | 101% | 99% |
| Class D | -0.07% | -0.09% | -0.28% | 102% | 100% | -0.02% | 0.33% | 0.07% | 102% | 94% |
| Class F | -0.14% | -0.12% | -0.13% | 100% | 99% |  |  |  |  |  |

# References

[1] ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).