| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0127-v1 |
| --- | --- |

| Title: | EE2-1.15: Horizontal and vertical planar modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li Ru-Ling Liao Jie Chen Yan Ye | Tel: Email: | {sid.lxw, ruling.lrl, jiechen.cj yan.ye,}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the simulation results of EE2 Test 1.15. In Test 1.15, two new additional planar modes are proposed where only the horizontal interpolation or only the vertical interpolation are used to obtain the predicted samples. It is reported that on top of ECM-6.0, the overall coding performance impact for {Y, U, V, EncT, DecT} are {-0.08%, 0.06%, 0.06%, 115%, 100%} for AI and {-0.03%, 0.02%, -0.04%, 102%, 100%} for RA.

Introduction

In ECM-6.0 [1], planar mode predicts the current sample from the reconstructed values of 4 reference samples as shown in Figure 1. Specifically, linear interpolation in the horizontal direction and vertical direction are performed respectively, and the two results are averaged to obtain the predicted sample, as shown in the following equations:

predVx,y=H-1-y*recx,-1+y+1*rec-1,H≪log2W

predHx,y=W-1-x*rec-1,y+x+1*recW,-1≪log2H

predx,y=predVx,y+predHx,y+W*H≫(log2W+log2H+1)

Figure 1. The reference samples used in planar mode

In order to improve the coding efficiency of planar mode, horizontal and vertical planar modes are proposed in the last JVET meeting [2], and studied in EE2 Test 1.15 [3]. This contribution reports the simulation results of EE Test 1.15.

Proposed methods

In Test 1.15, two additional planar mode: horizontal planar mode and vertical planar mode are tested.

For horizontal planar mode, only the horizontal linear interpolation is performed based on the left reference sample and the top-right reference sample to predict the current sample as:

predx,y=W-1-x*rec-1,y+x+1*recW,-1+(W≫1)≫log2W

For vertical planar mode, only the vertical linear interpolation is performed based on the above reference sample and the bottom-left reference sample to predict the current sample as:

predx,y=H-1-y*recx,-1+y+1*rec-1,H+(H≫1)≫log2H

The proposed two additional planar modes are only applied to the luma component and is not used for ISP coded blocks. When the current block enables one of the two proposed planar modes, the block's propagation mode is set to the original planar mode.

For signaling, if the planar flag indicates that a planar mode is used for the current block and the current block is a non-ISP coded luma block, a syntax element is further signaled by truncated unary code to indicate which of the original planar mode, the horizontal planar mode and the vertical planar mode is selected to predict the current block.

Simulation results

The proposed methods are implemented on top of ECM-6.0 and are tested under CTC for enhanced compression tool testing [4] with all intra and random access configurations. The simulation result of Test 1.15 is shown in Table 1.

Table 1. The simulation results of Test 1.15

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.13% | 0.07% | 115% | 100% |
| Class A2 | -0.10% | 0.07% | 0.03% | 114% | 100% |
| Class B | -0.07% | 0.13% | 0.08% | 115% | 98% |
| Class C | -0.06% | 0.09% | 0.06% | 114% | 100% |
| Class E | -0.13% | 0.08% | 0.06% | 115% | 100% |
| Overall | -0.08% | 0.06% | 0.06% | 115% | 100% |
| Class D | -0.03% | 0.13% | -0.01% | 114% | 100% |
| Class F | -0.08% | -0.04% | 0.11% | 107% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.13% | 0.08% | 102% | 99% |
| Class A2 | -0.06% | 0.15% | -0.12% | 101% | 99% |
| Class B | -0.03% | 0.13% | -0.20% | 102% | 100% |
| Class C | 0.03% | -0.10% | 0.11% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.02% | -0.04% | 102% | 100% |
| Class D | 0.05% | 0.11% | -0.26% | 101% | 100% |
| Class F | 0.02% | -0.09% | -0.10% | 101% | 101% |

References

- M. Coban and et. al., “Algorithm description of Enhanced Compression Model 6 (ECM 6),” JVET-AA2025, Oct. 2022.

- X. Li and et. al., “Non-EE2: On planar horizontal mode and planar vertical mode,” JVET-AA0104, July 2022.

- V. Seregin and et al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AA2024, July 2022.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Feb. 2022.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).