| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0129 |
| --- | --- |

| Title: | EE2-1.1: Reduced Complexity Spatial GPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K.Naser Y. Chen A. Robert K. Reuze | Email: | karam.naser@interdigital.com |  |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution presents EE2-1.1 test results of the simplification of SGPM. On top of ECM-6.0, simulation results are reported as follows:

- AI: -0.15%	-0.17%	-0.18%	with 105% EncT and 107% DecT.

- RA: xx%	xx%	xx%	with xx% EncT and xx% DecT.

# Proposed method

The test is about simplifying spatial GPM (SGPM) based on JVET-AA0045. Specifically, the following modifications are made:

- Block size restriction:
4<=width<=64, 4<=height<=64, width<height*8, height<width*8, width*height>=32.

- 26 predefined partition modes are used.

- For each partition mode, an IPM list is derived for each part. The IPM list size is 3. The IPM list derivation method reuses the method introduced by intra-inter GPM, and the only difference is TIMD derived mode which is replaced by 2 derived modes with horizontal and vertical orientations, both derived by a modified TIMD process.

- The template size is set equal to 1, i.e. the height of the top template part is 1-pel and the width of the left template part is also 1-pel.

- Possible combinations of one partition mode and two intra prediction modes are ranked in ascending order based on their SAD between the prediction and reconstruction of the template. The weights in the template are either 1 or 0 according to the partition mode. The candidate list length of combinations is set equal to 16.

# Experimental results

The proposed method is tested on top of ECM-6.0 software under the common test conditions [1] in AI and RA configurations. The following results are obtained:

Table 1: performance of the proposed method over ECM-6.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.14% | -0.18% | 105% | 104% |
| Class A2 | -0.18% | -0.24% | -0.23% | 104% | 108% |
| Class B | -0.11% | -0.07% | -0.05% | 104% | 107% |
| Class C | -0.10% | -0.16% | -0.16% | 107% | 110% |
| Class E | -0.29% | -0.31% | -0.39% | 104% | 107% |
| Overall | -0.15% | -0.17% | -0.18% | 105% | 107% |
| Class D | -0.05% | -0.06% | -0.21% | 103% | 111% |
| Class F | -0.06% | -0.18% | -0.01% | 101% | 105% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | 96% | 85% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.02% | -0.10% | 103% | 108% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.08% | -0.02% | -0.35% | 101% | 105% |
| Class C | -0.05% | -0.05% | -0.04% | 101% | 106% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.04% | -0.09% | 0.03% | 103% | 110% |
| Class F | -0.05% | -0.03% | -0.11% | 102% | 109% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# References

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).