| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0135-v1 |
| --- | --- |

| Title: | AHG9: On complexity metrics for NN post-filter characteristics SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Jacob Ström | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to add two additional syntax elements descriptive of the complexity of the neural network signalled by the NN postfilter characteristics SEI message. The proposed additional syntax elements are nnpfc_total_kilobyte_size and nnpfc_num_layers.

In addition, it is asserted that the current extension mechanism in the syntax structure for the complexity elements does not allow for signalling additional complexity syntax elements while also signaling the first set of complexity syntax elements. It is proposed to modify this such that each bit in the binary representation of nnpfc_complexity_idc corresponds to one group of syntax elements.

# Introduction

At the 26th JVET meeting the NN postfilter characteristics SEI message of JVET-Z0244 was adopted. The post-filter characteristics SEI message specifies information necessary to apply a NN post-filter to a decoded picture. It also includes information descriptive of the complexity of the NN based post-filter. The relevant parts of the SEI message as in JVET-AA2006-v2 is shown below.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| nnpfc_complexity_idc | ue(v) |
| if( nnpfc_complexity_idc > 0 ) |  |
| nnpfc_complexity_element( nnpfc_complexity_idc ) |  |
| … |  |
| } |  |
| } |  |
| … |  |
| } |  |

| nnpfc_complexity_element( nnpfc_complexity_idc ) { | Descriptor |
| --- | --- |
| if( nnpfc_complexity_idc = = 1 ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if (nnpfc_parameter_type_idc ! = 2) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| } |  |
| } |  |

nnpfc_complexity_idc greater than 0 specifies that one or more syntax elements that indicate the complexity of the post-processing filter associated with the nnpfc_id may be present. nnpfc_complexity_idc equal to 0 specifies that no syntax elements that indicate the complexity of the post-processing filter associated with the nnpfc_id are present. The value nnpfc_complexity_idc shall be in the range of 0 to 255, inclusive. Values of nnpfc_complexity_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_complexity_idc.

nnpfc_parameter_type_idc equal to 0 indicates that the neural network uses only integer parameters. nnpfc_parameter_type_flag equal to 1 indicates that the neural network may use floating point or integer parameters. nnpfc_parameter_type_idc equal to 2 indicates that the neural network uses only binary parameters. nnpfc_parameter_type_idc equal to 3 is reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved value of nnpfc_parameter_type_idc.

nnpfc_log2_parameter_bit_length_minus3 equal to 0, 1, 2, and 3 indicates that the neural network does not use parameters of bit length greater than 8, 16, 32, and 64, respectively. When nnpfc_parameter_type_idc is present and nnpfc_log2_parameter_bit_length_minus3 is not present the neural network does not use parameters of bit length greater than 1.

nnpfc_num_parameters_idc indicates the maximum number of neural network parameters for the post processing filter in units of a power of 2048. nnpfc_num_parameters_idc equal to 0 indicates that the maximum number of neural network parameters is not specified. The value nnpfc_num_parameters_idc shall be in the range of 0 to 52, inclusive. Values of nnpfc_num_parameters_idc greater than 52 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_num_parameters_idc.

If the value of nnpfc_num_parameters_idc is greater than zero, the variable maxNumParameters is derived as follows:

maxNumParameters = ( 2048  <<  nnpfc_num_parameters_idc ) − 1	(89)

It is a requirement of bitstream conformance that the number of neural network parameters of the post-processing filter shall be less than or equal to maxNumParameters.

nnpfc_num_kmac_operations_idc greater than 0 specifies that the maximum number of multiply-accumulate operations per sample of the post-processing filter is less than or equal to nnpfc_num_kmac_operations_idc * 1000. nnpfc_num_kmac_operations_idc equal to 0 specifies that the maximum number of multiply-accumulate operations of the network is not specified. The value of nnpfc_num_kmac_operations_idc shall be in the range of 0 to 232 − 1, inclusive.

# Problem statement

The complexity parameters signalled in the NN post filter characteristics SEI message gives an indication of the complexity of the NN network. However, we believe that including additional parameters descriptive of the complexity would be helpful for a decoder in determining whether it will be capable to run the NN post-filter, in particular the total size needed for storing the NN and the number of layers used by the NN.

The syntax structure for the complexity elements has a mechanism for extending the SEI message with additional complexity parameters by allowing for future specifications to use the reserved values of nnpfc_complexity_idc. However, with a new value of nnpfc_complexity_idc, e.g. 2, the original complexity parameters signalled when nnpfc_complexity_idc is equal to 1, may not be signalled at the same time. For an extension, it is expected that also the original complexity parameters may be useful to have and not only the new parameters corresponding to the new value of nnpfc_complexity_idc.

# Proposals

We propose to include two additional syntax elements descriptive of the complexity of the neural network signalled by the NN postfilter characteristics SEI message. The proposed additional syntax elements are nnpfc_total_kilobyte_size and nnpfc_num_layers.

In addition, we propose to fix the extension mechanism of the syntax structure for the complexity elements such that each bit in the binary representation of nnpfc_complexity_idc corresponds to one group of syntax elements.

## Proposed syntax and semantics

Proposed syntax and semantics on top of JVET-AA2006v2 are shown below with added text in yellow:

| nnpfc_complexity_element( nnpfc_complexity_idc ) { | Descriptor |
| --- | --- |
| if( ( nnpfc_complexity_idc & 1) = = 1 ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if (nnpfc_parameter_type_idc ! = 2) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| nnpfc_num_layers | ue(v) |
| } |  |
| } |  |

nnpfc_total_kilobyte_size greater than 0 indicates a total size in kilobytes required to store the uncompressed parameters for the neural network. The total size in bytes is a number equal to or greater than (NumParams1bit + 8 · NumParams8bits + 16 · NumParams16bits + 32 · NumParams32bits + 64 · NumParams64bits) / 8, where NumParamsNbits is the number of parameters stored with N bits. nnpfc_total_kilobyte_size is the total size in bytes divided by 1000, rounded up. nnpfc_total_kilobyte_size equal to 0 indicates that the total size required to store the parameters for the neural network is not specified. The value of nnpfc_total_kilobyte_size shall be in the range of 0 to 232 − 1, inclusive.

nnpfc_num_layers greater than 0 indicates a maximum number of layers used by the neural network. nnpfc_num_layers equal to 0 indicates that a maximum number of layers used by the neural network is not specified. The value of nnpfc_num_layers shall be in the range of 0 to 232 − 1, inclusive.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).