| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0138 |
| --- | --- |

| Title: | EE2-related: MRL candidate list reordering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yujin Lee (SKKU) Bumyoon Kim (SKKU) Byeungwoo Jeon (SKKU) 2066 Seoburo, Jangangu Suwon, Korea | Tel: Email: | +82-31-290-7186 yjlee0816@skku.edu kbumyoon@skku.edu bjeon@skku.edu |
| Source: | Sungkyunkwan University (SKKU) |  |  |

_____________________________

- Abstract

This contribution proposes a reordering scheme of the MRL candidate list. The MRL candidate list is reordered according to the SAD between the predictor generated with adjoined reference line (i.e., intra_luma_ref_idx=0) and the predictor generated with another reference line in MRL candidate list (i.e., intra_luma_ref_idx≠0).

Proposed method achieves BDBR gain compared to ECM-6.0 as follows:

AI - -0.01% (Y) / 0.04% (Cb) / -0.03% (Cr), Enc: 106%, Dec: 101%.

Introduction

In VVC, the multiple reference line (MRL) candidate list consists of two reference lines, i.e., {1, 2}. Extension of MRL candidate list proposed in JVET-Y0116 allows to construct MRL candidate list with 5 reference lines, i.e., {1, 3, 5, 7, 12}. Each reference line in the MRL candidate list is coded using bin strings of 10, 110, 1110, 11110, 11111. In the current design of ECM, the order of reference lines in MRL candidate list is kept unchanged in all CUs.

Proposed method

There are 5 reference lines in the MRL candidate list, i.e., {1, 3, 5, 7, 12}. Each number in the MRL candidate list is a reference line offset. The reference line offset corresponding to intra_luma_ref_idx is shown in Table 1.

Figure 1. reference line offset and intra_luma_ref_idx according to reference line

Table 1. Reference line offset and binarization according to intra_luma_ref_idx

| intra_luma_ref_idx | reference line offset | Bin string |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 1 | 10 |
| 2 | 3 | 110 |
| 3 | 5 | 1110 |
| 4 | 7 | 11110 |
| 5 | 12 | 11111 |

This contribution proposes a reordering scheme of MRL candidate list. MRL candidate list is reordered according to the SAD (Sum of Absolute Differences) between the predictor generated with adjoined reference line (i.e., intra_luma_ref_idx=0) and the predictor generated with each reference line in MRL candidate list (i.e., intra_luma_ref_idx≠0). The SAD is expressed as follows:

SADi=SADpred0, predi,  i∈ MRL candidate list,

where predi indicates the predictor generated using the reference line with offset i, and pred0 indicates the predictor generated using the reference line with offset 0.

For example, if SAD5<SAD1<SAD12<SAD7<SAD3, the reference line offset corresponding to intra_luma_ref_idx after reordering is shown in Table 2. Note that, intra_luma_ref_idx=0 always indicates reference line offset 0.

Table 2. Reference line offset and binarization according to intra_luma_ref_idx after reordering

| intra_luma_ref_idx | reference line offset | Bin string |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 5 | 10 |
| 2 | 1 | 110 |
| 3 | 12 | 1110 |
| 4 | 7 | 11110 |
| 5 | 3 | 11111 |

Simulation Results

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.12% | -0.10% | 107% | 100% |
| Class A2 | -0.01% | 0.04% | -0.02% | 107% | 101% |
| Class B | -0.02% | 0.08% | -0.03% | 106% | 101% |
| Class C | -0.03% | 0.08% | -0.06% | 104% | 101% |
| Class E | -0.01% | 0.09% | 0.04% | 104% | 100% |
| Overall | -0.01% | 0.04% | -0.03% | 106% | 101% |
| Class D | -0.01% | 0.14% | -0.15% | 104% | 100% |
| Class F | 0.00% | -0.05% | -0.10% | 106% | 101% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusions

Proposed reordering scheme for MRL candidate list improves coding performance with an increase of the complexity by 4%. It is recommended to further study this method in EE2.

Reference

- K. Cao, et. al., “EE2-2.1: Extended MRL candidate list,” JVET-Y0116, January 2022.

Patent rights declaration(s)

Sungkyunkwan University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).