| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0146-v2 |
| --- | --- |

| Title: | EE1-1.8-related: encoder-only optimization for NN based in-loop filter with a single model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liqiang Wang Xiaozhong Xu Shan Liu | Email: | liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, an encoder-only tool for EE1-1.8, which is derived based on EE1-1.6, is studied on top of NCS-1.0 (filter set #0). The specific encoder implementation is same to one of EE1-1.6, but the filters in the RDO process are retained based on the network structure of in-loop filter in EE1-1.8. Then, the well-trained models are used to further enhance the solution in EE1-1.8. Based on the NNVC-2.0, the test results are shown in order of RA and AI configurations as follows.

RA: -9.43%	-19.68%	-19.12%	EncT: 158%	DecT: 65467%

AI : -7.06%		-15.37%	-16.46%	EncT: 172%	DecT: 34972%

Compared with EE1-1.8 under RA and AI configurations, additional 0.25% (9.18% to 9.43%) and 0.43% (6.63% to 7.06) BD-rate luma gains are observed. In terms of decoder side, there is not any increase in both MAC result and memory size. As for the runtime at encoder side, there are about 8% (150% to 158%) and 26% (146% to 172%) increase additionally under RA and AI configurations, respectively.

Introduction

In JVET-AA0113 [], a NN filter is involved in the partitioning decision process, and this solution is further studied in EE1-1.6. In this contribution, this encode-only tool is introduced into NCS-1.0 (filter set #0) []. The encoder implementation is same to the one of EE1-1.6, but the filters in the RDO process are retained based on the network structure of in-loop filter in EE1-1.8. Then, the well-trained models are used to further enhance the solution in EE1-1.8.

The network structure of the proposed NN filter in the RDO process is shown in Fig. 1.

Along with the reconstructed image (rec_y), additional side information is also fed into the network, such as the prediction image (pred_y) [], slice QP and base QP. As for the structure of Resblock, the number of channels firstly goes up before the activation layer, and then goes down after the activation layer.

The network structure is similar to the one in EE1-1.8, but there are several differences as follows.

- Only 4 Resblocks are used in network structure of the proposed filter.

- There are two models for I slices and B slices, respectively.

- Only luma related images are fed into the network.

Fig. 1: Network architecture

The related information of the proposed models only used at encoder side is shown in the following table.

| Number of Parameters (Each Model) | 279K |
| --- | --- |
| Parameter Precision (Bits) | 16 |
| Memory Parameter (MB) | 558KB/model, 2 models in all |
| Multiply Accumulate (kMAC/sample) | 77.5K |
| Calculation Method | block basis (128x128) |

Network information in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla V100 32GB |
|  | Framework: | Pytorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~100 |
|  | Batch size: | 64 |
|  | Loss function: | L1 |
|  | Training time (for 1 model): | ~60h |
|  | Training data information: | DIV2K, TVD, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations | 1200 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Learning rate update strategy |  |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Training data update strategy: |  |
|  | Other information: |  |

Network information in inference stage

In the inference stage, SADL [] is adopted as the inference library. In addition, all the models in this contribution are quantized into int16 precision.

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.9M |
|  | Total Number of Parameters (All Models) | 1.9M |
|  | Parameter Precision (Bits) | 16 |
|  | Memory Parameter (MB) | 3.8MB/model, 1 model in all |
|  | Multiply Accumulate (kMAC/pixel) | 485K (w/o block extension) 615K (w/ block extension) |
|  | Calculation Method | block basis |
| Optional |  |  |
|  | Total Conv. Layers | 101 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Results

All the experiments are performed based on the common test conditions in []. The test result of the proposed method is provided in Table 1. For the convenient comparison, the test result of EE1-1.8.2 is also provided here in Table 2. The average results can also be summarized as follows.

The proposed method:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -9.43% | -19.68% | -19.12% | 158% | 65467% |
| AI | -7.06% | -15.37% | -16.46% | 172% | 34972% |

EE1-1.8.2:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -9.18% | -19.65% | -19.17% | 150% | 65582% |
| AI | -6.63% | -15.33% | -16.40% | 146% | 35084% |

Table 1. Performance of the proposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.56% | -16.00% | -18.26% | 164% | 71169% |
| Class A2 | -10.13% | -19.20% | -15.07% | 162% | 66996% |
| Class B | -8.96% | -19.94% | -19.18% | 164% | 68640% |
| Class C | -9.39% | -22.48% | -22.73% | 143% | 56964% |
| Class E |  |  |  |  |  |
| Overall | -9.43% | -19.68% | -19.12% | 158% | 65467% |
| Class D | -10.36% | -20.94% | -22.74% | 143% | 50104% |
| Class F | -4.19% | -13.30% | -11.98% | 194% | 28156% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.52% | -14.30% | -16.73% | 218% | 47139% |
| Class A2 | -6.24% | -15.60% | -12.98% | 175% | 38162% |
| Class B | -6.42% | -15.27% | -16.71% | 167% | 34936% |
| Class C | -7.08% | -16.24% | -18.59% | 148% | 23570% |
| Class E | -9.45% | -15.23% | -16.41% | 173% | 40304% |
| Overall | -7.06% | -15.37% | -16.46% | 172% | 34972% |
| Class D | -6.93% | -15.37% | -17.99% | 141% | 21708% |
| Class F | -4.52% | -12.25% | -12.20% | 126% | 29028% |

Table 2. Performance of EE1-1.8.2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.48% | -16.04% | -18.26% | 159% | 71370% |
| Class A2 | -9.84% | -19.26% | -14.99% | 154% | 67099% |
| Class B | -8.72% | -19.88% | -19.33% | 155% | 68824% |
| Class C | -9.04% | -22.36% | -22.80% | 135% | 56964% |
| Class E |  |  |  |  |  |
| Overall | -9.18% | -19.65% | -19.17% | 150% | 65582% |
| Class D | -10.06% | -21.18% | -22.57% | 133% | 50114% |
| Class F | -4.10% | -13.15% | -12.05% | 187% | 28434% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.20% | -14.25% | -16.91% | 193% | 47275% |
| Class A2 | -5.83% | -15.58% | -12.94% | 151% | 38308% |
| Class B | -5.99% | -15.20% | -16.60% | 140% | 34985% |
| Class C | -6.60% | -16.33% | -18.55% | 121% | 23724% |
| Class E | -8.94% | -15.05% | -16.12% | 148% | 40366% |
| Overall | -6.63% | -15.33% | -16.40% | 146% | 35084% |
| Class D | -6.52% | -15.21% | -18.12% | 114% | 21619% |
| Class F | -4.23% | -12.11% | -12.15% | 117% | 29366% |

Conclusions

In this contribution, the performance of EE1-1.8 is further improved by introducing the proposed encoder-only method. Compared with EE1-1.8 under RA and AI configurations, additional 0.25% (9.18% to 9.43%) and 0.43% (6.63% to 7.06) BD-rate luma gains are observed. In terms of decoder side, there is not any increase in both MAC result and memory size. As for the runtime at encoder side, there are about 8% (150% to 158%) and 26% (146% to 172%) increase additionally under RA and AI configurations, respectively.

References

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[] J. Li, Y. Li, K. Zhang, L. Zhang. EE1-1.6-related: RDO Considering Deep In-Loop Filter with SADL. JVET-AA0113, 27th Meeting, by teleconference, 13–22 July 2022.

[] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tags/NCS-1.0.

[] H. Zhu, X. Xu, S. Liu. Deep learning based in-loop filter with QP and residual distribution information. AVS-M5654, Aug. 2020.

[] F. Galpin, T. Dumas, P. Bordes, E. François. AHG11: Small Ad-hoc Deep-Learning Library (SADL) update. JVET-AA0086, 27th Meeting, by teleconference, 13–22 July 2022.

[] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AA2016, 27th Meeting, by teleconference, 13–22 July 2022.