| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0150-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AB0118 (EE2-2.5a: Enhanced temporal motion information derivation) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lien-Fei Chen, Guichun Li, Xiaozhong Xu, Xin Zhao, Shan Liu 2747 Park Boulevard Palo Alto, CA 94306, U.S.A. | Email: | lienfeichen@global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results of EE2-2.5 proposed in JVET-AB0118. Since EE2-2.5b was withdrawn by the proponent, only EE2-2.5a is crosschecked in this report. EE2-2.5a proposes two aspects to improve the temporal motion information derivation process for SbTMVP and TMVP candidates. Firstly, two collocated pictures are utilized to provide temporal motion information. Secondly, the motion shift to locate TMVP is adaptively determined from multiple locations according to template costs. The crosschecking BD-rate results match with the results reported in JVET-AB0118. During the study of the software, an issue of the template matching process for SbTMVP mode was identified in EE2-2.5a, and a modified version is proposed in this contribution. The modified version on top of EE2-2.5a was tested and the following results were observed over ECM-6.0 anchor.

RA: {xxx%, xxx%, xxx%; xxx%, xxx%};

LDB: {xxx%, xxx%, xxx%; xxx%, xxx%}.

# Crosscheck results

In EE2-2.5a, the test result is shown in the following table.

Table 1: Simulation results of Test 2.5a in RA and LB test conditions

| Test 2.5a | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.23% | -0.05% | 101% | 102% |
| Class A2 | -0.11% | -0.09% | -0.19% | 101% | 102% |
| Class B | -0.07% | -0.11% | -0.23% | 102% | 102% |
| Class C | -0.20% | -0.18% | -0.21% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.15% | -0.18% | 102% | 102% |
| Class D | -0.12% | -0.11% | -0.05% | 103% | 103% |
| Class F | -0.03% | -0.13% | -0.05% | 102% | 102% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Test 2.5a | Low delay B |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.27% | -0.28% | -0.20% | 105% | 106% |
| Class E | -0.04% | 0.76% | 0.54% | 103% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.16% | -0.06% | 0.82% | 104% | 105% |
| Class F | -0.12% | -0.46% | -0.41% | 102% | 104% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Subblock-based template-matching for SbTMVP in Test 2.5a

In Test 2.5a, the determination of the inter prediction direction is based on the inter prediction direction of center MV of SbTMVP for all subblock templates. When the center MV is bi-prediction MV, each subblock template is a weighted average of the subblock reference template from reference list 0 and the subblock reference template from reference list 1. For the subblock reference template from each reference list, the MV is derived from the adjacent subblock MV of SbTMVP if the reference index of that reference list is valid, otherwise, a (0, 0) motion vector is used as MV to point to the subblock reference template. Figure 1 shows two examples of the MV derivation for each subblock template in a 16×16 coding block where each square block in white represents a 4×4 subblock within SbTMVP and each gray block stands for the subblock current template. Figure 1(a) shows an example of the MV derivation for each subblock template when the center MV of SbTMVP is a bi-prediction MV, and Figure 1(b) shows another example of the MV derivation for each subblock template when the center MV of SbTMVP is a uni-prediction MV. Based on the above example, the MV of each subblock template cannot be directly derived from the adjacent subblock MV of SbTMVP in current Test 2.5a.

- Center MV is bi-prediction                   (b) Center MV is uni-prediction

Figure 1: Example of subblock-based template-matching for SbTMVP in Test 2.5a

# Proposed subblock-based template-matching for SbTMVP

In order to address the above issue, a straightforward method of subblock-based template-matching is proposed for Test 2.5a. In the proposed method, when a subblock in template area has bi-prediction MV, the bi-prediction is applied to obtain the corresponding sub-block reference template; When a subblock has uni-prediction MV, uni-prediction is applied to get the subblock reference template. Finally, the reference template is comprised of these subblock templates, and the template matching cost is derived from the distortion between the current template and the reference template. Figure 2 shows an example of the proposed method for a 16×16 coding block, where each square block in white represents a 4×4 subblock within SbTMVP and each gray block stands for the subblock template. In Figure 2, the MV of each subblock template is simply copied from the adjacent subblock MV of SbTMVP.

Figure 2: An example of proposed subblock-based template-matching for SbTMVP mode.

The overall performance of the proposed approach on top of Test 2.5a is shown in Table 2.

Table 2: Simulation results of the proposed method on top of Test 2.5a over ECM-6.0.

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.07% | -0.06% | -0.25% | 102% | 102% |
| Class C | -0.19% | -0.10% | -0.21% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.15% | -0.13% | -0.09% | 102% | 103% |
| Class F | -0.05% | -0.12% | -0.09% | 102% | 102% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.29% | -0.25% | -0.04% | 103% | 106% |
| Class E | -0.02% | -0.07% | 0.21% | 102% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.18% | -0.25% | 0.13% | 103% | 105% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

Test EE2-2.5a was crosschecked and the BD-rates match with the result provided by the proponent. An issue was identified on the template matching process of SbTMVP in the implementation, and a modified approach was provided and tested. It is proposed to apply the modification to EE2-2.5a.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).