| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0157-v2 |
| --- | --- |

| Title: | EE2-1.12: Combination of EE2-1.10 and EE2-1.11 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu Yue Yu Haoping Yu Dong Wang (OPPO) Hongtao Wang Vadim Seregin Bappaditya Ray Marta Karczewicz (Qualcomm) | Tel: Email: | xuluhang@oppo.com yue.yu@oppo.com v-yuhaoping@oppo.com wangdong7@oppo.com hongtaow@qti.qualcomm.com vseregin@qti.qualcomm.com bray@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | OPPO, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents the results of EE2-1.12a (the combination of EE2-1.10 and EE2-1.11a) and EE2-1.12b (the combination of EE2-1.10 and EE2-1.11b).

EE2-1.10 is a method with template-based multiple reference line intra prediction (TMRL).

EE2-1.11a is an intra prediction fusion method that blends intra predictors derived from up to two different reference lines.

EE2-1.11b is an alternative method in which fusion is performed on the two reference lines instead, and then an intra predictor is derived from the fused reference line.

It is reported that, on top of ECM-6.0 [1], the overall coding performance improvement for {Y, U, V, EncT, DecT} is:

EE2-1.12a:

AI -0.26%/-0.14%/-0.13%, Enc: 104%, Dec: 105%,

RA -0.11%/-0.02%/-0.08%, Enc: 102%, Dec: 99%

EE2-1.12b:

AI 0.09%/0.19%/0.19%, Enc: 105%, Dec: 104%,

RA 0.04%/0.06%/-0.01%, Enc: 101%, Dec: 99%

# Introduction

## EE2-1.10 description

The TMRL mode proposed in JVET-AA0120 [3] includes the following aspects:

- Extended reference line candidate list and the intra-prediction-mode candidate list.

The extended reference line candidate list is {1, 3, 5, 7, 12}. The restriction on the top CTU row is unchanged. The size of the intra-prediction-mode candidate list is 10. The construction of the intra-prediction-mode candidate list is similar to MPM. The differences are that

- PLANAR mode is excluded from the proposed intra-prediction-mode candidate list.

- DC mode is added after 5 neighboring PUs’ modes and DIMD modes if it is not included.

- The angular modes with delta angles from ±1 to ±4 (compared the existing angular modes in the intra-prediction-mode candidate list) are added.

- Construction of the TMRL candidate list.

There are 5x10=50 combinations of the extended reference line and the allowed intra-prediction modes for a block. Since the extended reference line starts from reference line 1, the area covered by reference line 0 is used for template matching. The SAD costs over the template area (see Figure 1) are calculated between the predictions (generated by 50 combinations) and the reconstructions. The 20 combinations with the least SAD cost are selected in an ascending order to form the TMRL candidate list.

Figure 1. Illustration of the template area

- Signaling of TMRL

Instead of coding the reference line and the intra mode directly, an index to the TMRL candidate list is coded to indicate which combination of reference line and prediction mode is used for coding the current block. In the proposed TMRL mode, a truncated Golomb-Rice coding with a divisor 4 is employed to code selected combinations from the combination list. The binarization process and the codewords are shown in Table 1.

Table 1 the binarization process of the TMRL index

| Index | Bin string (prefix) | Bin string (suffix) |
| --- | --- | --- |
| 0 | 0 | 00 |
| 1 | 0 | 01 |
| 2 | 0 | 10 |
| 3 | 0 | 11 |
| 4 | 10 | 00 |
| … | … |  |
| 18 | 1111 | 10 |
| 19 | 1111 | 11 |

- Encoder-side modifications

Encoder-side modification is tested to further improve the coding efficiency. For intra blocks larger than 8x8, an additional TMRL RDO is added if no TMRL modes are selected by SATD comparison.

## EE2 Test-1.11 description

Test-1.11 studies intra prediction fusion using multiple predictors generated from different modes/reference lines.

In sub-test a, multiple intra predictors are generated and then fused by weighted averaging. The process of deriving the predictors to be used in the fusion process is described as follows:

- For angular intra prediction modes including the single mode case of TIMD and DIMD, the proposed method derives intra prediction by weighting intra predictions obtained from multiple reference lines represented as pfusion=w0pline+w1pline+1, where pline is the intra prediction from the default reference line and pline+1 is the prediction from the line above the default reference line. The weights are set as w0=3/4 and w1=1/4.

- For TIMD mode with blending, pline is used for the 1st mode (w0=1,w1=0) and pline+1 is used for the 2nd mode (w0=0, w1=1).

- For DIMD mode with blending, the number of predictors selected for a weighted average is increased from 3 to 6.

In sub-test b, intra prediction fusion is performed on reference lines instead of prediction blocks. Two reference lines (referred to as rline and rline+1) are used for intra prediction fusion. DeltaInt of the corresponding intra prediction angle is considered in the fusion process. Each value in the fused reference line (rfusion[i]) is derived from:

rfusioni= 3*rlinei+rline+1i+DeltaInt≫2

The proposed intra prediction fusion is applied to luma blocks when angular intra mode has non-integer slope (required reference samples interpolation) and the block size is greater than 16, it is used with MRL and not applied for ISP coded blocks. In the method studied in the sub-test a, PDPC is applied for the intra prediction mode using the closest to the current block reference line.

# Simulation Results

Both EE2 Test-1.12a and EE2 Test-1.12b were implemented on top of the ECM-6.0 [1] software and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-AA2024 [2]). The test results are summarized below.

The AI and RA results of Test-1.12a,

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.11% | -0.03% | 104% | 103% |
| Class A2 | -0.23% | -0.06% | -0.12% | 104% | 105% |
| Class B | -0.27% | -0.14% | -0.12% | 104% | 105% |
| Class C | -0.29% | -0.18% | -0.23% | 104% | 107% |
| Class E | -0.27% | -0.21% | -0.16% | 103% | 105% |
| Overall | -0.26% | -0.14% | -0.13% | 104% | 105% |
| Class D | -0.27% | -0.21% | -0.25% | 104% | 106% |
| Class F | -0.17% | -0.12% | -0.14% | 103% | 104% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.11% | -0.12% | 102% | 99% |
| Class A2 | -0.10% | 0.00% | 0.03% | 102% | 99% |
| Class B | -0.11% | 0.13% | -0.17% | 102% | 99% |
| Class C | -0.09% | -0.14% | -0.02% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.02% | -0.08% | 102% | 99% |
| Class D | -0.11% | -0.07% | -0.21% | 102% | 100% |
| Class F | -0.10% | 0.08% | -0.03% | 101% | 99% |

The AI and RA results of Test-1.12b,

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.01% | 0.05% | 105% | 102% |
| Class A2 | 0.03% | 0.09% | 0.08% | 105% | 102% |
| Class B | 0.08% | 0.23% | 0.26% | 105% | 106% |
| Class C | 0.11% | 0.23% | 0.09% | 104% | 106% |
| Class E | 0.29% | 0.35% | 0.44% | 105% | 105% |
| Overall | 0.09% | 0.19% | 0.19% | 105% | 104% |
| Class D | 0.09% | 0.10% | -0.12% | 104% | 107% |
| Class F | 0.08% | 0.08% | 0.21% | 102% | 105% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.01% | 0.06% | 101% | 100% |
| Class A2 | 0.04% | 0.08% | 0.00% | 101% | 98% |
| Class B | 0.07% | 0.15% | -0.13% | 101% | 99% |
| Class C | 0.04% | -0.04% | 0.09% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.06% | -0.01% | 101% | 99% |
| Class D | 0.04% | -0.14% | -0.10% | 101% | 99% |
| Class F | 0.02% | -0.08% | 0.07% | 101% | 100% |

# Conclusions

The proposed combination Test-1.12a can further improve the coding efficiency of ECM with a minor complexity increase. It is recommended to adopt Test-1.12a into the next version of ECM software.

# Reference

- ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-6.0

- V. Seregin, et. al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AA2024, October 2022

- L. Xu, et. al., “Non-EE2: Template-based multiple reference line intra prediction,” JVET-AA0120, October 2022

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).