| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0163-v2 |
| --- | --- |

| Title: | EE2-1.2: Test on Spatial GPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ashwin Natesan, Jay N. Shingala, Jeeva Raj Arumugam, Vaibhav Valvaiker (Ittiam) Taoran Lu, Fangjun Pu, Peng Yin, Tong Shao, Arjun Arora, Sean McCarthy (Dolby) | Email: | {jeeva.raj,jay.shingala} @ittiam.com; {tlu}@dolby.com |
| Source: | Ittiam System, Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE2 Test 1.2 for spatial GPM. A fast GPM partitioning list derivation algorithm is employed to adaptively determine the GPM partitioning mode; Intra modes in the IPM list (excluding TIMD modes) are used as candidates of SGPM intra mode; SGPM can be enabled for CU sizes of 4xH and WxH; Adaptive blending for GPM is enabled in SGPM. RDO candidates are reduced for encoder speed up.

The impact on coding efficiency and runtimes over ECM-6.0 is reportedly {for Y, U, V, EncT, DecT}:

AI:  { -0.12%, -0.15%, -0.16%, 103%, 101%}

RA: { -0.09%, -0.05%, -0.07%, 100%, 100%}

# Algorithm description

## Introduction

JVET-AA0149 [1] proposed a method to enable GPM in Intra slices (SGPM) including three aspects:

- Introduce a fast GPM partitioning list derivation algorithm to reduce the number of GPM partitions used in template-based partition type and intra mode selection.

- Use intra prediction modes, excluding the TIMD modes, in the IPM list for SGPM intra prediction modes.

- Use template size equal to 1 for SGPM candidate list derivation.

This contribution tested the performance of the AA0149 method on top of ECM6.0, with the following improvements:

- Extended CU size support to enable SGPM for 4xH and Wx4 blocks

- Enable GPM adaptive blending for spatial GPM

- Encoder speed up: RDO candidate numbers are reduced to 0 from 8

# Simulation results

The described method is implemented on top of ECM6.0 and is evaluated using the common test conditions.

Table 1 AI and RA coding performance compared with ECM6.0.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.24% | -0.11% | 103% | 102% |
| Class A2 | -0.11% | -0.12% | -0.15% | 103% | 101% |
| Class B | -0.08% | -0.04% | -0.11% | 103% | 101% |
| Class C | -0.12% | -0.15% | -0.22% | 104% | 102% |
| Class E | -0.21% | -0.30% | -0.23% | 104% | 102% |
| Overall | -0.12% | -0.15% | -0.16% | 103% | 101% |
| Class D | -0.10% | -0.10% | -0.26% | 103% | 103% |
| Class F | -0.09% | -0.27% | -0.14% | 105% | 102% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.25% | 0.06% | 100% | 99% |
| Class A2 | -0.11% | 0.02% | -0.07% | 101% | 99% |
| Class B | -0.07% | 0.02% | -0.22% | 98% | 101% |
| Class C | -0.08% | -0.03% | 0.03% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.05% | -0.07% | 100% | 100% |
| Class D | -0.07% | 0.01% | 0.02% | 103% | 101% |
| Class F | -0.03% | -0.05% | -0.20% | 100% | 103% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusions

This contribution reports EE2 test 1.2 results for spatial GPM.

# References

- A. Natesan, J. N. Shingala, J. R. Arumugam, V. Valvaiker (Ittiam), T. Lu, P. Yin, F. Pu, T. Shao, A. Arora, S. McCarthy (Dolby), “EE2-1.4 related: Improvements on Spatial GPM”, JVET-AA0149, July. 2022

# Patent rights declaration(s)

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).