| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0168-v3 |
| --- | --- |

| Title: | Non-EE2: Pixel based affine motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang, Han Huang, Yan Zhang, Patrick Garus, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to set the minimum affine subblock size to be 1×1 for luma channel of an affine coded block. It proposes to adaptively apply pixel based affine motion compensation when OBMC is not applied. It also proposes to only signal OBMC flag when the current block is an affine AMVP coded block without Multi-Hypothesis Predictor.

The proposed method was implemented on top of ECM-6.0 and the test results are as follows:

RA: -0.11% (Y), -0.02% (U), -0.05 % (V), 102% (EncT), 100% (DecT).

LDB: -0.02% (Y), 0.01% (U), 0.14% (V), 100% (EncT), 100% (DecT)

# Introduction

In ECM-6.0, an affine block is predicted by applying subblock based affine motion compensation. Given a W×H affine block, the minimum subblock size is set to 4×4, and the subblock size M×N is determined by checking the motion vector differences, wherein, 4≤M≤W and 4≤N≤H. The prediction block is possible to be refined by applying Prediction Refinement with Optical Flow (PROF).

When Local Illumination Compensation (LIC) flag is set to true, the affine predicted block is further refined by applying the LIC model. When Overlapped Block Motion Compensation (OBMC) flag is set to true, the affine predicted block is further refined on CU boundary and subblock boundaries by blending samples predicted by using neighbour motion information. LIC and OBMC can not be applied at same time.

# Proposal

This contribution proposes to set the minimum affine subblock size to be 1×1 for luma channel of an affined coded block. When OBMC is not applied to the current block, pixel based affine MC or subblock based affine MC is adaptively applied depending on the motion vector difference. When adaptive affine subblock size is determined to have either width or height is smaller than 4, PROF is disabled.

It also proposes to only signal OBMC flag when the current block is an affine AMVP coded block without Multi-Hypothesis Predictor (MHP). The OBMC flag is set to false when the current block is an affine coded block with MHP. Otherwise, the OBMC flag is set to true. The OBMC enabling condition in ECM-6.0 is unchanged, e.g. block size is larger than 16 and LIC flag is equal to false.

# Simulation results

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.09% | -0.04% | 103% | 101% |  |  |  |  |  |
| Class A2 | -0.15% | 0.09% | 0.07% | 103% | 102% |  |  |  |  |  |
| Class B | -0.18% | -0.02% | -0.18% | 102% | 101% | 0.05% | -0.13% | -0.17% | 101% | 101% |
| Class C | -0.05% | -0.06% | 0.00% | 99% | 97% | -0.10% | -0.03% | 0.12% | 100% | 99% |
| Class E |  |  |  |  |  | -0.03% | 0.31% | 0.67% | 100% | 101% |
| Overall | -0.11% | -0.02% | -0.05% | 102% | 100% | -0.02% | 0.01% | 0.14% | 100% | 100% |
| Class D | 0.01% | -0.03% | 0.02% | 100% | 99% | -0.09% | 0.04% | 0.45% | 101% | 96% |
| Class F | -0.71% | -0.36% | -0.44% |  |  | -0.25% | -0.67% | -0.56% |  |  |

# References

[1] ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).