| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0170-v1 |
| --- | --- |

| Title: | Non-EE2: Block Vector Difference Prediction for IBC blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy (Ofinno) | Tel: Email: | afilippov@ofinno.com vrufitskiy@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution presents a technique to predict signs and magnitudes of x- and y-components of block vector differences (BVDs) for IBC blocks. In particular, BVD signs and suffix bins of exponential Golomb code used to represent BVD magnitudes are predicted by estimating template matching cost of candidate blocks to take advantage of regular CABAC mode rather than its by-pass mode at the entropy coding stage. On top of ECM-6.0, the following BD-rate gain results are reportedly obtained:

- AI configuration: {-0.50%, -0.62%, -0.32%, xxx%, xxx%} for class F and {-1.57%, -1.32%, -1.38%, xxx%, xxx%} for class TGM;

- RA configuration: {-0.22%, -0.13%, 0.05%, xxx%, xxx%} for class F and {-0.63%, -0.64%, -0.62%, xxx%, xxx%} for class TGM;

- LDB configuration: {-0.xx%, -0.xx%, -0.xx%, xxx%, xxx%} for class F and {-0.xx%, -0.xx%, -0.xx%, xxx%, xxx%} for class TGM.

# Introduction

Although sign prediction is a known technique used to improve the compression performance of MVD and coefficient coding [1], BVD signs and suffix bins of exponential Golomb code used to represent BVD magnitudes are coded using by-pass mode in ECM-6.0, whereas the first 5 bins of BVD prefix are coded in a bistream with a CABAC context model. This contribution presents a technique of predicting both signs and magnitude suffixes of BVDs.

# Proposed method

In the method it is proposed to apply sign prediction to BVD (as shown in Fig. 1) and further extend this approach to predict suffix bins of BVD magnitudes (as shown in Fig. 2).

As shown in Fig. 1, instead of explicit sign coding, template matching operation is used to determine a BVD candidate with the best cost, and indicate in the bitstream whether the best candidate is predicted correctly or not.

Figure 1. Sign prediction of a BVD.

Suffix bins are also derived at the decoder side by comparing signalled bits values with the bits of the best candidates obtained with template matching.

Figure 2. Prediction of suffix bins of a BVD magnitude.

A maximum number of bins to be predicted for a PU is controlled by a macro. The most significant bins of suffixes of BVD horizontal and vertical components are predicted, and the prediction match is coded in the bitstream using CABAC context mode. The less significant bins of suffixes of horizontal and vertical components are coded in by-pass mode.

# Simulation results

The proposed methods are implemented on top of ECM-6.0 and are tested under CTC for enhanced compression tool testing [2]. Simulation results for different maximum number of predicted bins are given in Tables 1-4.

Table 1. Coding performance of the proposed method, 10 bins

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.50% | -0.62% | -0.32% | #NUM! | #NUM! |
| Class TGM | -1.57% | -1.32% | -1.38% | #NUM! | #NUM! |
| Overall | -1.04% | -0.97% | -0.85% | #NUM! | #NUM! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.22% | -0.13% | 0.05% | 101% | 100% |
| Class TGM | -0.63% | -0.64% | -0.62% | 100% | 101% |
| Overall | -0.43% | -0.39% | -0.29% | 101% | 101% |

Table 2. Coding performance of the proposed method, 8 bins

|  | Over ECM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -0.41% | -0.52% | -0.41% | #NUM! | #NUM! |
| Class TGM | -1.12% | -1.09% | -1.09% | #NUM! | #NUM! |
| Overall | -0.77% | -0.81% | -0.75% | #NUM! | #NUM! |

Table 3. Coding performance of the proposed method, 6 bins

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.37% | -0.48% | -0.27% | #NUM! | #NUM! |
| Class TGM | -0.98% | -0.94% | -0.97% | #NUM! | #NUM! |
| Overall | -0.68% | -0.71% | -0.62% | #NUM! | #NUM! |

Table 4. Coding performance of the proposed method, 4 bins

|  | Over ECM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -0.31% | -0.44% | -0.15% | #NUM! | #NUM! |
| Class TGM | -0.85% | -0.82% | -0.86% | #NUM! | #NUM! |
| Overall | -0.58% | -0.63% | -0.51% | #NUM! | #NUM! |

Figure 3. Dependency of luminance BD-rate performance improvement over maximum number of predicted BVD suffix bins.

# Conclusion

This contribution presents the method of BVD sign and BVD suffix bins prediction for improving the compression performance of IBC. Therefore, we recommend further study of this method in the Exploration Experiment 2.

# References

- M. Coban, F. Le Léannec, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 6 (ECM 6),” JVET-AA2025, July 2022.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Feb. 2022.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).