| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, De, 21–28 October 2022 | Document: JVET-AB0181-v1 |
| --- | --- |

| Title: | AHG12: Using prediction samples or residual samples for adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changyue Ma, Xiaoyu Xiu, Che-Wei Kuo, Wei Chen, Hong-Jheng Jhu, Ning Yan, Xianglin Wang | Email: | machangyue@kuaishou.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

In ECM-6.0, ALF uses the reconstructed samples after SAO and before deblocking filter as input. In this contribution, it is proposed to additionally utilize the prediction samples or the residual samples as the input to the ALF. Two filter shapes, 1x1 and 3x3 diamond shapes, are tested to be applied on the additional ALF input. The simulation results on top of ECM-6.0 are summarized as follows.

Test 1: prediction samples, 1x1 filter shape

AI: {-0.07%, 0.00 %, 0.00%, 101%, 101%}; RA: {-0.06%, 0.03 %, 0.01 %, 100%, 100%}; LDB: {0.03%, 0.08 %, 0.29%, 100%, 100%};

Test 2: prediction samples, 3x3 filter shape

AI: {-0.08%, 0.01%, 0.01%, 102%, 102%}; RA: {-0.06%, 0.10 %, 0.00 %, 101%, 101%}; LDB: {0.04%, 0.01 %, 0.31%, 101%, 101%};

Test 3: residual samples, 1x1 filter shape

AI: {-0.07%, 0.01 %, 0.01 %, 101%, 101%}; RA: {-0.07%, 0.02 %, -0.01 %, 100%, 100%}; LDB: {-0.01%, 0.10 %, 0.40 %, 100%, 100%}.

Test 4: residual samples, 3x3 filter shape

AI: {-0.07%, 0.01%, 0.01 %, 102%, 102%}; RA: {-0.07%, 0.07 %, 0.09 %, 101%, 101%}; LDB: {-0.01%, 0.01 %, 0.30%, 101%, 101%}.

Introduction

In ECM-6.0, for luma component, two fixed 13x13 diamond filters are applied to derive two intermediate samples. After that, an adaptive filter is signaled and applied to these two intermediate samples, the reconstructed samples after sample adaptive offset (SAO) and the reconstructed samples before deblocking filter (DBF) [1], as illustrated as

Rx,y=Rx, y+i=019cifi,0+fi,1+i=2021cigi+i=2224cihi,0+hi,1

(1)

where Rx, y indicates the current sample; fi,j indicates the clipped difference between a neighboring sample after SAO and Rx, y; gi indicates the clipped difference between an intermediate sample and Rx, y; hi,j indicates the clipped difference between a neighboring sample before DBF and Rx, y. The filter coefficients ci, i=0,…24, are signalled.

Proposal

In this contribution, to further improve the ALF performance, it is proposed to utilize the prediction samples or residual samples as the additional input to the ALF of the luma component. Specifically, assuming that the prediction samples are used, a filtered sample in the proposed method is derived as

Rx,y=Rx, y+i=019cifi,0+fi,1+i=2021cigi+i=2224cihi,0+hi,1+i=25Ncipi,0+pi,1

(2)

where pi,j is the clipped difference between a neighboring prediction sample and current sample Rx, y.

Alternatively, when the residual samples are used, a filtered sample is derived as

Rx,y=Rx, y+i=019cifi,0+fi,1+i=2021cigi+i=2224cihi,0+hi,1+i=25Nciri,0+ri,1

(3)

where ri,j is the clipped neighboring residual sample value. In the contribution, when either prediction samples or residual samples are applied, two filter shapes, i.e., 1x1 and 3x3 diamond shapes as shown in Fig. 1, are tested.

Figure 1. 1x1 and 3x3 filter shapes which are applied to the prediction/residual samples of the proposed ALF.

In an adaptation parameter set, a flag is signalled to indicate whether prediction samples/residual samples are used for the ALF. In this proposal, this flag is always set as true at encoder.

Experimental results

The proposed method was implemented on top EMC-6.0 [2] and tests were performed following ECM CTC [3]. The test results of the proposed method are summarized in the next tables.

Table 1. Proposed method using 1x1 filter shape for prediction samples.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | -0.07% | 0.00% | 0.00% | 101% | 98% |
| Class B | -0.06% | 0.00% | 0.00% | 101% | 101% |
| Class C | -0.09% | 0.01% | 0.01% | 101% | 102% |
| Class E | -0.06% | 0.00% | 0.00% | 100% | 102% |
| Overall | -0.07% | 0.00% | 0.00% | 101% | 101% |
| Class D | -0.05% | 0.03% | 0.03% | 100% | 101% |
| Class F |  |  |  |  |  |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.00% | 0.11% | 103% | 101% |
| Class A2 | -0.10% | 0.08% | -0.04% | 99% | 95% |
| Class B | -0.04% | 0.11% | -0.08% | 99% | 101% |
| Class C | -0.03% | -0.07% | 0.06% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.03% | 0.01% | 100% | 100% |
| Class D | -0.04% | -0.02% | 0.03% | 99% | 97% |
| Class F |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.08% | 0.42% | 100% | 100% |
| Class C | -0.03% | 0.08% | 0.31% | 100% | 101% |
| Class E | 0.08% | 0.36% | 0.06% | 101% | 101% |
| Overall | 0.03% | 0.08% | 0.29% | 100% | 100% |
| Class D | -0.10% | -0.73% | -0.11% | 101% | 103% |
| Class F |  |  |  |  |  |

Table 2. Proposed method using 3x3 filter shape for prediction samples.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | 0.01% | 0.01% | 102% | 102% |
| Class A2 | -0.09% | 0.01% | 0.01% | 102% | 104% |
| Class B | -0.07% | 0.01% | 0.01% | 102% | 101% |
| Class C | -0.09% | 0.01% | 0.01% | 101% | 102% |
| Class E | -0.07% | -0.01% | -0.01% | 102% | 102% |
| Overall | -0.08% | 0.01% | 0.01% | 102% | 102% |
| Class D | -0.05% | 0.04% | 0.04% | 102% | 101% |
| Class F |  |  |  |  |  |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.09% | 0.11% | 99% | 101% |
| Class A2 | -0.10% | 0.07% | -0.01% | 103% | 97% |
| Class B | -0.05% | 0.19% | 0.02% | 100% | 105% |
| Class C | -0.04% | 0.01% | -0.09% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.10% | 0.00% | 101% | 101% |
| Class D | -0.01% | -0.12% | -0.08% | 102% | 101% |
| Class F |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.02% | 0.27% | 101% | 101% |
| Class C | -0.11% | -0.06% | 0.41% | 100% | 101% |
| Class E | 0.19% | 0.18% | 0.26% | 102% | 101% |
| Overall | 0.04% | 0.01% | 0.31% | 101% | 101% |
| Class D | -0.06% | 0.06% | 0.51% | 97% | 106% |
| Class F |  |  |  |  |  |

Table 3. Proposed method using 1x1 filter shape for residual samples.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.01% | 0.01% | 101% | 105% |
| Class A2 | -0.07% | 0.00% | 0.00% | 101% | 101% |
| Class B | -0.06% | 0.00% | 0.00% | 101% | 99% |
| Class C | -0.09% | 0.01% | 0.01% | 102% | 102% |
| Class E | -0.06% | 0.01% | 0.01% | 101% | 100% |
| Overall | -0.07% | 0.01% | 0.01% | 101% | 101% |
| Class D | -0.05% | 0.03% | 0.03% | 101% | 103% |
| Class F |  |  |  |  |  |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.07% | 0.02% | 99% | 100% |
| Class A2 | -0.11% | 0.03% | -0.09% | 100% | 100% |
| Class B | -0.03% | 0.11% | 0.02% | 101% | 101% |
| Class C | -0.06% | -0.04% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | 0.02% | -0.01% | 100% | 100% |
| Class D | -0.06% | -0.19% | 0.00% | 100% | 100% |
| Class F |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.12% | 0.00% | 99% | 100% |
| Class C | -0.05% | -0.09% | 0.31% | 101% | 100% |
| Class E | 0.04% | 0.30% | 1.19% | 100% | 99% |
| Overall | -0.01% | 0.10% | 0.40% | 100% | 100% |
| Class D | -0.10% | 0.46% | 0.51% | 100% | 99% |
| Class F |  |  |  |  |  |

Table 4. Proposed method using 3x3 filter shape for residual samples.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.01% | 0.01% | 102% | 103% |
| Class A2 | -0.08% | 0.01% | 0.01% | 102% | 100% |
| Class B | -0.06% | 0.01% | 0.01% | 102% | 102% |
| Class C | -0.09% | 0.01% | 0.01% | 102% | 102% |
| Class E | -0.06% | -0.01% | -0.01% | 102% | 101% |
| Overall | -0.07% | 0.01% | 0.01% | 102% | 102% |
| Class D | -0.04% | 0.03% | 0.03% | 101% | 103% |
| Class F |  |  |  |  |  |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.08% | 0.12% | 100% | 99% |
| Class A2 | -0.12% | 0.22% | 0.04% | 102% | 101% |
| Class B | -0.03% | 0.10% | -0.03% | 100% | 103% |
| Class C | -0.05% | -0.09% | 0.23% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | 0.07% | 0.09% | 101% | 101% |
| Class D | -0.01% | 0.01% | 0.04% | 99% | 99% |
| Class F |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.05% | 0.17% | 101% | 101% |
| Class C | -0.09% | 0.00% | 0.13% | 103% | 102% |
| Class E | 0.04% | -0.06% | 0.74% | 99% | 101% |
| Overall | -0.01% | 0.01% | 0.30% | 101% | 101% |
| Class D | 0.02% | 0.00% | 0.63% | 99% | 97% |
| Class F |  |  |  |  |  |

Conclusion

The contribution proposes to additionally utilize the prediction samples or the residual samples as the input to the ALF. Simulation results show that approximately 0.1% coding gain can be achieved by the proposed method. It is recommended to study the proposed method in the next round of EE.

References

- N. Hu, V. Seregin, M. Karczewicz, W. Yin, K. Zhang, L. Zhang, “EE2-5: Adaptive filter shape switch and using samples before deblocking filter for adaptive loop filter”, JVET-AA0095, July 2022.

- ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-6.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).