| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, De, 21–28 October 2022 | Document: JVET-AB0191-v1 |
| --- | --- |

| Title: | Non-EE2: Combining intra block copy and intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changyue Ma, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Che-Wei Kuo, Ning Yan, Xianglin Wang | Email: | machangyue@kuaishou.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to combine IBC prediction with intra prediction. The design of the proposed method is similar to the current CIIP mode except that the inter part in CIIP is replaced with IBC mode. The method was tested on top of ECM6.0 and the simulation results are summarized as follows

Results on natural content compared with ECM6.0 with IBC on

AI: {-0.08%, -0.20%, -0.16%, 100%, 100%}; RA: {xxx%, xxx%, xxx %, xxx%, xxx%}.

Results on Class F

AI: {-0.09%, -0.10 %, 0.04%, 100%, 101%}; RA: {0.00%, -0.05%, 0.10%, 99%, 100%};

Introduction

In ECM-6.0 [1], CIIP which combines inter merge mode and intra mode is utilized. The inter merge mode can be regular merge mode or TM merge mode, and the intra mode can be PDPC mode or the mode derived according to TIMD. If the intra prediction is planar mode, the weights for intra and inter merge predicted samples are uniform in the whole CU. If the derived intra prediction mode is an angular mode, the weights for intra and inter merge predicted samples can be different in different position of the CU.

In ECM6.0, IBC is applied which searches the predicted block from the reconstructed region in the current frame. Besides the regular IBC AMVP or IBC merge mode, IBC is also combined with TM, MMVD mode. Under the common test condition (CTC) [2], IBC is only enabled for SCC sequences but is disabled for natural content sequences due to its relative high encoding complexity.

Proposal

To further improve the IBC performance, the contribution proposes to combine the IBC with the intra prediction. The design of the combined IBC and intra prediction mode is similar to that of the existing CIIP, except that the inter part in CIIP is replaced with the IBC. Specifically, the IBC regular merge mode or IBC TM merge mode is combined with the PDPC mode or the TIMD mode.

For the syntax change, a flag is first signalled to indicate whether a prediction unit is coded with combined IBC and intra prediction mode. If this flag is true, another flag is signaled to indicate whether the intra part is generated based on the TIMD or the PDPC.

Experimental results

The proposed method was implemented on top EMC-6.0 [1] and tests were performed by enabling the IBC tool for natural content sequences. In the current implementation, only the combination of the regular IBC merge and the intra mode is completed. Specifically, in the below, the anchor is the ECM-6.0 with IBC being enabled and the test is the anchor plus the proposed combined IBC and intra mode.

Table 1. Proposed combined IBC and intra method.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0-IBC-on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.10% | 0.00% | 100% | 103% |
| Class A2 | -0.18% | -0.31% | -0.24% | 100% | 97% |
| Class B | -0.09% | -0.21% | -0.11% | 100% | 98% |
| Class C | -0.03% | -0.11% | -0.15% | 100% | 103% |
| Class E | -0.10% | -0.29% | -0.31% | 100% | 100% |
| Overall | -0.08% | -0.20% | -0.16% | 100% | 100% |
| Class D | 0.00% | -0.17% | 0.00% | 99% | 98% |
| Class F | -0.09% | -0.10% | 0.04% | 100% | 101% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-6.0-IBC-On |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.06% | 0.09% | 99% | 100% |
| Class C | 0.03% | -0.10% | -0.15% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.01% | 0.05% | -0.04% | 100% | 100% |
| Class F | 0.00% | -0.05% | 0.10% | 99% | 100% |

Conclusion

The contribution proposes to combine IBC with intra prediction. Simulation results show the effectiveness of the proposed method. It is recommended to study the proposed method in the next round of EE.

References

- ECM-6.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-6.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).