| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0198-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AB0067 (EE2-4.1: Modification of LFNST for MIP coded block) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xinwei Li | Tel: Email: | sid.lxw@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-AB0067. It is confirmed that the implementation matches the proposal and the results match with the proponent’s results.

Introduction

In JVET-AB0067, it is proposed to use DIMD to derive a traditional intra predition mode based on the prediction samples before interpolation of MIP modes, then the LFNST transform set and LFNST transpose flag are both determined by the derived intra mode. Moreover, LFNST is enable for MIP coded blocks of width and height greater than or equal to 4.

Simulation results

The simulation results of the proposed method in JVET-AB0067 are verified. The tests were conducted under the common test conditions (CTC) of EE2.

Table 1. The simulation results of JVET-AB0067

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.15% | -0.05% | 104% | 99% |
| Class A2 | -0.06% | -0.04% | -0.04% | 104% | 99% |
| Class B | -0.09% | -0.01% | -0.09% | 104% | 98% |
| Class C | -0.14% | -0.05% | -0.07% | 103% | 99% |
| Class E | -0.17% | -0.04% | -0.19% | 103% | 99% |
| Overall | -0.11% | -0.05% | -0.09% | 104% | 99% |
| Class D | -0.14% | -0.18% | -0.16% | 103% | 99% |
| Class F | -0.19% | -0.20% | 0.00% | 101% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.06% | -0.10% | 100% | 98% |
| Class A2 | -0.08% | -0.01% | -0.10% | 100% | 99% |
| Class B | -0.08% | 0.26% | -0.25% | 98% | 98% |
| Class C | -0.09% | -0.14% | -0.07% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.03% | -0.14% | 99% | 98% |
| Class D | -0.04% | -0.07% | -0.20% | 100% | 99% |
| Class F | -0.08% | -0.21% | -0.11% | 100% | 99% |

Patent rights declaration(s)

Alibaba Group does not have any current or pending patent rights relating to the technology described in this contribution.