| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0199-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AB0156 (EE2-1.10: Template-based multiple reference line intra prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xinwei Li | Tel: Email: | sid.lxw@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-AB0156. It is confirmed that the implementation matches the proposal and the results match with the proponent’s results.

Introduction

In JVET-AB0156, a template-based multiple reference line intra prediction (TMRL) mode is proposed, which uses a template matching method to construct a list of candidate combinations where each combination comprises a reference line number and an intra prediction mode. An index to the candidate combination list is coded to indicate which particular combination of the reference line and the prediction mode in the candidate list is used in coding the current block. In JVET-AB0156, the regular multiple reference line (MRL) for the non-TIMD part is replaced by the proposed mode.

Simulation results

The simulation results of the proposed method in JVET-AB0156 are verified. The tests were conducted under the common test conditions (CTC) of EE2.

Table 1. The simulation results of JVET-AB0156

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.11% | -0.03% | 103% | 102% |
| Class A2 | -0.10% | 0.03% | -0.09% | 102% | 103% |
| Class B | -0.09% | 0.05% | -0.07% | 102% | 103% |
| Class C | -0.17% | -0.05% | -0.16% | 101% | 107% |
| Class E | -0.18% | -0.14% | -0.25% | 103% | 105% |
| Overall | -0.12% | -0.03% | -0.12% | 102% | 104% |
| Class D | -0.12% | -0.14% | -0.13% | 102% | 106% |
| Class F | -0.13% | -0.16% | -0.08% | 101% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.06% | 0.02% | 100% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.07% | 0.06% | -0.21% | 100% | 100% |
| Class C | -0.06% | -0.01% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.04% | 0.03% | -0.23% | 100% | 100% |
| Class F | -0.13% | -0.01% | 0.13% | 100% | 101% |

Patent rights declaration(s)

Alibaba Group does not have any current or pending patent rights relating to the technology described in this contribution.