| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0200-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AB0157 (EE2-1.12: Combination of EE2-1.10 and EE2-1.11) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xinwei Li | Tel: Email: | sid.lxw@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-AB0157. It is confirmed that the implementation matches the proposal and the results match with the proponent’s results.

Introduction

JVET-AB0157 presents the results of EE2-1.12a (the combination of EE2-1.10 and EE2-1.11a) and EE2-1.12b (the combination of EE2-1.10 and EE2-1.11b). EE2-1.10 is a method with template-based multiple reference line intra prediction (TMRL). EE2-1.11a is an intra prediction fusion method that blends intra predictors derived from up to two different reference lines. EE2-1.11b is an alternative method in which fusion is performed on the two reference lines instead, and then an intra predictor is derived from the fused reference line.

Simulation results

The simulation results of the proposed methods in JVET-AB0157 are verified. The tests were conducted under the common test conditions (CTC) of EE2.

Table 1. The simulation results of JVET-AB0157 test a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.11% | -0.03% | 107% | 103% |
| Class A2 | -0.23% | -0.06% | -0.12% | 106% | 105% |
| Class B | -0.27% | -0.14% | -0.12% | 105% | 104% |
| Class C | -0.29% | -0.18% | -0.23% | 104% | 110% |
| Class E | -0.27% | -0.21% | -0.16% | 105% | 105% |
| Overall | -0.26% | -0.14% | -0.13% | 105% | 105% |
| Class D | -0.27% | -0.21% | -0.25% | 104% | 108% |
| Class F | -0.17% | -0.12% | -0.14% | 103% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.11% | -0.12% | 101% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.11% | 0.13% | -0.17% | 101% | 100% |
| Class C | -0.09% | -0.14% | -0.02% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.11% | -0.07% | -0.21% | 101% | 100% |
| Class F | -0.10% | 0.08% | -0.03% | 100% | 100% |

Table 2. The simulation results of JVET-AB0157 test b

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.01% | 0.05% | 105% | 102% |
| Class A2 | 0.03% | 0.09% | 0.08% | 105% | 104% |
| Class B | 0.08% | 0.23% | 0.26% | 104% | 104% |
| Class C | 0.11% | 0.23% | 0.09% | 103% | 108% |
| Class E | 0.29% | 0.35% | 0.44% | 105% | 105% |
| Overall | 0.09% | 0.19% | 0.19% | 104% | 105% |
| Class D | 0.09% | 0.10% | -0.12% | 103% | 108% |
| Class F | 0.08% | 0.08% | 0.21% | 102% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.01% | 0.06% | 101% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.07% | 0.15% | -0.13% | 101% | 100% |
| Class C | 0.04% | -0.04% | 0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.04% | -0.14% | -0.10% | 100% | 100% |
| Class F | 0.02% | -0.08% | 0.07% | 100% | 100% |

Patent rights declaration(s)

Alibaba Group does not have any current or pending patent rights relating to the technology described in this contribution.