| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0210-v2 |
| --- | --- |

| Title: | AHG3/10: VTM multilayer profile encoder fixes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Fabrice Urban Philippe de Lagrange Gwenaelle Marquant Charles Salmon-Legagneur | Email: | fabrice.urban@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This document presents simulation results of multilayer VTM encoder with two layers spatial scalability. Latest VTM code from master branch after VTM-18.0 is used to include recent merge requests for multilayer profile. Overall, a bdrate improvement of (Y; U; V) -0.3%;	-0.4%;	-0.3% is achieved, with same encoding / decoding time.

# Introduction

Spatial scalability has been studied and a few problems were identified and corrected. Merge requests have been proposed, and merged into the VTM master branch.

This document summarizes the changes and improvements for multilayer coding using a spatial scalability scenario with 2x spatial scalability regarding main changes.

# Reference frames availability checking

In the current VTM encoder, extra reference picture lists are signaled in the slice header for the base layer of multi-layer encoding.

Before encoding a picture, it is checked whether all reference pictures in the current reference picture lists (RPL) are available. If not, new RPL are computed and signaled in the Slice header. As stated by the VVC standard, the decoding process for reference picture marking requires setting inter-layer reference pictures (ILRP) as "long-term" reference pictures. This causes some reference pictures of the base layer not being found in the pre-check of the encoding of the base layer because they are expected as short-term. However, all the reference frames are found as expected, making the RPL in the slice header useless. A fix was proposed to set the conditions in the pre-check consistent with other parts of the VTM, and to avoid extra RPL signaling.

Results are given in Table 1 between VTM-18.0 master commit 72c11672 (Tag VTM-18.0 has a regression preventing multilayer encoding to run) and updated version commit a6140c0f. CTC class A and B sequences are used, with QP 22, 27, 32, 37 on base layer and QP 28, 33, 38, 43 on enhancement layer.

Table 1. Simulation performances of reference frames availability checking

|  | VTM-18 a6140c0f vs VTM-18 72c11672 overall (base layer + enhancement layer) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.05% | -0.05% | -0.05% |
| Class B | -0.11% | -0.11% | -0.10% |
| Overall | -0.08% | -0.08% | -0.07% |
|  | VTM-18 a6140c0f vs VTM-18 72c11672 base layer |  |  |
|  | Y | U | V |
| Class A | -0.04% | -0.05% | -0.04% |
| Class B | -0.11% | -0.11% | -0.11% |
| Overall (base layer) | -0.07% | -0.08% | -0.07% |

# Per-layer APS id range for ALF APSs

When coding a new picture with new ALF filter, the ALF encoder finds a new APS ID to code the filter parameters. Because the amount of APS IDs is limited to 8, the oldest APS is erased. However, in VTM-18.0 encoder before merge request !2320, multilayer encoding has not been considered, and APS from one layer could be overwritten by the encoding of another layer, creating conflicts between layers, even if the resulting bitstream is decodable.

Major impacts are:

- Two successive multilayer encodings with the same base layer parameters could give different base layers.

- Non optimal use of ALF tool.

To fix this issue, a new parameter “AlfapsIDShift” has been added. If no parameter is set for multilayer, the encoder detects that there will be a conflict and ALF APS ID range is automatically set per layer (this is an extension of JVET_X0143_ALF_APS_CHANGES). Each layer has a range of 4 ALF APS IDs.

Results are given in Table 2 between VTM-18.0 master commit a6140c0f and updated version commit a9634d33.

Table 2. Simulation performances of ALF APS range separation for multilayer encoding

|  | VTM-18 a9634d33 vs VTM-18 a6140c0f overall (base layer + enhancement layer) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.02% | -0.10% | -0.06% |
| Class B | -0.03% | -0.12% | -0.07% |
| Overall | -0.02% | -0.11% | -0.06% |
|  | VTM-18 a9634d33 vs VTM-18 a6140c0f base layer |  |  |
|  | Y | U | V |
| Class A | 0.01% | 0.01% | -0.03% |
| Class B | 0.00% | 0.01% | 0.01% |
| Overall (base layer) | 0.01% | 0.01% | -0.01% |

# Remove duplicate coding of RPL in both SPS and SH in multilayer

In the VTM encoder, reference picture lists for dependent layers are coded in the Slice header per picture. In addition, they are also coded in the SPS, but without inter-layer references, because the encoder builds the reference lists from the configuration file directly in the SPS structure at initialization. Dependent layers then contain reference lists that are not used.

Merge request !2333 avoids redundant coding of RPL in both SPS and SH in multilayer. A New parameter "RPLofDepLayerInSH" specifies if the list is coded in the SPS or SH (by default in SPS). For GOP 32 configuration, this saves roughly 2Kb of the Slice Headers (or SPS) in the dependent layer. Results are given in Table 3.

Table 3.Simulation performances of avoiding RPL coding redundancy in SPS and Slice Header

|  | VTM-18 eb421533 vs VTM-18 a9634d33 overall (base layer + enhancement layer) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% |
| Class B | -0.3% | -0.2% | -0.2% |
| Overall | -0.2% | -0.2% | -0.2% |
|  | VTM-18 eb421533 vs VTM-18 a9634d33 base layer |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Overall (base layer) | 0.0% | 0.0% | 0.0% |

# Overall multilayer improvements

Overall results performances of multilayer encoder improvements since VTM-18.0 are summarized in Table 4. Encoding and decoding times are not expected to vary.

Table 4.Simulation performances of multilayer encoder improvements since VTM-18.0

|  | VTM-18 eb421533 vs VTM-18 72c11672 overall (base layer + enhancement layer) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.2% | -0.3% | -0.2% |
| Class B | -0.4% | -0.5% | -0.4% |
| Overall | -0.3% | -0.4% | -0.3% |
|  | VTM-18 eb421533 vs VTM-18 72c11672 base layer |  |  |
|  | Y | U | V |
| Class A | -0.1% | -0.1% | -0.2% |
| Class B | -0.3% | -0.3% | -0.3% |
| Overall (base layer) | -0.2% | -0.2% | -0.2% |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).