| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0211-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AB0130 (EE2-1.14: IntraTMP adaptation for camera-captured content) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Damian Ruiz Coll, Vikas Warudkar | Tel: Email: | duizcoll@ofinno.com, vwarudkar@ofinno.com, |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for JVET-AB0130, which proposes enabling IntraTMP for camera-captured contents, not only for the classes F and TGM, as required in the CTC. Specifically, it is proposed to process the template matching by subsampling the search range followed by iterative refinement to achieve a good trade-off between encoding time and efficiency.

# Introduction

In the current Common Test Conditions (CTC), IntraTMP is only enabled for screen contents (class F and class TGM). The IntraTMP is computed at each position in the four search regions and achieves a good coding gain but at the expense of a high encoding and decoding computing time.

Figure 1. Four search regions for IntraTMP in ECM software

JVET-AB0130 [1] proposes the adaptation of the IntraTMP for camera-captured contents as follows:

- Search range subsampling: subsample each search area by a sampling factor of 2. Consequently, only 1/4th of the template matching search is performed.

- Refinement: after finding the best matching within the subsampled search area, a second search is applied around the best match. The refinement is defined in a window in which dimensions are defined as min(w,h)/2, where w and h are the current CU width and height.

Cross-check of the source code and simulation results has been conducted. Inspected source code follows the description in the proposal. The simulation is performed on top of ECM-60. The cross-check results are provided in the tables below and match the results provided by the proponent. Note that coding time may not be accurate.

The following simulation results were obtained for AI and RA:

Table I. Proposed method on top of ECM-6.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.06% | 0.06% | 102% | 103% |
| Class A2 | -0.32% | -0.51% | -0.44% | 106% | 108% |
| Class B | -0.34% | -0.30% | -0.37% | 101% | 101% |
| Class C | -0.11% | -0.14% | -0.12% | 104% | 96% |
| Class E | -0.48% | -0.63% | -0.67% | 102% | 103% |
| Overall | -0.26% | -0.32% | -0.30% | 103% | 102% |
| Class D | -0.02% | -0.05% | -0.23% | 102% | 97% |
| Class F | 0.04% | -0.08% | 0.16% | 94% | 93% |
| Class TGM | 0.06% | 0.09% | 0.05% | 87% | 83% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.04% | 0.01% | 104% | 103% |
| Class A2 | -0.19% | -0.23% | -0.24% | 104% | 103% |
| Class B | -0.18% | -0.04% | -0.36% | 103% | 103% |
| Class C | -0.04% | -0.14% | 0.00% | 101% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.09% | -0.17% | 103% | 104% |
| Class D | 0.01% | 0.06% | 0.02% | 103% | 105% |
| Class F | 0.03% | 0.04% | -0.02% | 104% | 105% |
| Class TGM | 0.06% | 0.08% | 0.12% | 99% | 103% |

# Conclusion

Results reported at the JVET-AB0130 contribution have been confirmed.

# Reference

[1] K. Naser, T. Poirier, F. Galpin, A. Robert, “EE2-1.14: IntraTMP adaptation for camera-captured content,” JVET-AB0130, October 2022