| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0223-v1 |
| --- | --- |

| Title: | AHG2: Text improvement for Timing / DU information SEI message in HEVC and VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Ye-Kui Wang Benjamin Bross | Email: | first.last@hhi.fraunhofer.de yekui.wang@bytedance.com |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution reports an issue in Picture Timing and DU information SEI message in HEVC and VVC and proposed a text fix.

# Problem description

## Problem #1

In VVC, Picture timing SEI semantics of pt_dpb_output_delay contains the following text:

For pictures that are not output by the "bumping" process because they precede, in decoding order, a CVSS AU that has sh_no_output_of_prior_pics_flag equal to 1 or inferred to be equal to 1, the output times derived from pt_dpb_output_delay shall be increasing with increasing value of PicOrderCntVal relative to all pictures within the same CVS.

In VVC sh_no_output_of_prior_pics_flag does not have any inference. The highlighted part seems to be a leftover from AVC, where the flag could be inferred by the decoder:

When the IDR picture is not the first IDR picture in the bitstream and the value of PicWidthInMbs, FrameHeightInMbs, or max_dec_frame_buffering derived from the active sequence parameter set is different from the value of PicWidthInMbs, FrameHeightInMbs, or max_dec_frame_buffering derived from the sequence parameter set active for the preceding picture, no_output_of_prior_pics_flag equal to 1 may (but should not) be inferred by the decoder, regardless of the actual value of no_output_of_prior_pics_flag.

VVC instead derives the variable NoOutputOfPriorPicsFlag in Annex C for the same purpose:

When the current picture is the first picture of a CVSS AU that is not AU 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the value of PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, MaxBitDepthMinus8, or dpb_max_dec_pic_buffering_minus1[ Htid ] derived for the current AU is different from the value of PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, MaxBitDepthMinus8, or dpb_max_dec_pic_buffering_minus1[ Htid ], respectively, derived for the preceding AU in decoding order, NoOutputOfPriorPicsFlag may (but should not) be set equal to 1 by the decoder under test, regardless of the value of sh_no_output_of_prior_pics_flag of the current AU.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to sh_no_output_of_prior_pics_flag of the current AU is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag equal to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to sh_no_output_of_prior_pics_flag of the current AU.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD, such that when the value of NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and the DPB fullness is set equal to 0.

Thus the semantics should be changed to refer to NoOutputOfPriorPicsFlag instead.

The phrase “sh_no_output_of_prior_pics_flag equal to 1 or inferred to be equal to 1” occurs in total three times in the standard text:

- semantics of pt_dpb_output_delay (Picture timing SEI message semantics)

- semantics of pt_dpb_output_du_delay (Picture timing SEI message semantics)

- semantics of dui_dpb_output_du_delay (DU information SEI message semantics)

All three references should be changed to “NoOutputOfPriorPicsFlag equal to 1”.

The same problem exists in in HEVC with slightly different syntax element names. The phrase “no_output_of_prior_pics_flag equal to 1 or inferred to be equal to 1” occurs in

- semantics of pic_dpb_output_delay (Picture timing SEI message semantics)

- semantics of pic_dpb_output_du_delay (Picture timing SEI message semantics)

- semantics of pic_spt_dpb_output_du_delay (Decoding unit information SEI message semantics)

All three references should be changed to “NoOutputOfPriorPicsFlag equal to 1”.

## Problem #2

In VVC the

- semantics of pt_dpb_output_du_delay (Picture timing SEI message semantics)

- semantics of dui_dpb_output_du_delay (DU information SEI message semantics)

refer to “a CLVSS picture that has sh_no_output_of_prior_pics_flag equal to 1 or inferred to be equal to 1”.  The part “a CLVSS picture” should be replaced by “a CVSS AU” as in the semantics of pt_dpb_output_delay.

## Problem #3

In VVC a minor text improvement for the semantics of sh_no_output_of_prior_pics_flag is suggested. The word value should be removed as follows:

sh_no_output_of_prior_pics_flag affects the output of previously-decoded pictures in the DPB after the decoding of a picture in a CVSS AU that is not the first AU in the bitstream as specified in Annex ‎C.

It is a requirement of bitstream conformance that the value of sh_no_output_of_prior_pics_flag shall be the same for all slices in an AU that have sh_no_output_of_prior_pics_flag present in the SHs.

When all slices in an AU have sh_no_output_of_prior_pics_flag present in the SHs, the value of sh_no_output_of_prior_pics_flag in the SHs is also referred to as the value sh_no_output_of_prior_pics_flag of the AU.

# Patent rights declaration(s)

Fraunhofer HHI does not have any current or pending patent rights relating to the technology described in this contribution.

Bytedance does not have any current or pending patent rights relating to the technology described in this contribution.