| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0227-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AB0092 (EE2-1.8: Gradient linear model with luma value) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Che-Wei Kuo, Hong-Jheng Jhu | Email: | {cheweikuo, jhuhong-jheng}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

Introduction

This document reports the crosschecking results of EE2 Test 1.8. In Test 1.8, a new GLM mode is presented where the chroma sample is predicted based on both the gradient Gi,j of luma samples and the reconstructed value recL'i,j of the down-sampled luma sample with different parameters:

predCi,j=α0∙Gi,j+α1∙recL'i,j+α2∙midValue#

In Test 1.8a, the original GLM mode is replaced by the proposed new GLM mode, while in Test 1.8b, the proposed new GLM mode is signaled as an additional mode.

Results over ECM-6.0 show that BD-rate points are perfectly matched with the results provided from the proponents.

Simulation results

Table 1 Test 1.8a (Gradient linear model with luma value)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.51% | -0.29% | 102% | 98% |
| Class A2 | 0.01% | -0.69% | -0.87% | 101% | 100% |
| Class B | 0.00% | -0.29% | -0.26% | 101% | 99% |
| Class C | 0.01% | -0.22% | -0.22% | 101% | 100% |
| Class E | 0.00% | -0.14% | -0.19% | 101% | 98% |
| Overall | 0.00% | -0.35% | -0.35% | 101% | 99% |
| Class D | 0.01% | -0.43% | -0.32% | 101% | 98% |
| Class F | -0.27% | -1.52% | -1.55% | 100% | 98% |
| Class TGM | -1.32% | -3.99% | -3.57% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.14% | -0.08% | 101% | 98% |
| Class A2 | -0.03% | -0.26% | -0.49% | 100% | 99% |
| Class B | -0.01% | -0.07% | -0.15% | 101% | 99% |
| Class C | 0.02% | -0.07% | -0.10% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.12% | -0.19% | 101% | 99% |
| Class D | 0.03% | -0.31% | -0.30% | 100% | 99% |
| Class F | -0.47% | -1.74% | -1.51% | 100% | 99% |
| Class TGM | -0.67% | -2.69% | -2.34% | 101% | 100% |

Table 2 Test 1.8b (Gradient linear model with luma value as an additional mode)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.51% | -0.30% | 102% | 100% |
| Class A2 | 0.01% | -0.69% | -1.02% | 102% | 101% |
| Class B | -0.01% | -0.30% | -0.27% | 102% | 99% |
| Class C | 0.01% | -0.10% | -0.16% | 102% | 99% |
| Class E | -0.02% | -0.18% | -0.18% | 102% | 99% |
| Overall | 0.00% | -0.34% | -0.36% | 102% | 99% |
| Class D | 0.00% | -0.14% | -0.20% | 102% | 98% |
| Class F | -0.28% | -1.94% | -1.96% | 100% | 98% |
| Class TGM | -1.34% | -4.86% | -4.13% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.12% | -0.21% | 102% | 99% |
| Class A2 | -0.02% | -0.21% | -0.54% | 101% | 99% |
| Class B | 0.00% | 0.02% | -0.22% | 101% | 99% |
| Class C | 0.00% | -0.14% | -0.13% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.10% | -0.26% | 101% | 99% |
| Class D | 0.04% | -0.20% | -0.07% | 100% | 99% |
| Class F | -0.47% | -1.87% | -1.94% | 100% | 100% |
| Class TGM | -0.69% | -2.96% | -2.49% | 101% | 100% |

Patent rights declaration(s)

Kwai Inc. does not have any current or pending patent rights relating to the technology described in this contribution.