| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB0244-v1 |
| --- | --- |

| Title: | AHG9: BoG Report on Neural-network Post-filter Characteristics SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This is a report of activities from the BoG on Neural-network Post-filter Characteristics (NNPFC) SEI Message.  The BoG conducted following meetings during the 28th JVET meeting:

- October 21 – 14:00-18:25 CEST

BoG was closed at 18:25 CEST on October 21.

The BoG recommendations are described and highlighted in section 1 and 2 below.

# BoG Related Documents

The scope of this BoG activities is related to following documents and NNPFC related ballot comments in MPEG input document m60678.

JVET-AB0046 AHG9: On StrengthControlVal of the NNPFC SEI message [M. M. Hannuksela, M. Santamaria, F. Cricri (Nokia)]

JVET-AB0047 AHG9: nnpfc_mode_idc related changes to the NNPFC SEI message [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AB0049 AHG9: Miscellaneous aspects of the two neural-network post-filtering SEI messages [Y.-K. Wang, Y. Li, C. Lin, J. Li, K. Zhang, L. Zhang (Bytedance)]

JVET-AB0050 AHG9: Activation of a neural-network post-processing filter for multiple pictures [Y.-K. Wang, K. Zhang, L. Zhang, C. Lin, J. Li, Y. Li (Bytedance)]

JVET-AB0058 AHG9: Frame Rate Upsampling Information in Neural-network Post-filter Characteristics SEI Message [S. Deshpande, A. Sidiya (Sharp)]

JVET-AB0059 AHG9: Comments on Neural-network Post-filter Characteristics SEI Message [S. Deshpande (Sharp)]

JVET-AB0060 AHG9: On activation of the neural-network post-filter characteristics SEI message [T. Chujoh, Y. Yasugi, T. Ikai (Sharp)]

JVET-AB0074 AHG9: Auxiliary input for neural-network post-processing filter [Y. Li, J. Li, C. Lin, K. Zhang, L. Zhang, Y.-K. Wang (Bytedance)]

JVET-AB0075 AHG9: Separate processing of chroma components for neural-network post-processing filter [Y. Li, J. Li, C. Lin, K. Zhang, L. Zhang, Y.-K. Wang (Bytedance)]

JVET-AB0134 AHG9: On NN post-filter activation SEI message [M. Pettersson, R. Sjöberg, J. Ström (Ericsson)]

JVET-AB0135 AHG9: On complexity metrics for NN post-filter characteristics SEI message [M. Pettersson, R. Sjöberg, J. Ström (Ericsson)]

JVET-AB0152 AHG9: Regional on/off control and selection of NNPFs [J. Li, C. Lin, K. Zhang, L. Zhang, Y.-K Wang, Y. Li (Bytedance)]

Topic of T.35 mode for NNPFC was discussed at 6:10 CEST:

A draft for signaling was presented by S. McCarthy.

It was commented that the T.35 syntax structure should be copied into the NNPFC message when the nnpfc_mode_idc indicates signaling via T.35 syntax structure to resolve any ambiguity of which T.35 message has the NN data.

Revisit for the text in JVET.

# BoG Activities

Following mandate as specified for this BoG in JVET-AB_Notes-d* document:

“BoG (S. Deshpande) to continue review of the open items from here. Meeting in parallel with EE2 activities in JVET plenary, starting from 1400 CEST on Friday 21 Oct.”

BoG discussed following items (please note the numbering below starts at 6 to match the corresponding identically numbered items in JVET-AB_Notes-d* document:

- On NNPF auxilary inputs

  - Move the normalization of the quantization strength input variable StrengthControlVal from the VSEI text to the use of NNPFC SEI message in the VVC text. (JVET-AB0046 item 1)

  - Change the normalization of StrengthControlVal from 2(StrengthControlVal − 42) / 6 to a division (÷) by 63. With this change, it is asserted that StrengthControlVal becomes a variable that has been normalized to the range 0..1. In Table 23, insert the given StrengthControlVal directly into the input tensor: inputTensor[ ][ ][ ][ ] = StrengthControlVal. (JVET-AB0046 item 2, BC#027)

  - Define and specify the use of more auxiliary inputs to the neural network post-processing filter, e.g., prediction samples. (BC#040, JVET-AB0074 item 1)

  - Make the following change (BC#045, JVET-AB0058 and JVET-AB0074 item 2):

When nnpfc_inp_order_idc is equal to 3, nnpfc_component_last_flag is equal to 0, and nnpfc_auxiliary_inp_idc is equal to 0, no auxiliary input matrix is used. In other words, remove the assignment operation "inputTensor[ 0 ][ 6 ][ yP + overlapSize ][ xP + overlapSize ] = 2(StrengthControlVal – 42)/6'' for this case from Table 23.

Discussion:

Regarding items a, b, the goal is to move VVC specific initialization to VVC text instead of it being in VSEI text (where it manifests as 2(StrengthControlVal − 42) / 6 – especially the values 42 and 6 in that equation).

BoG recommends to adopt items 6a and 6b, with additionally specifying in VSEI text that the range of StrengthControlVal is in the range of 0 to 1, inclusive.

Regarding item C: It was commented that we should consider which information the post processing filter has access to. For e.g. it was commented that information from slice layer above is easily accessible. Previously there was boundary strength information as an additional auxiliary input which was gotten rid of due to consideration that all post filters may not easily have access to that information. It was commented that such auxiliary information may be easily accessible to post filter via a software decoder. It was commented by one participant that in their opinion the prediction samples information may not easily be accessible if the decoder is not software decoder.

It was commented that the quoted references 1-2 in the proposal use this information via a modified VTM software.

It was also asked what is CroppedYPred, CroppedCbPred, CroppedCrPred and where it is defined. It was agreed by the proponents that maybe it is not clearly defined (and also not defined in VVC spec).

It was asked by proponents if our design principle is to have the auxiliary input information easily obtainable from all types of decoder.

It was decided to raise if this should be our design principle as a question to the larger group.

Recommendation: Discuss this design question in JVET.

Regarding item d: (Chaired by M. M. Hannuksela): It was agreed that this was an oversight bug in the current specification.

BoG recommends to adopt item 6d.

- (On syntax elements specified as UTF-8 character strings UTF-8 ) There are two syntax elements that are specified as UTF-8 character strings. One of them says it shall be as specified in ISO/IEC 10646 and the other does not. Shouldn’t they both be as specified in ISO/IEC 10646? Clarify. (BC#028)

  - Clarified by removing explicit reference to ISO/IEC 10646 from the semantics of nnpfc_uri. Both of the syntax elements are of type st(v), which is specified in VSEI to be null-terminated string encoded as universal coded character set (UCS) transmission format-8 (UTF-8) characters as specified in ISO/IEC 10646. (JVET-AB0047, one of the editorial changes)

BoG recommends to agree to this editorial change (adopt 7a)

- On value ranges for nnpfc_id and nnpfa_id and decoder handling of their reserved values

  - When encountering an NNPFC SEI messages with a reserved value of nnpfc_id, the decoder shall ignore the NNPFC SEI message instead of ignoring nnpfc_id value. (BC#030, JVET-AB0049 item 1)

  - Specify the value range and decoder handling of reserved values for nnpfa_id similarly as for nnpfc_id. (BC#055, JVET-AB0049 item 10)

Discussion:

About 8a: current spec says ignore the reserved values of nnpfc_id. It was asserted by the proponent that this is not sufficient. It was commented that the current language is motivated by the corresponding language in frame packing SEI. It was agreed that this case is different than that.

BoG recommends to adopt item 8a.

About 8b: nnpfa_id in the activation SEI message is similar to nnpfc_id and so this is a bug-fix.

BoG recommends to adopt item 8b.

- (On array convention) The standard generally uses an array convention in which the horizontal index is first and the vertical index is second. In several of these expressions, the indexing is swapped relative to that usual convention, e.g. for InpSampleVal( ) indexing. Is the order swapping necessary? Use the same convention consistently. If using an inconsistent convention is necessary (e.g. for compatibility with the NNR standard), highlight the difference in a NOTE to minimize reader confusion. (BC#036)

It was commented that this was intentionally done. This is because in inference engines that the commenter had checked (i.e. SADL, Tensorflow, PyTorch, MXNet) the vertical index was first and horizontal index was second.  It was also commented that NNR does not have a particular convention.

It was commented that specifying this in our specification maybe just a matter of editorial convention.

It was commented by a few participants that since this is about deriving input tensors and storing output tensors, we should specify an order and that the tensors should use that order.

BoG recommends to keep the current order and add a NOTE.

- On the value of nnpfc_out_sub_c_flag (BC#037, JVET-AB0049 item 5)

Replace the following sentence:

If SubWidthC is equal to 2 and SubHeightC is equal to 1, nnpfc_out_sub_c_flag shall not be equal to 0.

with the following:

When ChromaFormatIdc is equal to 2 and nnpfc_out_sub_c_flag is present, the value of nnpfc_out_sub_c_flag shall be equal to 1.

This discussion was chaired by M. M. Hannuksela.

It was commented that the current phrasing in the draft is equivalent to the proposed phrasing. It was commented that however the proposed phrasing is easier to understand.

BoG recommends to adopt item 10.

- (On coupled or separate filtering of chroma components) Enable separate processing of chroma components, including processing only one of the chroma components by one NNPFC SEI message. (BC#044, JVET-AB0075)

Discussion: It was commented by proponent that current specification operates in the same manner on both chroma components and that same filtering is applied to the two components.

It was commented by participant that currently actual neural-network could decide to operate only on Cb or Cr (and ignore the other component).

It was also commented that maybe the same process could be applied if we change inp_order_idc to mean one input matrix (not just luma). To that it was commented that then we may need some other signaling which specifies whether the component was luma or Cb or Cr.

It was commented that this adds certain amount of additional specification text and cases which would need to be checked carefully.

BoG recommends Further study.

- (On NNPF patch size) Specify the following: When nnpfc_constant_patch_size_flag is equal to 0, the patch size width, denoted by inpPatchWidth, and the patch size height, denoted by inpPatchHeight, are provided by external means. An example of such external means is an API that passes the values of inpPatchWidth and inpPatchHeight to the decoder and render entity in a video application system, and the values may be configured by a user through a user interface of the application. (BC#046, JVET-AB0049 item 6)

Discussion: It was commented by two participants that some clarification is indeed useful for nnpfc_constant_patch_size_flag equal to 0. The text in JVET-AB0049 item 6 was preferred. An additional suggestion was to add the quoted text:

The values of inpPatchWidth and inpPatchHeight are provided by external means not specified in this Specification “or determined by the post-processing unit”.

BoG recommends to adopt JVET-AB0049 item 6 with additional change noted above.

- On NNPF activation, on/off control, and filter selection

  - Enable activating an NNPF for a set of consecutive pictures in output order in a CLVS by using one NNPFA SEI message. (BC#049, JVET-AB0050, JVET-AB0060)

    - Yes, enable it by one of the following (JVET-AB0050):

      - Add a flag: nnpfa_on_flag.

      - Add a new SEI message for deactivating a filter.

      - Add two flags: nnpfa_cancel_flag and nnpfa_persistence_flag.

    - Yes, enable it by not changing the NNPFA SEI message but adding an activation type to the NNPFC SEI message, with the following specified activation types: 0 - the NNPFA SEI message is applied; 1 – all pictures in the current CVS use this NNPF; 2 – all pictures in the current CVS with the same nuh_layer_id and TempralId, respectively, as the NAL unit containing the NNPFC SEI message use this NNPF. (JVET-AB0060)

  - Replace nnpfa_id with nnpfa_activation_id_plus1 and specify that nnpfa_activation_id_plus1 equal to 0 in the NNPFA SEI message indicates that no NNPF is activated for the current picture, to be able to explicitly indicate that no NNPF is activated for the current picture. (JVET-AB0134 item 1)

Item 13-a and 13-b was chaired by M. M. Hannuksela.

Discussion: The main usage scenario considered is activating same NNPF for multiple pictures.

It was commented that the current method only uses 1 byte per activation and if there is motivation for these proposals besides the bit savings. It was pointed out that rather it is one SEI message per activation which will be more than 1 byte so it could be about 6 bytes or so. It was also argued that if it is common usage to have same NNPF for multiple pictures then it may be a good design to support signaling as proposed in these proposals.

It was commented that the motivation for proposal b was to make the signaling robust to loss of SEI message(s).

It seemed there is some inclination to take an action based on above discussion about bit savings.

It was commented by a proponent of 13a that solution in 13b does not support signaling activating a NNPF for multiple pictures. Although proponent commented that item 13b can be combined with item 13a.

Thus, it was suggested to first decide regarding item 13a.

It was asked if it is a common use case to have different NNPF for different temporal sublayers and/or for different layers. It was answered by a proponent that for RA case since base QP is typically different for different temporal sublayer this may be the common. It was commented that this aspect may also be handled by using auxiliary input.

It was asked if the following case is supported by the proposals in item 13a: that a first NNPF if first activated and then a second NNPF filter is activated without deactivating the first NNPF. Is this allowed or forbidden. It was commented that this is already supported in design in current spec. For example, current specification allows activating multiple NNPFs for same picture.

It was commented by proponent of 13a-i that the aspect of 13a-ii which adds an activation type to the NNPFC SEI message is mixing the concept of NNPF activation and NNPF filter signaling which may not be desirable.

BoG recommends to adopt 13-a-i-3.

Discussion of b (and other aspects) after above recommendation:

A comment was made that we should consistently use same type of persistence across different SEIs. And that other SEI messages don’t have id associated with cancellation. It was commented that for NNPF you maybe have multiple NNPFs which can apply to same picture which is different aspect than those SEIs. It was commented that there may be a robustness issue for a splicer if cancelling is associated with an id. The counter argument was that typically a splicer starts a new CVS which cancels the active filter.

The aspect of changing the name nnpfa_id to further distinguish it from nnpfc_id is left to the editors. BoG recommends no further action on item b.

  - Enable activating different NNPFs for different regions (e.g., subpictures, slices, tiles, CTUs) of a picture? (BC#050, JVET-AB0152 option 2)

    - Yes, activate one or more NNPFs in an NNPFA SEI message, and a selected NNPF can be used for the entire current picture, some slices of the current picture, or some CTUs of the current picture. (JVET-AB0152 option 2)

Discussion: It was commented CTU wise signaling and activation of different NN post filters was tested by one company. It was observed that the gain from this was somewhat cancelled by additional overhead (e.g of using CABAC coded CTU wise signaling).

Separately it was commented that such an idea has shown gain for NN in-loop filter.

It was asked if this would be useful for artistic use (different than coding gain aspect) – e.g. bokeh effect. There seemed some interest in that aspect.

There was a comment that JVET should first conclude on benefit of region-wise NN PF and then the syntax/ HLS aspect of SEI signaling should be considered.

It was commented that there could be a region-wise NNPF activation message in future and so we are not closing door on this.

BoG recommends further discussion in JVET.

  - Enable regional on/off control of NNPF? (JVET-AB0134 item 2, JVET-AB0152)

    - Yes, activate one NNPF in an NNPFA SEI message and the NNPF can be used for the entire current picture or a list of explicitly signalled regions (with region width, height, top and left positions signalled) of the current picture. (JVET-AB0134 item 2a)

      - Alternatively, same functionality, but add four more syntax elements targeting at reducing the signalling overhead. (JVET-AB0134 item 2b)

    - Yes, activate one NNPF in an NNPFA SEI message and the NNPF can be used for the entire current picture, some slices of the current picture, or some CTUs of the current picture. (JVET-AB0152 option 1)

    - Yes, also enable activating different NNPFs for different slices or CTUs of a picture. (JVET-AB0152 option 2)

It seems item 13-d-iii is covered by discussion in c above.

Regarding items 13-d i-ii, the high-level question is if we want to enable regional on/off control of NNPF.

Currently an NNPF active for a picture is applied to the entire picture. Do we want to allow only applying it to part of that picture (and turn it off for the other part of the picture).

There was no apparent example of doing such on-off control for NN PF, but there has been example of doing it for NN in-loop filter.

It was commented that doing NN PF on part of the picture would have less complexity that doing it on entire picture.

There was a comment that JVET should first conclude on benefit (including simplicity) of region-wise NN PF being turned on/ off and then the syntax/ HLS aspect of SEI signaling should be considered.

It was commented that there could be a region-wise NNPF on/ off message in future and so we are not closing door on this.

BoG recommends further discussion in JVET.

- On presence and decoding order of NNPFC and NNPFA SEI messages

  - Specify the following constraint ("no NNPFC, no NNPFA"): An NNPFA SEI message with a particular value of nnpfa_id shall not be present in a current PU unless within the current CLVS there is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_id present in a PU preceding the current PU in decoding order or in the current PU. (BC#053, JVET-AB0049 item 8)

  - Specify the following constraint ("NNPFC precedes NNPFA"): When a PU contains both an NNPFC SEI message with a particular value of nnpfc_id and an NNPFA SEI message with nnpfa_id equal to the particular value of nnpfc_id, the NNPFC SEI message shall precede the NNPFA SEI message in decoding order. (BC#054, JVET-AB0049 item 9)

  - Is it allowed for more than one NNPFC SEI message to be present for the same picture? If so, what does this mean?

Is there some limit on the number of them? If more than one is present, what happens if the nnpfc_purpose values are the same or if they are different? What happens if the nnpfc_mode_idc values are the same or if they are different? Add appropriate clarification, explanation and/or constraints. (BC#056)

Discussion 14-a: It was commented by multiple participants that “no NNPFC means no NNPFA” makes sense.

BoG recommends to adopt 14-a.

Discussion 14-b: It was commented by multiple participants that in that case NNPFC should precede NNPFA and the proposal makes sense.

BoG recommends to adopt 14-b.

Discussion 14-c: About “Is it allowed for more than one NNPFC SEI message to be present for the same picture”- the answer is the current spec allows it. For the rest of the questions the group agrees that this should be answered and clarified and that some sensibility constraints maybe needed. It was suggested that offline discussion maybe useful to arrive at how to clarify the answers to these questions and what (if any) specification text is needed to clarify this.

It was agreed that all these questions should be answered. Such a discussion is encouraged and if it results in a new input, that will be discussed.

- (On NNR base and update) It is our understanding that a neural network representation (NNR) can either be “intra” (coded without reference to some other NNR) or predictively/incrementally coded. Is there a way to identify which is which? In the case of incremental NNR, what is the reference? For example, is the reference the “base” representation, or could it be the accumulated result of several updates relative to a base representation? In what order are the updates applied? Clarify. (BC#058)

It was commented that JVET-AB0047 provides editorial clarifications on this. These were described verbally.

The current understanding of the group:

It is our understanding that a neural network representation (NNR) can either be “intra” (coded without reference to some other NNR) or predictively/incrementally coded.

Is there a way to identify which is which? For NNR, profile 0 is intra. For NNR profile 1 can be intra or differential update.

Also, the answer to the next question defines “base”, which is the “intra” NNR and the other NNR is update of the base.

In the case of incremental NNR, what is the reference? The reference is the “base”, and the “base” is the NNR corresponding to first NNPFC SEI with a particular nnpfc_id.

For example, is the reference the “base” representation, or could it be the accumulated result of several updates relative to a base representation? Answered above.

In what order are the updates applied? The updates are not cumulative (due to robustness issue if some updates are lost and that it may make SEI message design complicated), so an update is always applied on top of a base. Also, the order is the bitstream order.

BoG recommendation: Delegated to the editors to clarify based on JVET-AB0047 and above answers.

- Editorial changes not covered above:

  - BC#008, 012~016, 018, 023, 025, 026, 029, 031~033, 038, 039, 041~043, 047, 048, 051, 052, 057, 059~065

  - JVET-AB0047, excluding the technical changes in items 1 to 3

Editorial aspects were not discussed in the BoG.

----