| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0006-v1 |
| --- | --- |

| Title: | JVET AHG report: ECM software development (AHG6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Fabrice Le Leannec Kai Zhang | Tel: Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com fabricel@xiaomi.com zhangkai.video@bytedance.com |
| Source: | AHG6 on ECM software development |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG6 on ECM software development that has taken place between the 28th and 29th JVET meetings.

# Introduction

The mandates for the AHG are as follows:

- Coordinate development of the ECM software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Prepare and deliver ECM-7.0 software version and the reference configuration encodings according to the ECM common test conditions.

- Investigate encoder speedup and other encoder software optimization.

- Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate.

# Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

VTM-11.0ecm anchor https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC is used for ECM performance evaluation.

The following adopted aspects were integrated into ECM-7.0:

JVET-AB0155: Spatial GPM (Test 1.6b) (MR 271)

JVET-AB0092: GLM with luma value (Test 1.8b) (MR 267)

JVET-AB0157: Template based MRL and intra fusion (Test 1.12a) (MR 275)

JVET-AB0143: CCCM with template selection (Test 1.13a) (MR 260)

JVET-AB0130: IntraTMP search speedup (Test 1.14). Enable Intra TMP in classes A-E (CTC) (MR 274)

JVET-AB0078: AMVPmerge mode for LB (Test 2.1a) (MR 253)

JVET-AB0079: TM-based BCW index derivation (Test 2.2) (MR 262)

JVET-AB0112: Affine DMVR (Test 2.6) (MR 263)

JVET-AB0061: Using block vector derived from IntraTMP for IBC (Test 3.2) (MR 257)

JVET-AB0067: Modification of LFNST for MIP coded block (Test 4.1) (MR 272)

JVET-AB0184: Extended fixed filter output for ALF (Test 5.1) (MR 261)

JVET-AB0082: Fix for ARMC and use 12-tap filter for picture upscaling (MR 276)

JVET-AB0174: Division free CCCM (MR 257)

JVET-AB0189: RMVF bit length control (MR 265)

Included fixes:

Fix to use unfiltered source for HDRmetrics when MCTF is enabled (MR 273)

Fix ISP CHECKD condition (MR 277)

Fix out of bound access to array EXT_REF_LINE_IDX (MR 281)

Disable EncDbOpt when deblocking filter is disabled (MR 288)

Picture padding to include template (290)

Check if IBC is used before resetting IBC buffer (MR 297)

Memory deallocation for CCSAO (MR 298)

ECM-7.0 was tagged on December 7, 2022.

There are no changes to VTM-11 anchor, the latest version is VTM-11ecm6 which was tagged on September 4, 2022.

## CTC Performance

In this section, ECM test results following ECM CTC configuration descried in JVET-Y2017 are summarized.

Next tables show ECM-7.0 performance over ECM-6.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.57% | -1.41% | -1.14% | 121% | 104% |
| Class A2 | -1.03% | -2.81% | -3.34% | 119% | 107% |
| Class B | -1.00% | -2.22% | -2.00% | 120% | 110% |
| Class C | -0.83% | -1.17% | -1.16% | 123% | 121% |
| Class E | -1.52% | -1.92% | -1.78% | 125% | 119% |
| Overall | -0.98% | -1.90% | -1.86% | 122% | 112% |
| Class D | -0.65% | -0.71% | -0.94% | 115% | 112% |
| Class F | -0.78% | -2.74% | -2.70% | 105% | 105% |
| Class TGM | -1.81% | -5.38% | -4.66% | 102% | 92% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.86% | -0.75% | -1.37% | 104% | 93% |
| Class A2 | -1.08% | -1.93% | -2.16% | 103% | 98% |
| Class B | -0.93% | -1.55% | -1.60% | 110% | 96% |
| Class C | -0.62% | -0.84% | -0.71% | 115% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.86% | -1.28% | -1.43% | 109% | 99% |
| Class D | -0.58% | -0.79% | -0.92% | 115% | 105% |
| Class F | -1.12% | -2.69% | -2.59% | 109% | 102% |
| Class TGM | -1.05% | -3.12% | -2.73% | 108% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.81% | -1.44% | -1.25% | 105% | 93% |
| Class C | -0.67% | -0.93% | -0.90% | 108% | 93% |
| Class E | -0.64% | 0.17% | -0.71% | 115% | 95% |
| Overall | -0.72% | -0.87% | -1.00% | 108% | 93% |
| Class D | -0.77% | -0.48% | -0.14% | 117% | 101% |
| Class F | -0.74% | -2.06% | -1.44% | 107% | 100% |
| Class TGM | -0.37% | -1.36% | -1.10% | 102% | 94% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over ECM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.46% | -0.79% | -0.49% | 103% | 93% |
| Class C | -0.40% | -0.21% | -0.68% | 105% | 95% |
| Class E | -0.27% | -0.04% | -0.03% | 114% | 99% |
| Overall | -0.39% | -0.41% | -0.44% | 106% | 95% |
| Class D | -0.50% | -0.17% | 0.24% | 111% | 98% |
| Class F | -0.42% | -1.65% | -1.25% | 103% | 97% |
| Class TGM | -0.43% | -1.44% | -1.20% | 100% | 92% |

The below tables show ECM-7.0 performance comparing to VTM-11.0ecm6 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -8.67% | -17.52% | -24.11% | 617% | 348% |
| Class A2 | -12.88% | -21.39% | -24.64% | 576% | 348% |
| Class B | -7.62% | -20.59% | -18.79% | 630% | 392% |
| Class C | -8.12% | -12.50% | -12.91% | 635% | 411% |
| Class E | -9.72% | -17.97% | -16.91% | 619% | 435% |
| Overall | -9.13% | -17.98% | -19.03% | 618% | 387% |
| Class D | -6.68% | -10.00% | -10.01% | 624% | 416% |
| Class F | -18.45% | -26.68% | -26.24% | 444% | 397% |
| Class TGM | -31.47% | -38.80% | -37.65% | 386% | 367% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -19.46% | -21.91% | -29.05% | 583% | 646% |
| Class A2 | -21.45% | -28.23% | -31.25% | 586% | 894% |
| Class B | -17.65% | -28.04% | -25.93% | 614% | 830% |
| Class C | -19.26% | -22.56% | -22.39% | 605% | 892% |
| Class E |  |  |  |  |  |
| Overall | -19.20% | -25.39% | -26.67% | 600% | 817% |
| Class D | -20.25% | -23.80% | -24.06% | 605% | 959% |
| Class F | -22.05% | -29.05% | -29.34% | 520% | 556% |
| Class TGM | -30.00% | -35.63% | -35.75% | 556% | 438% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -16.16% | -30.50% | -28.78% | 529% | 695% |
| Class C | -17.44% | -25.63% | -25.66% | 552% | 714% |
| Class E | -14.58% | -22.41% | -23.17% | 552% | 483% |
| Overall | -16.19% | -26.85% | -26.34% | 542% | 640% |
| Class D | -19.65% | -26.43% | -26.39% | 554% | 850% |
| Class F | -20.49% | -29.26% | -27.88% | 514% | 496% |
| Class TGM | -28.89% | -35.39% | -35.06% | 524% | 401% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -15.41% | -30.16% | -27.87% | 486% | 694% |
| Class C | -16.91% | -24.79% | -24.39% | 460% | 701% |
| Class E | -14.28% | -22.01% | -22.86% | 519% | 501% |
| Overall | -15.63% | -26.33% | -25.46% | 485% | 642% |
| Class D | -19.78% | -26.11% | -26.07% | 456% | 758% |
| Class F | -19.49% | -28.95% | -27.62% | 500% | 481% |
| Class TGM | -25.88% | -33.29% | -33.07% | 543% | 399% |

The Excel files with the complete test results are attached to this report.

# Recommendations

The AHG recommends to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.